| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4793 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.VCDN.1-NV proposed use case of vCDN: smart traffic control |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to provide a use case of vCDN: smart traffic control.

# Introduction

In the last meeting, the group created a new draft Recommendation to describe the network virtualization in the vCDN service.  In the legacy CDN, traffic control is managed based on the status reported form the monitor point in the network or CDN nodes. It is not the real status. Also the bandwidth is fixed, which is not suit for the service that across the peak time and normal time. But in vCDN approach, the NFV/SDN control provides a possibility for CDN to obtain the real network status and control the data transmission in the transport network. This contribution is proposed to add that user case into Appendix I.

# Proposal

- Proposes to add the new Appendix I.2 smart traffic control

# Start proposal

Appendix I

Use cases of network virtualization supporting VCDN services

(This Appendix does not form an integral part of this Recommendation.)

I.2 smart traffic control

In general, the quality of media content distribution and delivery in CDN is usually restricted to bandwidth, traffic congestion and network jitter. The routing path and transport control cannot be easily predicted in the legacy CDN because of the lack of sensing for the low layer network status. In vCDN approach, the traffic control can be centralized in a transport controller, e.g. a SDN controller, and the profile of network can be obtained by vCDN management node. Before the content transmission, the vCDN will plan a shortest (may not the shortest in geography) and efficient distribution/delivery path, according the profile of network status. Moreover, the traffic control may adjust the bandwidth for media transmission in the peak time and release the bandwidth after.

Figure A-2: smart traffic control in vCDN

As the Figure A-2 shows, the media content data can be distributed to the different edge nodes with an optimized path while the traffic congestion is detected. In addition, a virtualized switch/router can be created for adjust the bandwidth temporary.

# End Proposal

__________________