| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0045_proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-C0045 based on 3D-HEVC Test Model 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Liu He Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides text for Draft 2 of 3D-HEVC Test Model Description. The proposal only changes the motion vector candidate list derivation process as specified in sub-clause G.8.5.2.1.

All the changes are highlighted in green.

- G.8.5.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

the luma motion vectors mvL0 and mvL1,

the reference indices refIdxL0 and refIdxL1,

the prediction list utilization flags predFlagL0 and predFlagL1,

the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

the disparity vectors ivpMvDispL0 and ivpMvDispL1.

The variables singleMCLFlag is derived as follows.

If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1 are derived as specified by the following ordered steps:

  - The derivation process for spatial merge ing candidates from neighboring prediction unit partitions as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause G.8.5.2.1.7 is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(G-87) 
predFlagLXCol = availableFlagLXCol		(G-88)

  - Depending on multi_view_mv_pred_flag, the following applies.

If multi_view_mv_pred_flag is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause G.8.5.2.1.9 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the output is assigned to the view order index of the reference view refViewIdx, the availability flags array availableFlagIvMC and availability flag availableFlagIvDC, the reference indicesindex array refIdxLXIvMC and reference index refIdxLXIvDC, the prediction list utilization flags array predFlagLXIvMC and prediction list utilization flag predFlagLXIvDC, and the motion vectors array mvLXIvMC and motion vector mvLXIvDC (with X being 0 or 1, respectively), and the disparity vector mvDisp. All 3 elements of the one-dimensional array IvMCIdx are initialized to -1.

  - The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - When availableFlagIvMC[ 0 ] is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC[ 0 ], IvMCIdx[ 0 ] is set to 0, and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1 and one or more of the following conditions is true,

availableFlagIvMC[ 0 ]  = = 0,

predFlagLXIvMC[ 0 ] ! = predFlagLXA1, (with X being replaced by 0 and 1),

mvLXIvMC[ 0 ] ! = mvLXA1 (with X being replaced by 0 and 1),

refIdxLXIvMC[ 0 ] ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1 and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, and one or more of the following conditions is true ,

availableFlagIvMC[ 0 ] = = 0,

predFlagLXIvMC[ 0 ] ! = predFlagLXB1, (with X being replaced by 0 and 1),

mvLXIvMC[ 0 ] ! = mvLXB1 (with X being replaced by 0 and 1),

refIdxLXIvMC[ 0 ] ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1 and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

…

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0 and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to B2 and the variable numMergeCand is increased by 1.

  - When numMergeCand is less than 5 + multi_view_mv_pred_flag, the following apply:

If availableFlagIvMC[ 1 ] is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC[ 1 ], IvMCIdx[ 1 ] is set to numMergeCand, and the variable numMergeCand is increased by 1 if one or more of the following conditions is true.

availableFlagIvMC[ 0 ]  = = 0,

predFlagLXIvMC[ 0 ] ! = availableFlagIvMC[1]  (with X being replaced by 0 and 1),

mvLXIvMC[ 0 ] ! = mvLXIvMC[ 1 ]  (with X being replaced by 0 and 1),

refIdxLXIvMC[ 0 ] ! = refIdxLXIvMC[ 1 ] (with X being replaced by 0 and 1),

otherwise, (availableFlagIvMC[1] is equal to 0) if availableFlagIvDC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvDC with the horizontal motion vector subtracted by 4 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

  - When numMergeCand is less than 5 + multi_view_mv_pred_flag, the following apply:

If availableFlagIvMC[ 2 ] is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC[ 2 ], IvMCIdx[ 1 ] is set to numMergeCand, and the variable numMergeCand is increased by 1 if one or more of the following conditions is true.

availableFlagIvMC[ 0 ]  = = 0,

predFlagLXIvMC[ 0 ] ! = availableFlagIvMC[ 2 ]  (with X being replaced by 0 and 1),

mvLXIvMC[ 0 ] ! = mvLXIvMC[ 2 ]  (with X being replaced by 0 and 1),

refIdxLXIvMC[ 0 ] ! = refIdxLXIvMC[ 2 ] (with X being replaced by 0 and 1),

otherwise, (availableFlagIvMC[ 2 ] is equal to 0) if availableFlagIvDC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvDC with the horizontal motion vector added by 4 and the variable numMergeCand is increased by 1.

- A1, if availableFlagA1 is equal to 1

…

  - The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp [Ed. (GT) There is some redundancy in draft and software since ivpMvDispLX is derived for each list, although it is always equal for both lists.]

availableFlagIvMC = = 1

MergeIdx = = 0

predFlagLXIvMC = = 1

MergeIdx = = IvMCIdx[ 0 ] and predFlagLXIvMC[ 0 ] = = 1

MergeIdx = = IvMCIdx[ 1 ] and predFlagLXIvMC[ 1 ] = = 1

MergeIdx = = IvMCIdx[ 2 ] and predFlagLXIvMC[ 2 ] = = 1

- [Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.

- G.8.5.2.1.9	Derivation process for inter-view merge candidates

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

…

Outputs of this process are (with X being 0 or 1, respectively)

a view order index refViewIdx specifying a reference view.

the availability flag array availableFlagIvMC and availability flag availableFlagIvDC specifying whether the inter-view merge candidates are available,

the reference indices index array refIdxLXIvMC and reference index refIdxLXIvDC,

the prediction list utilization flags array predFlagLXIvMC and prediction list utilization flag predFlagLXIvDC,

the motion vectors array mvLXIvMC and motion vector mvLXIvDC,

the disparity vector mvDisp.

…

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

  - The derivation process for a temporal inter-view motion vector merging candidate as specified in subclause G.8.5.2.1.15 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the disparity vector mvDisp, the prediction list indication X and the view identifier refViewIdx as the inputs and the outputs are the flag array availableFlagLXIvMC, the motion vector array mvLXIvMC and the reference index array refIdxLXIvMC.

  - The availability flag array availableFlagIvMC, and the prediction utilization flag array predFlagL0IvMC and predFlagL1IvMC are derived by

- For i being 0 to 2, the following apply:

availableFlagIvMC[ i ] = availableFlagL0IvMC[ i ] | | availableFlagL1IvMC[ i ]	(G-101)
predFlagL0IvMC [ i ] = availableFlagL0IvMC[ i ]	(G-102)
predFlagL1IvMC [ i ] = availableFlagL1IvMC[ i ] 	(G-103)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

…

- G.8.5.2.1.15	Derivation process for a temporal inter-view motion vector merging candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

…

Outputs of this process are:

a flag array availableFlagLXInterView specifying whether the inter-view motion vector candidates are is available,

a motion vector candidate array mvLXInterView (if availableFlagLXInterView is equal to 1),

a reference index array refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

All three elements of the one-dimensional array The flag availableFlagLXInterView is set equal to 0, the two-dimensional 3x2 array both components of mvLXInterView are set equal to 0, and one-dimensional array with three elements refIdxLX is set equal to −1.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

Set sftVal equal to 0.

For i being 0 to 2, the following apply:

Set sftVal equal to -16 and 16 when i is equal to 1 and 2, respectively.

The reference layer luma location ( xRef, yRef ) is derived by

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + sftVal + 2 ) >> 2 ) )	(G-127)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + sftVal + 2 ) >> 2 ))	(G-128)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component that closest (in terms of view_id) to the current view component and that is included in one of the reference lists of the current view component.]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply:

  - The variable refPredFlagLX is set equal to the prediction utilization flag predFlagLX of the prediction unit refPU.

The variable refRefIdxLX, is set equal to the reference index refIdxLX of the prediction unit refPU.

The variable refMvLX is set equal to the motion vector mvLX of the prediction unit refPU.

The variable refRefPicListLX, is set equal to the reference picture list RefPicListLX of the view component with ViewIdx equal to refViewIdx.

When refPredFlagLX is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive

When availableFlagLXInterView[ i ] is equal to 0, and the picture order count of the picture refRefPicListLX[ refRefIdxLX ] is equal to the picture order count of the picture RefPicListLX, the flag availableFlagLXInterView[ i ] is set equal to 1 and the following applies.

mvLXInterView[ i ][ 0 ] = refMvLX[ 0 ]		(G-129)
mvLXInterView[ i ] [ 1 ] = refMvLX[ 1 ]		(G-130)
refIdxLX[ i ] = i		(G-131)