| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0045 |
| --- | --- |

| Title: | 3D-CE5.h: Merge candidates derivation from disparity vector shifting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Liu He Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

When inter-view motion prediction is enabled, the current HTM design of the merge candidate list may include an inter-view predicted motion candidate before all spatial merging candidates. In addition, the disparity vector may also be converted to a disparity motion vector always following the above-right spatial candidate. As a follow-up proposal of JCT3V-B0048, it is proposed to add up to two more candidates derived with fixed horizontally shifted disparity vectors to compensate the inaccuracy of the current disparity vector. Compared to the current HTM design, the proposed method achieves compression efficiency gain of 0.2% for coded views in terms of BD rate.

# Introduction

In 3D-HEVC, up to six merging candidates are made available for selection when inter-view motion prediction is enabled. During the merge candidate list construction process, four main steps are required in current 3D-HTM when inter-view motion prediction is enabled:

- One candidate derived from inter-view motion prediction, is added to the merge candidate list if available.

As depicted in Figure 1, a corresponding block in the reference view is located based on the disparity vector [2]. The Inter-view Predicted Motion Candidate (IPMC), derived from the corresponding block, if available, is added. Note that IPMC is typically a candidate containing motion vectors referring to temporal reference pictures.

- Spatial merging candidates derived from neighbouring prediction unit (PU) partitions are added to the merge candidate list. This process is similar to that in HEVC, where redundant candidates among the initial available spatial candidates are eliminated from the merging list by comparing motion information of candidates. In addition, the spatial merging candidates derived from A1 and B1 are compared with IPMC to eliminate the redundancy. Figure 2 depicts the spatial neighbours (A1, B1, B0, A0, B2) which are the potential candidates for the merging list.

- The disparity vector is converted to a disparity motion vector and further added into the merge candidate list right after the spatial merging candidate from B0 and before the one from A0, if different from the candidates derived from A1 and B1.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

Figure 1. Derivation of inter-view predicted motion candidate for merge mode [1].

Figure 2. Spatial neighbors which are the potential candidates for the merge list. Arrows indicate which spatial candidate(s) are to be compared [2].

The current design may have a problem that the disparity vector may be not accurate enough which will result in sub-optimal candidates. Therefore, further coding efficiency improvement can be expected by including more candidates derived from inter-view motion prediction.

# Proposed Method

In JCT3V-B0048 [3], it is proposed that up to two additional candidates with shifted disparity vectors may be added. The horizontal shifted values to generate the disparity vectors are dependent on the prediction unit (PU) size of the reference block. To reduce the complexity, values of the horizontal shifts are fixed in this proposal thus independent to the PU sizes. Furthermore, the number of additional pruning operations is also reduced, i.e., from 7 to 2 for the worst case.

## Insertion of the additional candidates derived from shifted disparity vector

Two shifted disparity vectors are derived if applicable:

- Left shifted Disparity Vector (LDV): subtract the disparity vector (DV) by 16 for the horizontal component.

- Right shifted Disparity Vector (RDV): add the disparity vector by 16 for the horizontal component.

The usage of the LDV/RDV to derive a merge candidate is the same as the disparity vector in the current 3D-HEVC.

The candidates, namely left candidate and right candidate are generated. A left or right candidate is either an inter-view predicted motion vector candidate (from the reference view) or a disparity motion vector candidate.

If the inter-view predicted motion vector based on LDV or RDV is available, the motion vector candidate from the reference view is added into the merge candidate list after pruning; otherwise, a disparity motion vector converted from the following shifted disparity vector may be added into the merge candidate list as follows:

- Left candidate: the disparity motion vector candidate is set to a motion vector which has a horizontal component equal to DV[ 0 ] – 4 and vertical component equal to 0.

- Right candidate: the disparity motion vector candidate is set to a motion vector which has a horizontal component equal to DV[ 0 ] + 4 and vertical component equal to 0.

The left candidate and right candidate are inserted right before and after the temporal merging candidate, respectively.

To improve the coding efficiency while keep the complexity increase relatively small, duplicated candidates may be removed with limited additional number of pruning operations. In the proposed method, the newly added left or right candidate is only compared with the IPMC if it is an inter-view predicted motion vector candidate.

## Description of the whole process

To generate a merge candidate list, the following steps are performed in order: (newly introduced steps are highlighted)

- A disparity vector is derived and the corresponding block of current PU based on the disparity vector is identified. IPMC, if derived to be available, is added to the merge candidate list.

- Spatial merging candidates (SMC) are added to the merge candidate list as current design.

  - In addition, during SMC generation process, if a SMC derived from A1 or B1 is the same as the IPMC or the disparity motion vector candidate, they are excluded from the merge candidate list.

- Disparity motion vector converted from the disparity vector is added to the merge candidate list right after the SMC derived from B0 and before the SMC derived from A0.

- Temporal merging candidate is added to the merge candidate list as current design.

- Up to two candidates are derived from LDV and RDV and inserted into the merge candidate list right before and after the temporal merging candidate with the following pruning.

  - The left or right candidate, if it is an inter-view predicted motion vector candidate, is compared with only the IPMC in step 1, otherwise (it is a disparity motion vector candidate), is not compared with any existing candidate.

- Other motion vector merging candidates are added to the merge candidate list if the number of valid candidates is less than the maximum number.

It should be noted that the candidate list size is not changed (still six), once there are enough candidates in the candidate list, no more candidates will be further added.

# Compression Performance

This section provides simulation results of the proposed merge improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 5.0.1 [4] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [5].

As shown in Table 1, the overall average bitrate reduction is 0.2%, 0.2%, 0.2% for decoded texture views, synthesized views, coded and synthesized views, respectively.

Table 1: Coding gain with respect to anchor for 3-view case

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time |  | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% |  | 97% | 98% | 97% |
| Kendo | -0.8% | -0.8% | -0.4% | -0.3% | -0.3% |  | 95% | 98% | 104% |
| Newspapercc | -0.6% | -0.4% | -0.2% | -0.3% | -0.3% |  | 98% | 97% | 101% |
| GhostTownFly | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% |  | 97% | 109% | 106% |
| PoznanHall2 | 0.0% | -0.6% | -0.1% | -0.1% | -0.1% |  | 99% | 99% | 103% |
| PoznanStreet | -0.4% | -0.3% | -0.1% | -0.2% | -0.1% |  | 95% | 102% | 103% |
| UndoDancer | -0.5% | -0.5% | -0.2% | -0.2% | -0.3% |  | 100% | 108% | 107% |
| 1024x768 | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% |  | 97% | 98% | 101% |
| 1920x1088 | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% |  | 98% | 105% | 105% |
| average | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |  | 97% | 101% | 103% |

# References

- J. An, Y.-W. Chen, J.-L. Lin, Y.-W. Huang, and S. Lei, "3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A0049r1, 1st Meeting: Stockholm, SE, 16–20 July 2012.

- O. Bici, J. Lainema, K. Ugur, "CE9: Results of SP experiments on simplification of merge process," JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0252, Feb. 2012.

- L. Zhang, Y. Chen, L. He, "3D-CE5.h: 3D-CE5.h: Merge candidates derivation from disparity vector," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0048, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/ .

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, 2nd Meeting: Shanghai, CN, 13–19 Oct. 20122nd Meeting: Shanghai, CN, 13–19 Oct. 20122nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).