| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0096 |
| --- | --- |

| Title: | CE5.h The results on simplified illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiwook Jung Hongbin Liu Jie Jia Sehoon Yea | Tel: Email: | +82-2-6912-6477 jiwook.jung@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

In the 3rd JCT-3V meeting,  an offset model for Illumination Compensation(IC) was proposed to simplify the IC process. The offset model requires no multiplication in the parameter estimation/prediction process while retaining comparable coding efficiency with the linear model. In this proposal, results of the simplified IC using the offset model both for texture and depth codingis presented. It has BD-rate change 0.0% compared to current IC model on video only result with 100.3% encoding time, and 98.0% decoding time.

Introduction

In JCV3V-B0045[2], linear model compensation was applied to correct inter-view luminance / chrominance mismatches. When the IC flag on CU was turned on, each pixel on disparity compensated block were compensated the below equations.

Predx,y=α·Refx,y+ β

Where, Predx,y is compensated prediction samples in a block and Refx,y is prediction samples in a block before compensation is applied.

Parameter α and  β can be derived from reconstructed samples around redx,y , Refx,y. Both encoder and decoder should solve the linear least square equations to estimate α and  β. The below equations presents the linear least square solution. To reduce computing complexity, the division was replaced by multiplication, shift, and look-up-table. But, It has still multiplications of arbitrary sample values for computing cross-correlation of samples.

α= I∙i=1IPredNiRefNi- i=1IPredNi·i=1IRefNiI∙i=1IRefNiRefNi-i=1IRefNi2= A1A2

β= i=1IPredNi- αi=1IRefNiI

Where, PredNx,y , RefNx,y are left and top neighbor pixels by Predx,y , Refx,y, respectively.

In JCT3V-C0046[1], the IC on depth was adopted to compensate mismatches on inter-view depth map. The result shows IC can have benefit for inter-view depth coding.

Proposed Method

For the complexity reduction of current IC, Only offset model is applied to compensate luminance/chrominance mismatches. IC can be carried out using only additions and shifts without multiplications. The compensation model is just adding an offset as the below equation.

Predx,y=Refx,y+ β

The parameter β can be calculated by the below equation.

β= i=1IPredNi- i=1IRefNiI

There are some coding loss on the average with respect to reduce the estimation order. However, It has the advantage removing multiplications of arbitrary numbers on IC process.

Moreover, the process of motion estimation is calculated by Mean Removed SAD(MR-SAD). The search method is matched with offset model rather than linear model.

In this CE contribution, the offset model of IC is applied also on depth map.

Experimental results

The offset model is integrated into HTM-6.0. The configuration of common test condition is applied. Table1 and Table2 reportedly shows the experimental results. The simplification method is applied on texture, depth and texture & depth, respectively.

Table1. results on simplified method on only texture & depth

Although the complexity is reduced, there is coding gain in some cases, especially Poznan_Hall2 video2.

Complexity Analysis

Table4. comparison of operation number of current and proposed IC (per pixel)

| Prediction step | Current IC |  |  | Proposed IC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mul | Add | Shift | Mul | Add | Shift |
| Calculation of and for one block | ~=4·N +6 | ~=8·N +4 | ~=6 | 0 | ~=4·N +1 | ~=1 |
| Prediction for each pixel | 1 | 1 | 1 | 0 | 1 | 1 |
| Total operation number for each pixel prediction | 1+4/ N +6/ N2 | 1+8/ N +4/ N2 | 1+6/ N2 | 0 | 1+4/ N +1/ N2 | 1+1/N2 |

Table5. comparison of operation number of current and proposed IC (16x16 block example )

| Prediction step | Current IC |  |  | Proposed IC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mul | Add | Shift | Mul | Add | Shift |
| Calculation of and for 16x16 block | ~=70(34) | ~=132 | ~=6 | 0 | ~=64 | ~=1 |
| Prediction for each pixel | 1 (1) | 1 | 1 | 0 | 1 | 1 |
| Total operation number for a 16x16 block | 71(35) | 133 | 7 | 0 | 65 | 2 |

* The numbers (*) are the multiplications of cross-correlation (arbitrary samples)

Multiplication is not used for IC. And, the number of addition & shift can be reduced by around half.

WD Text

- 8.5.2.2.3.3.1 Illumination compensation based sample prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this prediction block, nPbW and nPbH,

–	two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	the width of current coding unit, nCbW,

–	neighboring samples of current coding unit p[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	availability of neighboring samples of current coding unit, avaiAboveRow, avaiLeftCol,

–	availability of neighboring samples of reference coding unit, avaiAboveRowRefL0, avaiLeftColRefL0, and avaiAboveRowRefL1, avaiLeftCol RefL1,

–	if predFlagL0 is equal to 1, neighboring samples of referenced coding unit of list 0 r0[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

–	if predFlagL1 is equal to 1, neighboring samples of referenced coding unit of list 1 r1[ x, y ], with x = -1, y = 0..nCbW−1, and y = -1, x = 0..nCbW−1,

Outputs of this process are:

–	the (nPbW)x(nPbH) array predSamples of prediction sample values.

Derive nS0 and nS1 using following two order steps, with idx in the following variables nSidx, avaiAboveRowRefLidx and avaiLeftColRefLidx replaced by 0 and 1:

1.  if avaiAboveRowRefLidx is equal to 0 or avaiAboveRow is equal to 0, nSidx is set to 1; otherwise, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, nSidx is set to 0,

2.  if avaiLeftColRefLidx is equal to 0 or avaiLeftCol is equal to 0, nSidx is set to nSidx+1; otherwise, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, nSidx is set to nSidx,

The parameters of linear illumination compensation model are derived as the following ordered steps. if predFlagL0 is equal to 1 and nS0 is not equal to 2, (ica0, ick0, icb0) used for list 0 is derived by replacing idx in avaiAboveRowRefLidx, avaiLeftColRefLidx, nSidx, ridx, icaidx, ickidx and icbidx with 0; otherwise, set ica0 to 1, ick0 to 0 and icb0 to 0. if predFlagL1 is equal to 1 and nS0 is not equal to 2, (ica1, ick1, icb1) used for list 1 is derived by replacing idx in avaiAboveRowRefLidx, avaiLeftColRefLidx, nSidx, ridx, icaidx, ickidx and icbidx with 1; otherwise, set a1 to 1, k1 to 0 and b1 to 0.

- Variable k3 is derived as:

k3 = Max( 0, BitDepth + Log2( nCbW >> nSidx ) − 14 )	(8-xxx)

- Variables R, P and k2 are derived as follows:

  - If avaiAboveRowRefLidx is equal to 1, avaiAboveRow is equal to 1, avaiLeftColRefLidx is equal to 1 and avaiLeftCol is equal to 1

R =		(8-xxx)

P =		(8-xxx)

  - Else, if avaiAboveRowRefLidx is equal to 1 and avaiAboveRow is equal to 1, and avaiLeftColRefLidx is equal to 0 or avaiLeftCol is equal to 0,

R =		 (8-xxx)

P =		(8-xxx)

  - Else, if avaiAboveRowRefLidx is equal to 0 or avaiAboveRow is equal to 0, and avaiLeftColRefLidx is equal to 1 and avaiLeftCol is equal to 1,

R =		(8-xxx)

P =		(8-xxx)

k2 = Log2( (2*(nCbW >> nSidx)) >> k3 )		(8-xxx)

- Variables icaidx, icbidx and ickcidx are derived as:

icbidx	= ( R – P ) + ( 1 << ( k2 − 1 ) ) ) >> k2		(8-xxx)

The prediction sample predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived by:

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( (predSamplesL0[ x ][ y ] + offset1 ) >> shift1) + icb0 ) 	(8-xxx)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x ][ y ] are derived by

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, ( ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1) + icb1 )	(8-xxx)

–	Otherwise, the final predicted sample values predSamples[ x ][ y ] are derived by

predSamples[ x ][ y ] = Clip3( 0, ( 1 << bitDepth ) − 1, 
	 predSamplesL0[ x ][ y ]   +  predSamplesL1[ x ][ y ]  + offset2 +
	  ( (icb0 + icb1 ) << shift1) ) >> shift2 )	(8-xxx)

Conclusion

In this contribution, a method to use simple offset model is proposed to simplify IC process. The current method for IC is relatively complex because the process contains multiplications of arbitrary samples. In this proposal, the process to calculate least square solution is replaced with the calculation of the difference of mean values. The proposed method has the benefits of multiplication-free and matching with RD-metric. The result reportedly shows the simplified method has negligible change on the average coding result.

It is recommended that proposed method is adopted into 3D-HEVC.

Reference

- J. Jung, H. Liu, etc., (LG), "Non-CE Simplification of Illumination Compensation", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-C0111, Geneva, Switzerland, January, 2013

- H. Liu, J. Jung, J. Sung, etc., (LG), "3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-B0045, Shanghai, China, October, 2012.

- H. Liu, J. Jung, J. Sung, etc., (LG), "CE5.h related : Bug Fix and Extension of Illumination Compensation", Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-C0046, Geneva, Switzerland, January, 2013.

Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).