| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0232 |
| --- | --- |

| Title: | Multi-layer MVD data coding for DIBR of complex 3D scenes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Jyranmaankatu 33, Nokia, Finland. | Tel: Email: | +358408235996 dmytro.rusanovskyy@ieee.org |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document consider coding of multilayer MVD data for purposes of enabling DIBR of complex 3D video scenes with transparent and semi-transparent objects through utilization of multi-layer MVD plus Alpha scene representation (MVD+A).

It is proposed to extend the 3DV test set and CTC to include 3D scene with transparent or semi-transparent objects with (MVD+A) data representation.

It is also proposed to establish AhG to study DIBR performance and coding approaches for such type of data.

It is also proposed to enable in 3DV standards support for coding of multi-layer MVD+A data represented in YUV color pipeline (depth and alpha information combined into a single YUV container).

# Introduction

Utilization of the multi-view video plus depth (MVD) format for representing 3D video scenes is a commonly accepted approach to enable DIBR in 3D video systems. MVD describes the 3D scene from multiple viewing angles through a combination of pixel-wise associated video (texture) and depth map samples. However, conventional MVD format has inherent limitation on describing 3D scenes with transparent or semi-transparent objects, since no multi-layer depth representation, nor information on transparency is considered in the format.

To enable DIBR rendering of 3D scenes with transparent or semi-transparent objects, it is proposed to utilize multi-layer MVD data combined with information on transparency (alpha channel) of each of the layers. In this document we consider currently available alternatives to enable DIBR of such scenes and propose to combine depth and alpha information into a date format (MVD+A).

# Description

Let's assume a 3D scene consisting of a semi-transparent object (red block) in the foreground of a background object (yellow block). Multi-view representation of such scene is visualized in Figure 1.

Conventional MVD data (and typical MVD content in 3DV test set) may be interpreted as that scene is fully described with a single layer MVD data, where each texture sample is associated with its depth value. In such assumption,  scene with semi-transparent object cannot be described adequately.

Consider in Figure 1, texture sample representing of fusion of red object and white background (left part of semi-transparent object) in view V0. This area can be described in MVD either as a foreground object with elements of white background projected on it, or as a background object with projection of the red object on it. This scene description from view 0 obviously would contradict to the scene description from the view V1, where foreground object would be filled with yellow textures of background object, and the white background would have no trace of red object projection on it. With such description, DIBR is likely to fail in generation adequate virtual views.

V1

V0

Figure 1. Visualization of multi-view representation of 3D scene with semi-transparent object.

In order to overcome this problem and to enable DIBR of such scenes, a multilayer MVD representation of the 3D scene should be utilized. 3D scene is decomposed in certain number of layers, with number of layers describing the complexity of the scene. Each of the layer is described in MVD representation, namely texture information associated with layer and depth. To enable composing multiple layers in the case of transparent and/or semi-transparent objects, a foreground layers of scene representation should include alpha (transparency) information as well. See Figure 2, where Layer 0 consists of MVD (texture+depth) representation of the background and Layer 1 consists of MVD representation of foreground (L1) and alpha information (A1).  In this document, we call this data MVD+A and Figure 3 presents example of scene description with MVC+D for 2 layers and 2-view scenario.

V0

V1

L0 (MVD)

L1(MVD)

A1

Figure 2. Visualization of multi-layer multi-view MVD+A representation of 3D scene with semi-transparent object.

Figure 3. MVD+A representation of 3D scene with 2 layers and 2 views.

The MVD+A data can be coded with any existing coding standards without changes to syntax or semantic, if representation in YUV color format is achieved, as shown in Figure 4. In such representation, texture, depth and alpha are coded as independent streams, and would require a separate coding pipeline.

Issues of efficient coding of texture and depth data has been addressed in work of JCT-3V study group. The alpha channel coding, on contrast was not considered, since alpha is traditionally coded as sequence of auxiliary pictures.

Figure 4. Example of MVD+A data representation in YUV color format for  a single layer.

However, conventional approach for coding alpha channel through the use of auxiliary picture may be not optimal in the case of multi-view/multi-layer scene description. First of all, such coding configuration would require a separate coding entity, which would lead to system complexity increase. Secondly, signaling overhead (such as SPS, PPS, slice header and camera parameters) may be quite significant compare to a total number of bits required for alpha information. And finally, coding of alpha information through a separate coding entity would not allow to utilize cross-component redundancy, e.g. motion information sharing between texture/depth and alpha data.

Considering similarity in natures of depth and alpha information (low variation of signal, object-description)  and taking into account that depth map information is currently coded with YUV 4:0:0 color format, we propose to signal depth and alpha information jointly, through YUV 4:X:X representation, where X not equal to 0, as shown in Figure 5.

Such data arrangement, would allow coding of MVD+A data without changes to system design and without additional bit overhead for signaling of alpha information.

Figure 5. Visualization of the MVD+A data format with alpha information coded jointly with depth map.

Moreover, a complete multi-layer MVD+A representation represented in Figure 5 may be found redundant for most typical scenes featuring a single layer with transparent objects in the foreground. In those cases, when alpha information for background is not necessary and scene can be represented with 2 layers of MVD data, we can combine depth information for 2 layers and alpha information for a foreground and represent this information with conventional YUV 4:X:X, e.g. 4:2:0 color format, as shown in Figure 6.

In Figure 6, MVD+A data for 2 layers and 2 views description of 3D scene is visualized. DX-0 defines depth map information for a layer 0 (background), and DX-1 defines depth map information for a layer 1 (foreground). The alpha information specifies the transparency of the this foreground layer.

Obviously, proposed data arrangement limits number of overlapping transparent object in the same spatial location of each view to 1. However, considering the nature of typical 3D scenery, such limitation may be found not sever, and arrangement would suite to most typical use-cases.

Figure 6. Non-complete MVD+A representation of a 3D scene with 2 layers and 2 views.

# Conclusion

It is proposed to extend the 3DV test set and CTC to include 3D scene with transparent or semi-transparent objects with (MVD+A) data representation.

It is also proposed to establish AhG to study DIBR performance and coding approaches for such type of data.

It is also proposed to enable in 3DV standards support for coding of multi-layer MVD+A data represented in YUV color pipeline (depth and alpha information combined into a single YUV container).

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).