| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-I0111 |
| --- | --- |

| Title: | AHG7: Complexity assessment of inter-prediction tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports a complexity assessment of inter prediction tools in terms of number of operation and memory bandwidth. This report has changes from the former report of JCT3V-G0208 in the following:

- Include memory bandwidth analysis instead of data transfer rate analysis

- Include depth access for memory bandwidth calculation

- Consider memory pattern for memory bandwidth calculation

- Add DBBP to the target tools

It is recommended to keep complexity in check and to improve the balance between performance and complexity until 3D-HEVC finalization.

Introduction

This complexity analysis includes depth access. Please recognize the following assumption about depth access.

About depth access

DoNBDV, VSP and DBBP needs four corner depth of CU, four corner depth of 4x8/8x4 subblock and whole depth respectively. While some implementation may fetch four corner depth of CU for DoNBDV, it can be safe to say that some implementation may fetch whole depth pixel of the CU which can be used in VSP and DBBP.

Therefore, partial depth access case (partial depth access) and full depth access case for DoNBDV are investigated.

Results of complexity analisys

In terms of number of operations, the followings are observed.

- MC, ARP and VSP doesn’t increase number of operations

- IC shows 105 % number of operations

- DBBP shows 127 % number of operations

In terms of number of operations, the followings are observed.

- MC and IC shows 101% to 106% (110 % to 114 %) memory bandwidth

- ARP shows 122 % to 164 % (132 % to 172 %) memory bandwidth

- DBBP shows 111 % to 119 % (120 % to 128 %) memory bandwidth

- VSP doesn’t increase memory bandwidth

Table 1. Summary of complexity

|  |  | Number of operation [%] |  | Data transfer rate [%] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3D-HEVC |  | Mult [%] | Add [%] | Partial depth access in DoNBDV |  |  |  | Full depth access in DoNBDV |  |  |  |
|  |  |  |  | 1x1 | 4x2 | 4x4 | 8x2 | 1x1 | 4x2 | 4x4 | 8x2 |
|  | MC | 100% | 100% | 101% | 103% | 105% | 106% | 110% | 113% | 111% | 114% |
|  | ARP | 46% | 38% | 122% | 138% | 133% | 164% | 132% | 148% | 139% | 172% |
|  | IC | 105% | 103% | 101% | 103% | 105% | 106% | 110% | 113% | 111% | 114% |
|  | VSP | 18% | 17% | 41% | 60% | 60% | 78% | 50% | 70% | 65% | 94% |
|  | DBBP | 100% | 127% | 111% | 116% | 116% | 119% | 120% | 125% | 123% | 128% |

Table 2. Summary of complexity (Partial depth access in DoNBDV, 1x1 memory pattern)

| 3D-HEVC |  | CU | PU | Mult [%] | Add [%] | TotalBandwidth [%] |
| --- | --- | --- | --- | --- | --- | --- |
|  | MC | 64x64 | 64x64 | 74% | 75% | 36% |
|  |  | 32x32 | 32x32 | 78% | 78% | 43% |
|  |  | 16x16 | 16x16 | 85% | 86% | 60% |
|  |  | 8x8 | 8x8 | 100% | 100% | 101% |
|  | Sub-block (8x8) | 64x64 | 64x64 | 100% | 100% | 100% |
|  |  | 32x32 | 32x32 | 100% | 100% | 100% |
|  |  | 16x16 | 16x16 | 100% | 100% | 100% |
|  |  | 8x8 | 8x8 | 100% | 100% | 101% |
|  | ARP | 64x64 | 64x64 | 43% | 36% | 93% |
|  |  | 32x32 | 32x32 | 43% | 37% | 97% |
|  |  | 16x16 | 16x16 | 44% | 37% | 105% |
|  |  | 8x8 | 8x8 | 46% | 38% | 122% |
|  | IC | 64x64 | 64x64 | 79% | 78% | 36% |
|  |  | 32x32 | 32x32 | 83% | 82% | 43% |
|  |  | 16x16 | 16x16 | 91% | 90% | 60% |
|  |  | 8x8 | 8x8 | 105% | 103% | 101% |
|  | VSP | 64x64 | 64x64 | 18% | 17% | 41% |
|  |  | 32x32 | 32x32 | 18% | 17% | 41% |
|  |  | 16x16 | 16x16 | 18% | 17% | 41% |
|  |  | 8x8 | 8x8 | 18% | 17% | 41% |
|  | DBBP | 64x64 | 64x64 | 74% | 102% | 46% |
|  |  | 32x32 | 32x32 | 78% | 105% | 53% |
|  |  | 16x16 | 16x16 | 85% | 113% | 70% |
|  |  | 8x8 | 8x8 | 100% | 127% | 111% |

Table 3. Summary of complexity (Full depth access in DoNBDV, 8x2 memory pattern)

| 3D-HEVC |  | CU | PU | Mult [%] | Add [%] | TotalBandwidth [%] |
| --- | --- | --- | --- | --- | --- | --- |
|  | MC | 64x64 | 64x64 | 74% | 75% | 30% |
|  |  | 32x32 | 32x32 | 78% | 78% | 39% |
|  |  | 16x16 | 16x16 | 85% | 86% | 59% |
|  |  | 8x8 | 8x8 | 100% | 100% | 114% |
|  | Sub-block (8x8) | 64x64 | 64x64 | 100% | 100% | 106% |
|  |  | 32x32 | 32x32 | 100% | 100% | 107% |
|  |  | 16x16 | 16x16 | 100% | 100% | 109% |
|  |  | 8x8 | 8x8 | 100% | 100% | 114% |
|  | ARP | 64x64 | 64x64 | 43% | 36% | 67% |
|  |  | 32x32 | 32x32 | 43% | 37% | 80% |
|  |  | 16x16 | 16x16 | 44% | 37% | 107% |
|  |  | 8x8 | 8x8 | 46% | 38% | 172% |
|  | IC | 64x64 | 64x64 | 79% | 78% | 30% |
|  |  | 32x32 | 32x32 | 83% | 82% | 39% |
|  |  | 16x16 | 16x16 | 91% | 90% | 59% |
|  |  | 8x8 | 8x8 | 105% | 103% | 114% |
|  | VSP | 64x64 | 64x64 | 18% | 17% | 73% |
|  |  | 32x32 | 32x32 | 18% | 17% | 74% |
|  |  | 16x16 | 16x16 | 18% | 17% | 76% |
|  |  | 8x8 | 8x8 | 18% | 17% | 81% |
|  | DBBP | 64x64 | 64x64 | 74% | 102% | 36% |
|  |  | 32x32 | 32x32 | 78% | 105% | 46% |
|  |  | 16x16 | 16x16 | 85% | 113% | 69% |
|  |  | 8x8 | 8x8 | 100% | 127% | 128% |

References

[1] T. Ikai, “AHG7: Complexity assessment of motion compensation process”, JCT3V-G0208, JCT3V 7th Meeting: San José,  US, 11–17 Jan. 2014