| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G166 |
| --- | --- |

| Title: | AHG21: Explicit Reference Pictures Signaling with Output Latency Count Scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Park Byeongmoon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-7738-9288 hendry.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

In the 6th JCTVC meeting, JCTVC-F493 proposed to signal reference pictures that are required by current and future pictures explicitly. The scheme is being further studied and developed in AHG21. This contribution proposes a variation to the scheme proposed in F493 in an attempt to improve it. The proposed scheme suggests signaling only reference pictures that are required by current pictures. Furthermore, a new syntax element so called Output Latency Count is introduced to determine when each reference picture should be marked as “unused for reference”. It is claimed that the proposed scheme can signal the list of reference pictures more efficiently and also by using Output Latency Count and signaling only reference pictures required by current picture, the proposed scheme separates tools for reference picture marking process and error resilience feature so that encoder may have more flexibility to decide whether it wants to use them both. Comparison of the proposed signaling scheme and signaling scheme that is currently studied AHG21 is also provided.

Introduction

Explicit Reference Picture Signaling (ERPS) was proposed in JCTVC-F493 [2] and currently being further discussed and developed in AHG21. The current ERPS (i.e., the ERPS scheme that was proposed in [2] and currently being further studied in AHG21) can be summarized as follows:

- In each slice header, except header of IDR slices, a set of reference pictures, called reference picture set (RPS), is signal to indicate the list of reference pictures that are required by current picture and future pictures.

- Each reference picture in RPS is signal by its delta POC relative to the POC of the current picture. Furthermore, there is a flag to indicate whether the reference picture is used by current picture.

- After parsing slice header of the current picture, but before decoding process begins, decoder performs reference picture marking process. Reference pictures in Decoded Picture Buffer (DPB) that are not listed in RPS shall be marked as “unused for reference”

- For bitstreams that have structure repetition, RPS can be signaled in PPS.

Problems with Current ERPS Scheme

Inefficiency for signaling long term reference pictures

In addition to signaling reference pictures required by current picture, current ERPS scheme also has to signal reference pictures that are required by future pictures. If a reference picture has to be kept for long time in DPB, it is inevitable that current ERPS has to keep on signaling it in every slice header until the reference pictures can finally marked as “unused for reference”.

Furthermore, the longer a reference picture stays in DPB, the bigger its delta POC relative to POC of current picture and thus will require more bits to signal it. Such situation can be clearly observed when long term reference pictures are used.

Tight coupling of reference picture marking process and error resiliency

The design of the current ERPS has consequence such that reference picture marking process and error resilience feature are tightly coupled because both use RPS in order to operate. For just picture marking process, encoder has no other choice but signaling all required reference pictures. There are cases where encoder might not need error resilience feature provided by current ERPS scheme.

For example, in low delay setting where 3 higher quality and 1 recent reference pictures are used for inter prediction, if buffer management scheme can manage to release reference picture in DPB as soon as it is not needed anymore, the number of reference pictures in DPB at each time shall always be equal to max_num_ref_frames + 1 (i.e., current pictures and reference pictures required by current pictures), except for max_num_ref_frame first frames in the bitstreams as shown in Table 1

Table 1 – Ideal DPB management in common test condition for LD

| POC of received pic | DPB content |  |  |
| --- | --- | --- | --- |
|  | Before decoding | After decoding | Released ref pic |
| 0 | - | 0 | - |
| 1 | 0 | 0,1 | - |
| 2 | 0,1 | 0,1,2 | - |
| 3 | 0,1,2 | 0,1,2,3 | - |
| 4 | 0,1,2,3 | 0,1,2,3,4 | - |
| 5 | 0,1,2,3,4 | 0,2,3,4,5 | 1 |
| 6 | 0,2,3,4,5 | 0,3,4,5,6 | 2 |
| 7 | 0,3,4,5,6 | 0,4,5,6,7 | 3 |
| 8 | 0,4,5,6,7 | 0,4,6,7,8 | 5 |
| 9 | 0,4,6,7,8 | 0,4,7,8,9 | 6 |
| 10 | 0,4,7,8,9 | 0,4,8,9,10 | 7 |
| 11 | 0,4,8,9,10 | 0,4,8,10,11 | 9 |
| 12 | 0,4,8,10,11 | 0,4,8,11,12 | 10 |
| 13 | 0,4,8,11,12 | 0,4,8,12,13 | 11 |
| 14 | 0,4,8,12,13 | 4,8,12,13,14 | 0 |
| 15 | 4,8,12,13,14 | 4,8,12,14,15 | 13 |
| 16 | 4,8,12,14,15 | 4,8,12,15,16 | 14 |
| 17 | 4,8,12,15,16 | 4,8,12,16,17 | 15 |
| 18 | 4,8,12,16,17 | 8,12,16,17,18 | 4 |
| … | … | … | … |

Thus, RPS is not needed since picture loss can easily be detected. For example, suppose that picture 9 (i.e., picture with POC 9) is lost and never be received, decoded, and stored in DPB. When picture 10 is received, decoder expects that DPB contains picture 9 (i.e., recent reference picture for picture 10) and picture 0, 4, and 8 (which are high quality reference picture). Note that decoder can easily calculate that picture 10 needs picture 0, 4, and 8 since it knows the GOP size is 8 (picture 0 and 8 are key pictures before picture 10 and picture 4 is the picture in the middle of those key pictures).

In the above situation, RPS is not necessary to be signaled because any picture loss can easily be detected. Therefore, ERPS scheme that mandate encoder to always signal RPS is not necessary and we can save bits by not signaling RPS. All that are required in the above situation is information to tell decoder when a reference picture must be released from DPB.

Proposed ERPS Scheme

Design objectives

There are two main objectives in the design of the proposed ERPS scheme:

- We would like to reduce number of bits required to signal RPS.

- We would like to separate information that are needed to do reference picture marking process and information that are needed to improve error resiliency.

In order to achieve those objectives, the proposed ERPS scheme is design as follow:

- The RPS signaled by the proposed ERPS signal contains only reference pictures that are required to decode current picture. This new RPS is used as information to improve error resiliency since any lost can be detected as soon as possible.

- In first slice header of each picture, a new element called “Output Latency Count” (OLC) is added to inform decoder of when the picture should be release from DPB. This new element is used as information to assist reference picture marking process. OLC is the delta between POC / frame number of current picture and the POC / frame number where current picture should be released from DPB.

By sending only reference pictures that are required by current picture, the proposed ERPS scheme certainly reduces the amount of bits required to signal RPS compared to current ERPS scheme. However, the addition of OLC and necessity to also signal frame number adds overhead bits for the proposed ERPS scheme since current ERPS does not have to signal them. Overhead bits comparison between the proposed and current ERPS scheme is given in section x (editor note. Comparison will be given in the later revision).

Proposed syntax and semantics

Table 2 tabulates the proposed syntax to signal RPS.

Table 2 – Syntax for RPS

| ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_ref_pics | ue(v) |
| for( i = 0; i < num_ref_pics; i++ ) { |  |
| sign_bit_of_ref_pic[ i ] | ue(v) |
| delta_poc_minus1[ i ] | u(1) |
| } |  |
| } |  |

num_ref_pics specifies the number of reference frames signaled in this RPS

sign_bit_of_ ref_pic[i] specifies sign bit of the i-th reference frame. 1 indicates positive sign while 0 indicates otherwise.

delta_poc_minus1[i] plus 1 specifies the absolute delta POC of the i-th reference frames and the POC of current picture.

Table 3 tabulates the proposed syntax to signal RPS in PPS

Table 3 – Syntax for signaling RPS in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| erps_flag | f(1) |
| if (erps_flag) { |  |
| num_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_ref_pic_sets; idx++) |  |
| ref_pic_set( idx ) |  |
| } | ue(v) |
| } |  |
| … |  |
| } |  |

erps _flag specifies whether the reference pictures are signaled explicitly. erps_flag equals 1 indicates that RPS is signaled explicitly and erps_flag equals 0 otherwise.

num_ref_pic_sets specifies the number of RPS sets signaled in this PPS

Table 4 tabulates the proposed syntax to signal OLC and RPS in slice header

Table 4 – Syntax for signaling RPS in PPS

| slice_header ( ) { | Descriptor |
| --- | --- |
| … |  |
| if (!lightweight_slice_flag) { |  |
| if (nal_ref_flag == 1) { |  |
| if (!use_LD) { |  |
| frame_num | u(v) |
| } |  |
| olc_minus2 | ue(v) |
| } |  |
| If ((slice_type == P ¦¦ slice_type == B) && erps_flag == 1) { |  |
| ref_pic_set_idx_plus1 | f(1) |
| if (ref_pic_set_idx_plus1 > 0) { //not using RPS in PPS |  |
| ref_pic_set( idx ) |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

frame_num as semantic specified in WD 4 of HEVC.

olc_minus2 plus 2 specifies the relative indicates how long the current picture shall stay in the DPB before it can be marked as “unused for reference”. For bitstream with low delay settings, OLC is relative to POC, otherwise, it is relative to frame_num.

If (nal_ref_flag == 0)

OutputLatencyCount = 0

Mark current picture as “unused for reference”

Else if (nal_ref_flag == 0 && !use_LD)

OutputLatencyCount = olc_minus2 + 2 + PicOrderCnt(CurrPic)

Mark current picture as “used for reference”

Else

OutputLatencyCount = olc_minus2 + 2 + frame_num(CurrPic)

Mark current picture as “used for reference”

Reference Frame Marking Process

This initialization process is invoked after decoding the first slice header of every frame and updating OutputLatencyCount of ExistingReferenceFrame in DPB.

If (!use_LD) {

For i = 0 to number of reference frames in DPB, do:

If (reference_frame[i] != CurrPic frame && OutputLatencyCount[i] > 0 
&& (OutputLatencyCount[i] – frame_num(CurrPic)) <= 0)

Mark reference_frame[i] as “unused for reference”

}

Else

{

For i = 0 to number of reference frames in DPB, do:

If (reference_frame[i] != CurrPic frame && OutputLatencyCount[i] > 0 
&& (OutputLatencyCount[i] – PicOrderCnt(CurrPic)) <= 0)

Mark reference_frame[i] as “unused for reference”

}

Discussion

Proposed ERPS Scheme vs. Picture Loss / Temporal Layer Removal

Since the proposed ERPS scheme separate the information to assist reference picture marking process and information to improve error resilience, the robustness of the proposed ERPS scheme to handle reference picture marking process might in questioned.

The following examples show the robustness of the OLC in the proposed ERPS scheme against picture loss and temporal layer removal.

Example 1 – Normal decoding, No picture loss & no temporal layer removal

Example 1 gives an example of decoding and reference frame marking process when all frames are received normally (no extracted temporal layer, no frame loss). Figure 1 illustrates the structure of bitstream for the example. In brief, the marking process is as follows:

- Frame 20 (i.e., frame with POC 20) can be released from buffer before decoding of frame with frame number 27 (that is 20 + 5 + 2) starts.

- Frame 21 can be released from buffer before decoding of frame 24 starts.

- Frame 22 can be released from buffer before decoding of frame 24 starts.

- Frame 23 can be released from buffer before decoding of frame 26 starts.

- Frame 24 can be released from buffer before decoding of frame 31 starts.

- Frame 25 can be released from buffer before decoding of frame 28 starts.

- Frame 26 can be released from buffer before decoding of frame 28 starts.

- And so forth…

Figure 1 – Bitstream structure for example 1, 2, and 3

Example 2 – Picture loss occurs

By using the structure shown in Figure 1, assume that:

- Frame 23 is lost: no action is needed. Reference picture marking process of other frames will be be affected. The lost of frame 23 will be detected when frame 25 is received.

- Frame 24 is lost: Frame 21 and 22 that supposed to be released from DPB when frame 24 is received can be released from DPB as soon as the next frame (i.e., frame 25) is received. The lost of frame 24 will be detected when frame 25 is received.

Example 3 – Temporal layer removal occurs

By using the structure shown in Figure 1, assume that the highest temporal layer is removed so that frame 21, 23, 25, and 27 are not received by decoder. Both decoding process and reference picture marking process will not be affected

Simulation Results

The proposed ERPS scheme has been implemented on top of HM-4.0 and the performance was measured under common test condition for RAHE, RALC, LBHE, and LBLC configuration. The results are shown in Table 5 and Table 6. The simulation for ERPS is conducted by signaling RPS in slice header, not in PPS. Furthermore, RPS is only signaled in random access settings (RAHE and RALC) while for low delay settings (LBHE and LBLC), we only signal OLC.

Overall, the overhead bit for the proposed ERPS scheme is 0.1% for random access settings. The overhead is relatively more visible for sequence with smaller resolution (class C and D).

Table 5 – Simulation result for RAHE & RALC – Anchor HM-4.0 common test condition

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 6 – Simulation result for LBHE & LBLC – Anchor HM-4.0 common test condition

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Conclusion

This contribution proposes an explicit reference picture signaling (ERPS) scheme for reference picture management of HEVC. The proposed scheme can be considered as a modified version of the current ERPS that is being discussed and developed in AHG21. We would like to recommend that the JCTVC considers the proposed ERPS scheme and further discussed it in the scope of AHG21.

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

- JCTVC-F803_d5, “WD4: Working Draft 4 of High-Efficiency Video Coding,” 6th JCT-VC Meeting, 6th Meeting: Torino, IT, 14-22 July, 2011.

- JCTVC-F493, “Absolute signaling of reference pictures,” 6th JCT-VC Meeting, 6th Meeting: Torino, IT, 14-22 July, 2011.