| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G383 |
| --- | --- |

| Title: | CE12 Subtest1: Deblocking of New Non-Square Blocks: Edge Shift for AMP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Xianglin Wang, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | geertv@qualcomm.com xianglin@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal addresses the adaptation of the deblocking filter in case of AMP partitions of size 16x4 or 4x16. Instead of deblocking the central edge on the 8x8 deblocking grid inside the 16x16 CU of the AMP type, the relevant internal AMP partition edge is deblocked, which keeps the number of filtering operations unchanged. The deblocking filter width is adapted to avoid filtering dependencies between nearby edges. The BD-rates and execution times are very similar to the HM4 anchor.

# Problem Statement

AMP (asymmetric motion partitioning) [1] splits CUs into non-square partitions such as 64x16, 64x48, 16x64, 48x64, 32x8, 32x24, 8x32, 24x32, 16x4, 16x12, 4x16, 12x16. The present HM4 deblocking filter, processes blocking artifacts on the 8x8 grid and, therefore, does not deblock artifact edges located at positions that are not multiples of 8 pixel locations. In addition, the HM deblocking filter supports parallel implementations of deblocking decisions and independent filtering for vertical or horizontal edges. The problem is that the non-square partitions of AMP that have a dimension smaller than the minimum size for deblocking of 8x8, such as the 16x4 and 4x16 PUs, are not appropriately processed by the HM4 deblocking filter, while block edge artifacts may be visible due to quantization, and differences in motion vectors and reference pictures of adjacent partitions. In addition, the present HM4 deblocking decisions and filters have supports (number of pixels used for computations and/or number of pixels to be modified) that maximally employ 4 pixels on each side of the edge. In case of a non-square 16x4 or 4x16 partition, the deblocking filters potentially overlap and dependencies are introduced that break the HM4 deblocking filter parallelism.

# Proposed Deblocking Filter Adaptation

As an example of deblocking filter adaptation, Figure 1 illustrates AMP cases for left/right neighboring blocks P and Q (similar cases for top/bottom P and Q). If one block (P or Q) is a 16x16 CU of the AMP type, while the other block is any other intra/inter type, including 16x16 CU of the AMP type, the deblocking filter of the common edge between blocks P and Q requires adaptation when there is at least one adjacent 16x4 or 4x16 AMP partition that is parallel with the edge between blocks P and Q (Figure 1c,f,g,h,i). The arrows in Figure 1 indicate the edges, including the internal AMP partition edge, for which the deblocking filter requires adaptation in order to avoid filtering dependencies between nearby edges.

Figure 1 Examples of neighboring blocks P and Q (left/right; similar for top/bottom) with at least one of them a 16x16 CU of the AMP type (except case (a)) with one 16x4 or 4x16 partition and corresponding 16x12 or 12x16 partition. The arrows indicate the edges for which the deblocking filter is adapted in order to avoid filtering dependencies between nearby edges.

The boundary strength and deblocking on/off decision from HM4 are applied to the edge. The on/off decision is computed based on 3 samples on both sides of the edge for locations 2 and 5 along the edge of length 8 samples. In case the current edge requires deblocking, the following deblocking filter is applied, which is based on the weak luma filter from HM4:

=  ( 9*(q0 - p0) - 3*(q1 - p1) + 8)/16

= Clip( -tc, tc,  )      (tc is HM4 deblocking threshold)

If abs() < 10*tc, then:

p0’ = p0 +

q0’ = q0 -

This filter modifies one sample on both sides of the current edge. The filter is computed based on two samples on both sides of the edge (). This way dependency between the filtering of nearby edges is avoided and the parallel filtering property is preserved.

Observe that the internal AMP edge is deblocked, while the central edge on the 8x8 deblocking grid is not deblocked. This way, the number of filtering operations is similar to the anchor, while the relevant AMP edge is processed.

# Objective Results

The BD-rate results are obtained following the common test conditions for HM4. The encoding times are measured in a heterogeneous computing environment, while the decoding times are measured on a single CPU.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | -0.3% |
| Class E | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.2% | -0.5% | 0.0% | -0.1% | -0.2% |
| Class E | -0.1% | 0.4% | -0.1% | 0.0% | -0.3% | 0.0% |
| Overall | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Subjective Results

# Figures 2 and 3 illustrate the subjective quality improvement of the proposed method compared with the HM4.0 anchor. Examples are selected from the class B sequence “Kimono” coded with low delay (LC) test conditions and QP = 32. In both examples block artifacts are reduced by the proposed method.

| - HM4.0 | - Proposal |
| --- | --- |

# Figure 2 Frame 54 from the “Kimono” sequence (low delay LC, QP=32).

| - HM4.0 | - Proposal |
| --- | --- |

# Figure 3 Frame 80 from the “Kimono” sequence (low delay LC, QP=32).

# Source Code

The software implementation utilizes the boundary strength array to indicate what type of adaptation the deblocking filter needs to perform in case of AMP 16x4 partitions. This is an implementation choice and it is not the intention to introduce boundary strength changes.

# Conclusion

This contribution proposed to adapt the deblocking filter in case of 16x4 AMP partitions. The AMP edge that is inside the 16x16 CU is not deblocking filtered in HM4. It is proposed to shift the deblocking from the central edge to the actual 16x4 partition edge without increasing the number of filtering operations. In addition, it is proposed to use the one sample weak filter in order to avoid introducing dependencies between nearby edges. The BD-rate and execution times are similar to the HM4 anchor. Subjective quality improvement of blocking artifacts is illustrated.

# References

[1]  I.-K Kim, W.-J Han, J. H. Park (Samsung), X. Zheng (HiSilicon), “CE2: Test result of asymmetric motion partition (AMP),” 6th JCT-VC Meeting, Torino, Italy, July 2011, Doc. JCTVC-F379

# Working Draft

# The following working draft description is based on WD4_d5 (JCTVC-F803).

# “8.6.1.2 Derivation process of prediction unit boundary

Inputs of this process are:

–	a variable log2CUSize specifying the coding unit size,

–	a prediction partition mode PartMode.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags, verEdgeFlags, horAMPEdgeFlags, verAMPEdgeFlags.

Depending on PartMode, the following applies:

–	If PartMode is equal to PART_2NxN or PART_NxN, horEdgeFlags[ k ][ 1 << ( log2CUSize – 1 ) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_Nx2N or PART_NxN, verEdgeFlags[ 1 << ( log2CUSize – 1 ) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_2NxnU and FilterInternalEdgesFlag is equal to 1, for k = 0.. ( 1 << log2CUSize ) – 1,

- horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ] is set equal to 1,

- horAMPEdgeFlags[ k ][ (1 << ( log2CUSize – 1) ] is set equal to 1.

–	If PartMode is equal to PART_2NxnD and FilterInternalEdgesFlag is equal to 1, for k = 0.. ( 1 << log2CUSize ) – 1

- horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ] is set equal to 1,

- horAMPEdgeFlags[ k ][ (1 << ( log2CUSize – 1) ] is set equal to 2.

–	If PartMode is equal to PART_nLx2N and FilterInternalEdgesFlag is equal to 1, for k = 0.. ( 1 << log2CUSize ) – 1

- verEdgeFlags[ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ][ k ] is set equal to 1,

- verAMPEdgeFlags[ k ][ (1 << ( log2CUSize – 1) ] is set equal to 1.

–	If PartMode is equal to PART_nRx2N and FilterInternalEdgesFlag is equal to 1, for k = 0.. ( 1 << log2CUSize ) – 1

- verEdgeFlags[ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ][ k ] is set equal to 1,

- verAMPEdgeFlags[ k ][ (1 << ( log2CUSize – 1) ] is set equal to 2.

…”

“8.6.1.4 Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength,

–	arrays horAMPEdgeFlags and verAMPEdgeFlags.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - If log2CUSize is not equal to 4, set the variables horAMPEdgeOffset and verAMPEdgeOffset equal to 0

    - Otherwise, if log2CUSize is equal to 4 and

      - verAMPEdgeFlags[ xDk ][ yDm ] is equal to 1, set the variable horAMPEdgeOffset equal to -4,

      - verAMPEdgeFlags[ xDk ][ yDm ] is equal to 2, set the variable horAMPEdgeOffset equal to 4,

      - horAMPEdgeFlags[ xDk ][ yDm ] is equal to 1, set the variable verAMPEdgeOffset equal to -4,

      - horAMPEdgeFlags[ xDk ][ yDm ] is equal to 2, set the variable verAMPEdgeOffset equal to 4.

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk + horAMPEdgeOffset ][ yDm + i ] ) for i = 0..7 	(8-428)

    - bStrength[1][k][m] is set equal to bSVer.

    - Decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk + horAMPEdgeOffset, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm + verAMPEdgeOffset] ) for i = 0..7 	(8-428)

    - bStrength[0][k][m] is set equal to bSHor.

    - Decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm + verAMPEdgeOffset), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - If log2CUSize is not equal to 4, set the variables horAMPEdgeOffset and verAMPEdgeOffset equal to 0

    - Otherwise, if log2CUSize is equal to 4 and

      - verAMPEdgeFlags[ xDk ][ yDm ] is equal to 1, set the variable horAMPEdgeOffset equal to -4,

      - verAMPEdgeFlags[ xDk ][ yDm ] is equal to 2, set the variable horAMPEdgeOffset equal to 4.

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk + horAMPEdgeOffset, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If log2CUSize is not equal to 4, set the variables horAMPEdgeOffset and verAMPEdgeOffset equal to 0

    - Otherwise, if log2CUSize is equal to 4 and

      - horAMPEdgeFlags[ xDk ][ yDm ] is equal to 1, set the variable verAMPEdgeOffset equal to -4,

      - horAMPEdgeFlags[ xDk ][ yDm ] is equal to 2, set the variable verAMPEdgeOffset equal to 4.

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm + verAMPEdgeOffset), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (3)x(nD), dSL are set as follows. dSL[m][0], dSL[m][1], and dSL[m][2], are set equal to dS[5], dS[6] and dS[7].

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc.

# Patent rights declaration

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).