| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G383 |
| --- | --- |

| Title: | CE12 Subtest1: Deblocking of New Non-Square Blocks: Edge Shift for AMP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | geertv@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal addresses the adaptation of the deblocking filter in case of AMP partitions of size 16x4 or 4x16. Instead of deblocking the central edge on the 8x8 deblocking grid inside the 16x16 CU of the AMP type, the relevant internal AMP partition edge is deblocked, which keeps the number of filtering operations unchanged. The deblocking filter width is adapted to avoid filtering dependencies between nearby edges. The BD-rates and execution times are very similar to the HM4 anchor.

# Problem Statement

AMP (asymmetric motion partitioning) [1] splits CUs into non-square partitions such as 64x16, 64x48, 16x64, 48x64, 32x8, 32x24, 8x32, 24x32, 16x4, 16x12, 4x16, 12x16. The present HM4 deblocking filter, processes blocking artifacts on the 8x8 grid and, therefore, does not deblock artifact edges located at positions that are not multiples of 8 pixel locations. In addition, the HM deblocking filter supports parallel implementations of deblocking decisions and independent filtering for vertical or horizontal edges. The problem is that the non-square partitions of AMP that have a dimension smaller than the minimum size for deblocking of 8x8, such as the 16x4 and 4x16 PUs, are not appropriately processed by the HM4 deblocking filter, while block edge artifacts may be visible due to quantization, and differences in motion vectors and reference pictures of adjacent partitions. In addition, the present HM4 deblocking decisions and filters have supports (number of pixels used for computations and/or number of pixels to be modified) that maximally employ 4 pixels on each side of the edge. In case of a non-square 16x4 or 4x16 partition, the deblocking filters potentially overlap and dependencies are introduced that break the HM4 deblocking filter parallelism.

# Proposed Deblocking Filter Adaptation

As an example of deblocking filter adaptation, Figure 1 illustrates AMP cases for left/right neighboring blocks P and Q (similar cases for top/bottom P and Q). If one block (P or Q) is a 16x16 CU of the AMP type, while the other block is any other intra/inter type, including 16x16 CU of the AMP type, the deblocking filter of the common edge between blocks P and Q requires adaptation when there is at least one adjacent 16x4 or 4x16 AMP partition that is parallel with the edge between blocks P and Q (Figure 1c,f,g,h,i). The arrows in Figure 1 indicate the edges, including the internal AMP partition edge, for which the deblocking filter requires adaptation in order to avoid filtering dependencies between nearby edges.

Figure 1 Examples of neighboring blocks P and Q (left/right; similar for top/bottom) with at least one of them a 16x16 CU of the AMP type (except case (a)) with one 16x4 or 4x16 partition and corresponding 16x12 or 12x16 partition. The arrows indicate the edges for which the deblocking filter is adapted in order to avoid filtering dependencies between nearby edges.

The boundary strength and deblocking on/off decision from HM4 are applied to the edge. The on/off decision is computed based on 3 samples on both sides of the edge for locations 2 and 5 along the edge of length 8 samples. In case the current edge requires deblocking, the following deblocking filter is applied, which is based on the weak luma filter from HM4:

=  ( 9*(q0 - p0) - 3*(q1 - p1) + 8)/16

= Clip( -tc, tc,  )      (tc is HM4 deblocking threshold)

If abs() < 10*tc, then:

p0’ = p0 +

q0’ = q0 -

This filter modifies one sample on both sides of the current edge. The filter is computed based on two samples on both sides of the edge (). This way dependency between the filtering of nearby edges is avoided and the parallel filtering property is preserved.

Observe that the internal AMP edge is deblocked, while the central edge on the 8x8 deblocking grid is not deblocked. This way, the number of filtering operations is similar to the anchor, while the relevant AMP edge is processed.

# Objective Results

The BD-rate results are obtained following the common test conditions for HM4. The encoding times are measured in a heterogeneous computing environment, while the decoding times are measured on a single CPU.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | -0.3% |
| Class E | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.2% | -0.5% | 0.0% | -0.1% | -0.2% |
| Class E | -0.1% | 0.4% | -0.1% | 0.0% | -0.3% | 0.0% |
| Overall | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Source Code

The software implementation utilizes the boundary strength array to indicate what type of adaptation the deblocking filter needs to perform in case of AMP 16x4 partitions. This is an implementation choice and it is not the intention to introduce boundary strength changes.

# Conclusion

This contribution proposed to adapt the deblocking filter in case of 16x4 AMP partitions. The AMP edge that is inside the 16x16 CU is not deblocking filtered in HM4. It is proposed to shift the deblocking from the central edge to the actual 16x4 partition edge without increasing the number of filtering operations. In addition, it is proposed to use the one sample weak filter in order to avoid introducing dependencies between nearby edges. The BD-rate and decoding times are similar to the HM4 anchor.

# References

[1]  I.-K Kim, W.-J Han, J. H. Park (Samsung), X. Zheng (HiSilicon), “CE2: Test result of asymmetric motion partition (AMP),” 6th JCT-VC Meeting, Torino, Italy, July 2011, Doc. JCTVC-F379

# Working Draft

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).