### Residual coding syntax

| numSigCoeff = 0 |  |
| --- | --- |
| sumAbs = 0 |  |
| cParam = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) { |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| if(coeff_abs_level_remaining[ n ] + baseLevel > 3*(1<< cParam) |  |
| cParam = Min(cParam+1,4) |  |
| } |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |

coeff_abs_level_remaining[ n ] is the remaining absolute value of a transform coefficient level that is coded with unary code at the scanning position n. The value of coeff_abs_level_remaining is constrained by the limits in subclause 8.6.3.When coeff_abs_level_remaining [ n ] is not present, it is inferred as 0.

#### Binarization process for coeff_abs_level_remaining

-

-

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the -unary binarization process as specified in subclause 9.2.2.1 - with input synElVal derived as follows:.

–	cLength is set equal to cParam and cVal is set equal to coeff_abs_level_remaining[ n ].

–	If coeff_abs_level_remaining[ n ] is less than (8<<cParam), synElVal is set equal to

(coeff_abs_level_remaining[ n ]>> cParam)

–	Otherwise, the value of synElVal is specified by a process equivalent to the following pseudo-code:

cVal = cVal - (8<<cParam )

while (cVal >= (1<<cLength))

{

cVal =  cVal - (1<<cLength)

cLength = cLength+1

}

synElVal = 8+ cLength – cParam

The suffix bin string is derived by invoking the FL binarization process as specified in subclause 9.2.2.5 with cMax equal to (1<<cLength)-1 and input equal to cVal % (1<<cLength). The value of cLength and cVal is specified by the process equivalent to the above pseudo-code.