| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0111 |
| --- | --- |

| Title: | AHG9: On DPB size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com +1-858-658-5804 aramasub@qualcomm.com +1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current draft of the HEVC standard, MaxDpbSize is limited to 6 for all levels. This document presents an analysis of the DPB size for commonly used hierarchical structures with GOP sizes of 8 and 16. It is asserted that when one reference picture is used for each reference picture list, depth-first coding order can be supported for GOP sizes of 8 and 16 with the existing maximum DPB size limit of 6. Furthermore, it is asserted that the current maximum DPB size limit of 6 is inadequate to support breadth-first coding order for GOP size of 16.

# Introduction

In the 9th JCT-VC meeting in Geneva, there was a discussion on whether MaxDpbSize of 6 is adequate for GOP size of 16. This document analyzes the DPB buffer size requirements for commonly used hierarchical structures with GOP sizes of 8 and 16.

# Discussion

We categorize the GOP structures broadly into two groups, namely, depth-first and breadth-first. In the breadth-first structure, the pictures at the same hierarchical level are consecutive in the decoding order. For a GOP size of 8, the decoding order for the breadth-first structure would be 0, 8, 4, 2, 6, 1, 3, 5, 7, for example, assuming that the numbers are POC values. This structure has the desirable property that lower temporal layers can be extracted by truncation. The depth-first structure is more demanding in terms of DPB size requirements, because when the first non-reference picture (1) is being decoded, all the reference pictures (0, 8, 4, 2, 6) must be in the DPB. For the depth-first structure the decoding order for the same example is 0, 8, 4, 2, 1, 3, 6, 5, 7.

In the analysis that follows, for each picture, the DPB size is shown after the entire picture data has been decoded. We present results for the simplest case of one reference picture per reference picture list. Pictures at each hierarchical level share the same temporal_id. When there is a choice of reference pictures, the nearest one that has the same or lower temporal_id is chosen.

Table 1: DPB buffer status for GOP size 8, depth-first, 1 reference picture per list, the minimum output delay is sps_num_reorder_pics[ sps_max_temporal_layers_minus1 ] = Log2( GOP size ) = 3, and the required DPB size is 5

| Decoding order | RPS | DPB | Output |
| --- | --- | --- | --- |
| 0 | - | 0 |  |
| 8 |  | 0, 8 |  |
| 4 | 0, 8 | 0, 4, 8 |  |
| 2 | 0, 4, 8 | 0, 2, 4, 8 | 0 |
| 1 | 0, 2, 4, 8 | 0, 1, 2, 4, 8 | 1 |
| 3 | 2, 4, 8 | 1, 2, 3, 4, 8 | 2 |
| 6 | 4, 8 | 2, 3, 4, 6, 8 | 3 |
| 5 | 4, 6, 8 | 3, 4, 5, 6, 8 | 4 |
| 7 | 6, 8 | 4, 5, 6, 7, 8 | 5 |
| 16 | 8 | 5, 6, 7, 8, 16 | 6 |
| 12 | 8, 16 | 6, 7, 8, 12, 16 | 7 |
| 10 | 8, 12, 16 | 7, 8, 10, 12, 16 | 8 |
| 9 | 8, 10, 12, 16 | 8, 9, 10, 12, 16 | 9 |
| … | … | … | … |

Table 2: DPB buffer status for GOP size 8, breadth-first, 1 reference picture per list, the minimum output delay is sps_num_reorder_pics[ sps_max_temporal_layers_minus1 ] = ( GOP size ) / 2 = 4, and the required DPB size is 6

| Decoding order | RPS | DPB | Output |
| --- | --- | --- | --- |
| 0 | - | 0 |  |
| 8 | 0 | 0, 8 |  |
| 4 | 0, 8 | 0, 4, 8 |  |
| 2 | 0, 4, 8 | 0, 2, 4, 8 |  |
| 6 | 0, 2, 4, 8 | 0, 2, 4, 6, 8 | 0 |
| 1 | 0, 2, 4, 6, 8 | 0, 1, 2, 4, 6, 8 | 1 |
| 3 | 2, 4, 6, 8 | 2, 3, 4, 6, 8 | 2 |
| 5 | 4, 6, 8 | 3, 4, 5, 6, 8 | 3 |
| 7 | 6, 8 | 4, 5, 6, 7, 8 | 4 |
| 16 | 8 | 5, 6, 7, 8, 16 | 5 |
| 12 | 8, 16 | 6, 7, 8, 12, 16 | 6 |
| 10 | 8, 12, 16 | 7, 8, 10, 12, 16 | 7 |
| 14 | 8, 10, 12, 16 | 8, 10, 12, 14, 16 | 8 |
| 9 | 8, 10, 12, 14, 16 | 8, 9, 10, 12, 14, 16 | 9 |
| 11 | 10, 12, 14, 16 | 10, 11, 12, 14, 16 | 10 |
| … | … | … | … |

Table 3: DPB buffer status for GOP size 16, depth-first, 1 reference picture per list, the minimum output delay is sps_num_reorder_pics[ sps_max_temporal_layers_minus1 ] = Log2( GOP size ) = 4, and the required DPB size is 6

| Decoding order | RPS | DPB | Output |
| --- | --- | --- | --- |
| 0 | - | 0 |  |
| 16 | 0 | 0, 16 |  |
| 8 | 0, 16 | 0, 8, 16 |  |
| 4 | 0, 8, 16 | 0, 4, 8, 16 |  |
| 2 | 0, 4, 8, 16 | 0, 2, 4, 8, 16 | 0 |
| 1 | 0, 2, 4, 8, 16 | 0, 1, 2, 4, 8, 16 | 1 |
| 3 | 2, 4, 8, 16 | 2, 3, 4, 8, 16 | 2 |
| 6 | 4, 8, 16 | 3, 4, 6, 8, 16 | 3 |
| 5 | 4, 6, 8, 16 | 4, 5, 6, 8, 16 | 4 |
| 7 | 6, 8, 16 | 5, 6, 7, 8, 16 | 5 |
| 12 | 8, 16 | 6, 7, 8, 12, 16 | 6 |
| 10 | 8, 12, 16 | 7, 8, 10, 12, 16 | 7 |
| 9 | 8, 10, 12, 16 | 8, 9, 10, 12, 16 | 8 |
| 11 | 10, 12, 16 | 9, 10, 11, 12, 16 | 9 |
| 14 | 12, 16 | 10, 11, 12, 14, 16 | 10 |
| 13 | 12, 14, 16 | 11, 12, 13, 14, 16 | 11 |
| 15 | 14, 16 | 12, 13, 14, 15, 16 | 12 |
| 32 | 16 | 13, 14, 15, 16, 32 | 13 |
| 24 | 16, 32 | 14, 15, 16, 24, 32 | 14 |
| 20 | 16, 24, 32 | 15, 16, 20, 24, 32 | 15 |
| 18 | 16, 18, 24, 32 | 16, 18, 20, 24, 32 | 16 |
| 17 | 16, 18, 24, 32 | 16, 17, 18, 20, 24, 32 | 17 |
| 19 | 18, 24, 32 | 18, 19, 20, 24, 32 | 18 |
| … | … | … | … |

Table 4: DPB buffer status for GOP size 16, breadth-first, 1 reference picture per list, with the minimum output delay sps_num_reorder_pics[ sps_max_temporal_layers_minus1 ] = Log2( GOP size ) / 2 = 8, and the required DPB size is 10

| Decoding order | RPS | DPB | Output |
| --- | --- | --- | --- |
| 0 | - | 0 |  |
| 16 | 0 | 0, 16 |  |
| 8 | 0, 16 | 0, 8, 16 |  |
| 4 | 0, 8, 16 | 0, 4, 8, 16 |  |
| 12 | 0, 4, 8, 16 | 0, 4, 8, 12, 16 |  |
| 2 | 0, 4, 8, 12, 16 | 0, 2, 4, 8, 12, 16 |  |
| 6 | 0, 2, 4, 8, 12, 16 | 0, 2, 4, 6, 8, 12, 16 |  |
| 10 | 0, 2, 4, 6, 8, 12, 16 | 0, 2, 4, 6, 8, 10, 12, 16 |  |
| 14 | 0, 2, 4, 6, 8, 10, 12, 16 | 0, 2, 4, 6, 8, 10, 12, 14, 16 | 0 |
| 1 | 0, 2, 4, 6, 8, 10, 12, 16 | 0, 1, 2, 4, 6, 8, 10, 12, 14, 16 | 1 |
| 3 | 2, 4, 6, 8, 10, 12, 16 | 2, 3, 4, 6, 8, 10, 12, 14, 16 | 2 |
| 5 | 4, 6, 8, 10, 12, 16 | 3, 4, 5, 6, 8, 10, 12, 14, 16 | 3 |
| 7 | 6, 8, 10, 12, 16 | 4, 5, 6, 7, 8, 10, 12, 14, 16 | 4 |
| 9 | 8, 10, 12, 16 | 5, 6, 7, 8, 9, 10, 12, 14, 16 | 5 |
| … | … | … |  |

# Conclusions

From Tables 1-4, the following conclusions can be drawn:

- For GOP size of 8, depth-first coding order can be supported with DPB size of 5.

- For GOP size of 8, breadth-first coding order can be supported with DPB size of 6.

- For GOP size of 16, depth-first coding order can be supported with DPB size of 6.

- For GOP size of 16, breadth-first coding order can be supported with DPB size of 10.

When one reference picture is used for each reference picture list, depth first coding order can be supported for GOP sizes of 8 and 16 with the existing maximum DPB size limit of 6. It is recommended that a note be added that for GOP size of 16, the current maximum DPB size limit of 6 can not support breadth-first coding order.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.