# Chroma QP adjustment spec, based upon N1005_v3

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... | ... |
| pps_extension1_flag | u(1) |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) | ue(v) |
| chroma_qp_adjustment[ i ] | se(v) |
| } |  |
| pps_extension2_flag | u(1) |
| } |  |
| ... |  |
| } |  |

#### Picture parameter set RBSP semantics

chroma_qp_adjustment_enabled_flag equal to 1 specifies that the cu_chroma_qp_adjustment_flag may be present in the transform unit syntax. cu_chroma_qp_adjustment_enabled_flag equal to 0 specifies that the cu_chroma_qp_adjustment_flag is not present in the transform unit syntax.

When ChromaArrayType is equal to 0, it is a requirement of bitstream conformance that the value of cu_chroma_qp_adjustment_enabled_flag shall be equal to 0.

diff_cu_chroma_qp_adjustment_depth specifies the difference between the luma coding tree block size and the minimum luma coding block size of coding units that convey cu_chroma_qp_adjustment_flag.  The value of diff_cu_chroma_qp_adjustment_depth shall be in the range of 0 to log2_diff_max_min_luma_coding_block_size, inclusive.

XXX: When not present, the value of diff_cu_chroma_qp_adjustment_depth is inferred to be equal to 0.

The variable Log2MinCuChromaQpAdjustmentSize is derived as follows:

Log2MinCuQpChromaAdjustmentSize = CtbLog2SizeY − diff_cu_chroma_qp_adjustment_depth	(7-31)

chroma_qp_adjustment_table_size_minus1 plus 1 specifies the size of the chroma_qp_adjustment[ ] table.  The value of chroma_qp_adjustment_table_size_minus1 shall be in the range of 0 to 4, inclusive.

chroma_qp_adjustment[ i ] specifies an offset used in the derivation of Qp′Cb and Qp′Cr. The values of chroma_qp_adjustment[ ] shall be in the range of 0 to +12, inclusive. When ChromaArrayType is equal to 0, chroma_qp_adjustment[ ] is not used in the decoding process and decoders shall ignore its value.

### Slice segment header syntax

#### General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !dependent_slice_segment_flag ) { |  |
| ... |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| } |  |
| ... |  |
| } |  |
| ... |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### General slice segment header semantics

slice_chroma_qp_adjustment_enabled_flag equal to 1 specifies that the cu_chroma_qp_adjustment_flag may be present in the transform unit syntax. cu_chroma_qp_adjustment_enabled_flag equal to 0 specifies that the cu_chroma_qp_adjustment_flag is not present in the transform unit syntax.

When not present, the value of slice_chroma_qp_adjustment_enabled_flag shall be inferred to be equal to 0.

#### Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| ... | ... |
| if( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| } |  |
| if( slice_chroma_qp_adjustment_enabled_flag && log2CbSize >= Log2MinCuChromaQpAdjustmentSize ) { |  |
| IsCuChromaQpAdjustmentCoded = 0 |  |
| CuChromaQpAdjustment = 0 |  |
| } |  |
| ... |  |
| } |  |

#### Transform unit syntax

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSizeC = log2TrafoSize − ( ChromaArrayType = = 3 ? 0 : 1 ) |  |
| cbfLuma = cbf_luma[ x0 ][ y0 ][ trafoDepth ] |  |
| cbfChroma = cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ ( ChromaArrayType = = 2 && ( cbf_cb[ x0 ][ y0 + ( 1 << log2TrafoSizeC ) ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 + ( 1 << log2TrafoSizeC ) ][ trafoDepth ] ) ) |  |
| if( cbfLuma ¦ ¦ cbfChroma ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| if( slice_chroma_qp_adjustment_enabled_flag && cbfChroma && !IsCuChromaQpAdustmentCoded ) { |  |
| cu_chroma_qp_adustment_flag | ae(v) |
| if( cu_chroma_qp_adjustment_flag && chroma_qp_adjustment_table_size > 1 ) |  |
| cu_chroma_qp_adjustment_idc | ae(v) |
| } |  |
| ... |  |
| } |  |
| } |  |

#### Transform unit semantics

cu_chroma_qp_adjustment_flag equal to 1 specifies that a chroma qp adjustment shall be applied to the chroma qp offset during the scaling process of chroma transform coefficients. cu_chroma_qp_adjustment_flag equal to 0 specifies that no adjustment is applied to the chroma qp offset.

When cu_chroma_qp_adjustment_flag is present, the variable IsCuChromaQpAdustmentCoded is set equal to 1

–	The variable qPa is derived as follows:

qPa = cu_chroma_qp_adjustment_flag ? chroma_qp_adjustment[ cu_chroma_qp_adjustment_idc ] : 0	(8-263)

cu_chroma_qp_adjustment_idc specifies the index to chroma_qp_adjustment[ ] that is used in the derivation of Qp′Cb and Qp′Cr. When present, the value of cu_chroma_qp_adjustment_idc shall be in the range of 0 to chroma_qp_adjustment_table_size − 1 inclusive. When not present, the value of cu_chroma_qp_adjustment_idc shall be inferred to be equal to 0.

## Scaling, transformation and array construction process prior to deblocking filter process

### Derivation process for quantization parameters

...

When ChromaArrayType is not equal to 0, the following applies.

–	The variables qPiCb and qPiCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset + qPa )	(8-263)

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset + qPa )	(8-264)

–	If ChromaArrayType is equal to 1, the variables qPCb and qPCr are set equal to the value of QpC as specified in Table 8-10 based on the index qPi equal to qPiCb and qPiCr, respectively.

–	Otherwise, the variables qPCb and qPCr are set equal to Min( qPi, 51 ), based on the index qPi equal to qPiCb and qPiCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC		(8-265)

Qp′Cr = qPCr + QpBdOffsetC		(8-266)

#### Initialization process for context variables

Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| transform_unit( ) | cu_qp_delta_abs | Table 9-24 | 0..1 | 2..3 | 4..5 |
|  | cu_chroma_qp_adjustment_flag | XXX | 0 | 1 | 2 |
|  | cu_chroma_qp_adjustment_idc | XXX | 0 | 1 | 2 |

Table 9-5 – Values of initValue for ctxIdx of cu_chroma_qp_adjustment_flag and cu_chroma_qp_adjustment_idc

| Initialization variable | ctxIdx of cu_chroma_qp_adjustment_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

### Binarization process

#### General

| Table 9-34 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| residual_coding( ) | ... | ... | ... |
|  | cu_chroma_qp_adjustment_flag | FL | cMax = 1 |
|  | cu_chroma_qp_adjustment_idc | TR | cMax = chroma_qp_adjustment_table_size_minus1, cRiceParam = 0 |

#### Derivation process for ctxTable, ctxIdx and bypassFlag

##### General

| Table 9-39 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_chroma_qp_adjustment_flag | 0 | na | na | na | na | na |
| cu_chroma_qp_adjustment_idc | 0 | na | na | na | na | na |