| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0041 |
| --- | --- |

| Title: | Adaptive reference sample smoothing simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy Huawei Research Center, 1/7, Altufievskoe shosse, Moscow, Russia | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution presents a simplification of the RDO procedure used by the adaptive reference sample smoothing (ARSS) filter, i.e. a non-normative modification of ARSS is considered. The results of different tests are presented and analyzed. The proposed solution provides a better ratio of the coding gain to encoder-side computational complexity.

# Introduction

This contribution addresses the question of the ARSS computational complexity at the encoder side and is aimed at simplifying its RDO-cost estimation, i.e. the proposed modification of ARSS is non-normative. As noted in [1], ARSS requires a joint optimization of the prediction generation decision together with the residual signal information. The reason for this is that instead of entropy coding, hiding an ARSS flag value within quantized transform coefficients is used for signaling the reference sample smoothing filter selection decision at the TU level. Since reference sample smoothing impacts the intra prediction mode selection procedure, the decision on what of the candidate intra prediction modes will be selected depends on the availability of the residues for hiding.

In this case, limiting the number of residual quad-tree (RQT) levels defined by HHI_RQT_INTRA_SPEEDUP can be resulted in such a situation, when, on a higher RQT level, the number of available quantized transform coefficients is enough to hide a ARSS flag in a TU but, on a lower level, hiding cannot be performed (see Figure 1). Hence, the estimation based on a RQT with a limited number of decomposition levels can be significantly more inaccurate than for coding tools that do not use the joint optimization of the prediction generation decision and the residual signal information. In other words, ARSS requires more detailed information on TU residues (to check whether the smoothing flag can be hidden or not) like ATMVP needs more accurate motion field [2].

Figure 1. Hiding procedure in ARSS and RQT decomposition

# Technical description

In this contribution, several mechanisms are used to simplify the ARSS computational complexity at the encoder side:

- If CBF equals 0 for default reference sample smoothing (i.e. ARSS is disabled), the RD-cost estimation is skipped for enabled ARSS as its decision cannot be signaled anyway.

- If ARSS is enabled and CBF equals 0 for a given RQT level, the RD-cost is not estimated for the lower RQT levels.

All the results given further are presented within a short length simulation using first 30 frames of each CTC test sequence. Current JEM1.0 ARSS “tool-ON” test results are given in Table 1.

Simulation results for the first simplification mechanism are presented in Table 2. For both simplification mechanisms the following simulation results has been reportedly observed (see Table 3).

Table 1. Performance of Adaptive reference samples smoothing filter. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.97% | -1.09% | -1.01% | 164% | 95% |
| Class B | -0.92% | -1.17% | -1.18% | 159% | 96% |
| Class C | -0.91% | -1.22% | -1.10% | 161% | 102% |
| Class D | -0.75% | -1.12% | -0.91% | 166% | 100% |
| Class E | -1.22% | -2.05% | -1.99% | 155% | 100% |
| Class F | -0.73% | -0.44% | -0.19% | 156% | 98% |
| Overall (Ref) | -0.90% | -1.14% | -1.03% | 160% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.32% | -0.83% | -0.82% | 106% | 106% |
| Class B | -0.40% | -0.86% | -0.97% | 105% | 106% |
| Class C | -0.49% | -0.77% | -1.03% | 105% | 100% |
| Class D | -0.21% | -0.02% | -2.26% | 106% | 101% |
| Class E |  |  |  |  |  |
| Class F | -0.52% | 0.37% | -0.73% | 104% | 100% |
| Overall (Ref) | -0.39% | -0.44% | -1.15% | 105% | 103% |

Table 2. Simplification of Adaptive reference samples smoothing filter by applying the first mechanism: if CBF equals 0 for default reference sample smoothing, the RD-cost estimation is skipped for enabled ARSS. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.96% | -1.08% | -0.99% | 158% | 89% |
| Class B | -0.92% | -1.16% | -1.20% | 154% | 93% |
| Class C | -0.94% | -1.15% | -1.18% | 158% | 100% |
| Class D | -0.75% | -1.11% | -0.90% | 160% | 101% |
| Class E | -1.22% | -1.93% | -2.02% | 147% | 99% |
| Class F | -0.73% | -0.34% | -0.29% | 151% | 97% |
| Overall (Ref) | -0.91% | -1.10% | -1.06% | 155% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.32% | -1.04% | -1.16% | 109% | 96% |
| Class B | -0.48% | -0.41% | -0.96% | 102% | 100% |
| Class C | -0.50% | -0.16% | -0.83% | 105% | 100% |
| Class D | -0.13% | -0.25% | -2.06% | 108% | 113% |
| Class E |  |  |  |  |  |
| Class F | -0.73% | -0.48% | -0.33% | 103% | 98% |
| Overall (Ref) | -0.43% | -0.47% | -1.06% | 105% | 101% |

Table 3. Simplification of Adaptive reference samples smoothing filter by applying both mechanisms. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.84% | -1.18% | -1.17% | 153% | 98% |
| Class B | -0.75% | -1.23% | -1.37% | 140% | 99% |
| Class C | -0.72% | -1.14% | -1.24% | 149% | 100% |
| Class D | -0.62% | -1.11% | -0.91% | 150% | 100% |
| Class E | -0.81% | -1.70% | -1.66% | 129% | 99% |
| Class F | -0.51% | -0.86% | -0.47% | 136% | 97% |
| Overall (Ref) | -0.71% | -1.18% | -1.13% | 143% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.25% | -1.14% | -1.17% | 106% | 102% |
| Class B | -0.41% | -1.17% | -1.02% | 104% | 101% |
| Class C | -0.53% | -0.40% | -0.67% | 105% | 101% |
| Class D | -0.14% | 0.43% | -1.51% | 103% | 100% |
| Class E |  |  |  |  |  |
| Class F | -0.41% | 0.50% | -0.03% | 103% | 101% |
| Overall (Ref) | -0.35% | -0.40% | -0.89% | 104% | 101% |

The same simplifications of ARSS filter were applied to JEM1.0. Resulting performance is presented in Table 4. Reference is JEM1.0, Reference-2 is HM16.

Table 4. Simplification of Adaptive reference samples smoothing filter by applying both mechanisms. Anchor is JEM1.0, all JEM1.0 tools are enabled

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.06% | 0.09% | -0.10% | 96% | 95% |
| Class B | 0.09% | -0.17% | -0.05% | 93% | 96% |
| Class C | 0.21% | -0.15% | -0.05% | 96% | 100% |
| Class D | 0.06% | 0.42% | 0.03% | 96% | 99% |
| Class E | 0.21% | -0.22% | -0.32% | 92% | 100% |
| Class F | -0.49% | -0.32% | -0.32% | 94% | 99% |
| Overall (Ref) | 0.02% | -0.06% | -0.12% | 95% | 98% |
| Overall (Ref-2) | -12.12% | -21.30% | -22.01% | 2012% | 139% |

To estimate the simplification effect the following procedure is performed for AI results of tool-on tests:

- Calculate average BD-rate BD=(6×Y+U+V)/8

- Complexity of the encoder C. Decoder complexity is not considered, because it is close to 100% for all the tests.

- Calculate a gain/complexity ratio R of coding gain BD to complexity increase (C-100).

This procedure estimates the amount of BD-rate reduction per one percent of complexity increase. Estimation results are presented in table 5

Table 5. Estimation of simplification effect of Adaptive reference samples smoothing filter using “tool-ON” tests

|  | Non-modified ARSS | First simplification mechanism | Simplified ARSS |
| --- | --- | --- | --- |
| Average BD-rate | -0.95% | -0.95% | -0.82% |
| Complexity | 160% | 155% | 143% |
| Gain/complexity ratio | -0.016 | -0.017 | -0.019 |

# Conclusions

Based on the simulation results, we propose to include the presented modifications to the JEM1 software.

# References

[1] G.J Sullivan, and J. Boyce. Report of Question 6/16 "Visual coding," TD 215/WP3, Geneva, Switzerland, 12-23 October 2015.

[2] J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, and X. Zhao. "Coding tools investigation for next generation video coding," COM 16 – C 806 – E, January 2015.

# Patent rights declaration

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).