| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0027 |
| --- | --- |

| Title: | Simplification and improvements for BIO design in JEM2. |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | E. Alshina, A. Alshin | Email: | elena_a.alshina@samsung.com |
| Source: |  | Samsung Electronics, Ltd. |  |  |

____________________________

Abstract

This contribution suggests 3 modifications of bi-directional optical flow (BIO) design in JEM. First modification is just twice increment for maximum allowed magnitude for motion vector refinement in BIO.  The second change is just lose-less implementation change which eliminates redundant calculations per PU and so saves encoding and decoding run time. The third modification is BIO extension to support up to 1/16 MV accuracy in JEM.

Introduction

Bi-directional optical flow is modification for bi-prediction which allows refinement of motion on sample level w/o any signaling. It was adapted to the HEVC-KTA at Geneva VCEG meeting [1], and later was modified by [2] for better adjustment to other tools. Eventually BIO has become the part of JEM [3].

This contribution suggests 3 modifications of bi-directional optical flow (BIO) design in JEM2.0. All modifications are just few lines of code change targeting to better adjustment of BIO to other JEM2.0 tools and complexity reduction.

Proposed modification

Longer MV regiment in RA configuration. First proposed modification relates to the magnitude of MV refinement in BIO. MV refinement is calculated for each sample in bi-predicted PUs additionally to the HEVC block-based motion compensation. MV regiment magnitude should not be very high. BIO equation might result in very high magnitude of MV refinement in the case liner system is ill-posed. In this case MV regiment is unreliable. So, in BIO the magnitude of MV regiment is clipped to the certain limit:

vx=Clip3(-thBIO,thBIO,-s3/s1); vy=Clip3(-thBIO,thBIO,-(s3- vxs2/2)/s5).

Here (vx,vy) is MV regiment, parameters s1,…,s6 are specified in [3].

Since recently common test condition in JEM were modified and GOP size was increased from 8 to 16 [16] we found that the doubling of thBIO improves BIO performance (especially for high resolution videos, classes A1 and A2). No modification of thBIO is needed for LBD case. No signalling is needed for thBIO, it is derived base on reference list information. Essentially there is only one line modification compared to JEM2.0 code, there is no additional calculation, BIO is applied on a same way in the same block as before, just clipping value is different. So, this change doesn’t lead to encoder of decoder run-time change.

BIO application in LD cases. Originally BIO was applied only in block has both forward and backward predictions, In [2] it was proposed to also apply BIO to a block which has two different reference pictures that are both temporally before or after the current picture and the block’s two motion vectors are non-zero and in proportion to the temporal distance between the current picture and reference pictures. This change enables BIO in LDB test configuration, but doesn’t affect RA test results. In JEM2.0 for each PU there is a condition check (pcCU->isBIOLDB( uiPartAddr) which invokes some calculations (for POC and block MV proportion) but results in false always in RA configuration. We propose to modify this code and check BIO enabling condition once per slice not per PU. This modification is lose-less, but leads to the significant reduction for both encoder and decoder run-time in RA test.

BIO extension to 1/16 MV resolution. In JEM2.0 MVD signalling is the same as in HEVC, but in MV prediction might result in up to 1/16 pel precision MV. This modification was first suggested in [5] and extended up to 1/16 pel MV precision in [6]. In BIO gradients of both predictions (from L0 and L1) are still calculated for MV precision ¼, using filters specified in Table 4 of [3]. Since BIO filters are not specified for fractional positions higher than ¼ MV is rounded to ¼ accuracy for each OU in BIO.  In order to illuminate this MV rounding operation per PU and better synchronize BIO with high precision MV in JEM2.0 we propose extension of BIO filters set as specified in Tables 1-2. Filters which exist in JEM2.0 description are not modified, additional filters for 1/16 MV precision support are marked with bold fonts. The generation procedure, coefficients precision, number of taps for BIO interpolation filters in JEM2.0 and additional ones is the same. Application condition and all calculations for BIO are unchanged, so again this modification has no implication or encoder or decoder run-time.

Table 1: Interpolation filters for gradients calculation in BIO (bold font – proposed for 1/16 MV support, regular font – existing in JEM2.0).

| Fractional pel position | Interpolation filter for gradient |
| --- | --- |
| 0 | { 8, -39, -3, 46, -17, 5 }, |
| 1/16 | { 8, -32, -13, 50, -18, 5,} |
| 1/8 | { 7, -27, -20, 54, -19, 5,} |
| 3/16 | { 6, -21, -29, 57, -18, 5,} |
| 1/4 | { 4, -17, -36, 60, -15, 4 }, |
| 5/16 | { 3, -9, -44, 61, -15, 4, |
| 3/8 | { 1, -4, -48, 61, -13, 3,} |
| 7/16 | { 0, 1, -54, 60, -9, 2,} |
| 1/2 | { -1, 4, -57, 57, -4, 1 }, |

Table 2: Interpolation filters for signal in BIO (bold font – proposed for 1/16 MV support, regular font – existing in JEM2.0).

| Fractional pel position | Interpolation filter for gradient |
| --- | --- |
| 0 | { 0, 0, 64, 0, 0, 0,}, |
| 1/16 | { 1, -3, 64, 4, -2, 0,} |
| 1/8 | { 1, -6, 62, 9, -3, 1,}, |
| 3/16 | { 2, -8, 60, 14, -5, 1,} |
| 1/4 | { 2, -9, 57, 19, -7, 2,} |
| 5/16 | { 3, -10, 53, 24, -8, 2,} |
| 3/8 | { 3, -11, 50, 29, -9, 2,} |
| 7/16 | { 3, -11, 44, 35, -10, 3,} |
| 1/2 | { 3, -10, 35, 44, -11, 3,} |

Test results

Proposed modification were integrated on top of JEM2.0 s/w and tested under JVET common test conditions [4]. For individual modification test results are as follows.

Longer MV regiment in RA configuration results in 0.1% average BD-rate improvement in RA test. LDB test performance is not affected by this change.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,2% | 0,1% | 0,2% |  |  |
| Class A2 | -0,2% | 0,1% | 0,1% |  |  |
| Class B | -0,1% | 0,1% | 0,2% |  |  |
| Class C | 0,0% | 0,0% | 0,0% | 93% | 103% |
| Class D | 0,0% | 0,1% | -0,1% | 85% | 95% |
| Overall JVET | -0,1% | 0,1% | 0,1% |  |  |
| Class F (optional) | 0,0% | 0,2% | 0,2% |  |  |

BIO application in LD cases modification is lose-less, but proposed implementation saves run-time in RA configuration as follows:

|  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| Class A1 |  | 0,0% | 0,0% | 0,0% |  |  |
| Class A2 |  | 0,0% | 0,0% | 0,0% |  |  |
| Class B |  | 0,0% | 0,0% | 0,0% |  |  |
| Class C |  | 0,0% | 0,0% | 0,0% | 96% | 100% |
| Class D |  | 0,0% | 0,0% | 0,0% | 87% | 95% |
| Overall JVET |  | 0,0% | 0,0% | 0,0% |  |  |
| Class F (optional) |  | 0,0% | 0,0% | 0,0% |  |  |
|  |  | Low delay B Main10 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class B |  | 0,0% | 0,0% | 0,0% |  |  |
| Class C |  | 0,0% | 0,0% | 0,0% | 103% | 101% |
| Class D |  | 0,0% | 0,0% | 0,0% | 103% | 102% |
| Class E |  | 0,0% | 0,0% | 0,0% | 102% | 101% |
| Overall JVET |  | 0,0% | 0,0% | 0,0% |  |  |
| Class F (optional) |  | 0,0% | 0,0% | 0,0% |  |  |

BIO extension to 1/16 MV resolution is helpful for low resolution classes mainly (as well as 1/16 MV precision itself).

|  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| Class A1 |  |  | 0,0% | 0,1% | 0,1% |  |  |
| Class A2 |  |  | -0,1% | -0,1% | 0,0% |  |  |
| Class B |  |  | 0,0% | 0,0% | 0,1% |  |  |
| Class C |  |  | 0,0% | 0,0% | -0,1% | 94% | 103% |
| Class D |  |  | -0,2% | -0,1% | -0,1% | 86% | 97% |
| Overall JVET |  |  | -0,1% | 0,0% | 0,0% |  |  |
| Class F (optional) |  |  | 0,0% | -0,1% | -0,1% | 94% | 101% |
|  |  |  | Low delay B Main10 |  |  |  |  |
|  |  |  | Y | U | V | EncT | DecT |
| Class B |  |  | 0,0% | -0,2% | 0,1% |  |  |
| Class C |  |  | 0,0% | -0,1% | -0,3% | 103% | 104% |
| Class D |  |  | 0,0% | -0,1% | -0,2% | 101% | 102% |
| Class E |  |  | -0,2% | 0,3% | -0,5% | 101% | 100% |
| Overall JVET |  |  | 0,0% | 0,0% | -0,2% |  |  |
| Class F (optional) |  |  | -0,1% | 0,4% | -0,1% | 101% | 100% |

An accurate run-time measurement for all tests will be up-date after simulations with sequential encoding will finish.

The performance of both three modifications enabled together is reported below.

|  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  | -0,1% | 0,1% | 0,0% | 94% | 105% |
| Class D |  |  | -0,1% | -0,1% | -0,1% | 87% | 97% |
| Overall JVET |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |  |  |
|  |  |  | Y | U | V | EncT | DecT |
| Class B |  |  | 0,0% | -0,2% | 0,1% |  |  |
| Class C |  |  | 0,0% | -0,1% | -0,3% | 103% | 104% |
| Class D |  |  | 0,0% | -0,1% | -0,2% | 101% | 102% |
| Class E |  |  | -0,2% | 0,3% | -0,5% | 101% | 100% |
| Overall JVET |  |  | 0,0% | 0,0% | -0,2% |  |  |
| Class F (optional) |  |  | -0,1% | 0,4% | -0,1% | 101% | 100% |

This table will be up-date after simulation will be complete.

|  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class D |  |  | -0,1% | -0,1% | -0,2% | 87% | 97% |
| Overall JVET |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |  |  |
|  |  |  | Y | U | V | EncT | DecT |
| Class B |  |  | 0,0% | -0,2% | 0,1% |  |  |
| Class C |  |  | 0,0% | -0,1% | -0,3% | 103% | 104% |
| Class D |  |  | 0,0% | -0,1% | -0,2% | 101% | 102% |
| Class E |  |  | -0,2% | 0,3% | -0,5% | 101% | 100% |
| Overall JVET |  |  | 0,0% | 0,0% | -0,2% |  |  |
| Class F (optional) |  |  | -0,1% | 0,4% | -0,1% | 101% | 100% |

Conclusion

Three modifications for BIO are proposed in this contribution. All three are targeting to better adjustment of BIO to other JEM2.0 tools, such as high precision MV (modification 3) and new test conditions GOP size 16 (modification 1), as well as lose-less complexity reduction (modification 2). Both the individual performance effect for each proposed modification is reported, as well as the net effect of both three (~0,1% average performance gain). Code modifications are minor and can be considered as fixes. Authors suggest the adoption of both three modifications to the next version of JEM.

References

- E.Alshina, A.Alshin, J.-H. Min, K. Choi, A. Saxena, M. Budagavi, “Known tools performance investigation for next generation video coding,” VCEG-AZ05, Warsaw, June

- X. Li, J. Chen, W.-J. Chien, M. Karczewicz, “Harmonization and improvement for BIO”, ITU-T SG16 Doc. COM16–C1045, Oct. 2015.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm description of Joint Exploration Test Model 2”, JVET-B1001, San-Diego, Feb. 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding”, ITU-T SG16 Doc. COM16–C806, Feb. 2015

- W. -J. Chien, J. Chen, S. Lee, M. Karczewicz (Qualcomm) Modification of merge candidate derivationJVET-B0058, 2nd Meeting, Feb. 2016.

Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).