| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0039 |
| --- | --- |

| Title: | Decoupled Luma/Chroma Transform Trees for Intra |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F.Urban, T.Poirier, F.Le Léannec 975 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 37 46 {fabrice.urban, tangi.poirier fabrice.leleannec }@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In this contribution, a Chroma Transform tree decoupled from the luma Transform Tree is proposed for Intra Coding Units. A Chroma split transform flag is added, which indicates if current chroma block is further split into 4 transform units. In addition, the chroma transform tree cannot be split in case luma Transform tree is not split. It is reported that the proposed method provides 0.21% BD-Rate saving in luma and between 1% and 2 % gain in chroma, with encoding and decoding time identical to those of the JEM2.0.

# Introduction

In JEM-2.0 [1], the Transform tree is shared between both Luma and Chroma components. In this contribution, it is proposed to separate the Transform tree for Luma and Chroma. It is shown this brings some BD rate gain, while keeping the coding/decoding times unchanged compared to the JEM2.0.

# Technology description

For intra mode, a Chroma Transform tree decoupled from the luma transform tree is proposed. A Chroma split transform flag is added to indicate whether or not the Chroma Transform Tree is split. The Chroma Transform tree can’t be more split than the Luma Transform tree. In other words, the chroma transform tree ‘follows’ the luma transform tree, but can stop splitting TUs before the luma transform tree.

This proposition is built on top of a new RDO process where CABAC contexts are updated after each Transform Unit coding estimation in Chroma.

To make the implementation more practical, in the Transform tree syntax, Luma and Chroma are no more interlaced, Luma is now coded before Chroma.

The coding of cbf_chroma flag is harmonized with cbf_luma for Intra CUs. It is coded only for the leaves of the transform tree.

In addition, a bug fix for chroma RDCost estimation is proposed. Indeed, the coding of emt_cu_flag was estimated also for chroma even if it is coded only for luma.

# Simulation results

The proposal was implemented on top of JEM-2.0 and simulations were performed following the common test conditions [2]. The anchor and the tested method were run on the same platform, to make coding/decoding times comparison consistent.

Table 1: performances of the proposed tool in AI and RA configurations

# Conclusions

This contribution proposed to add an independent Chroma Transform tree in JEM-2.0. Simulation results show some BD rate gains, without any runtime increase on the encoder or on the decoder side. 0.21% gain is obtained in Luma in AI configuration, and 1 to 2% gain is obtained in chroma. AI configuration is still running at the time the contribution is uploaded, and will be provided in a revised version of the contribution.

# References

- JEM-2.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 3rd Meeting

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).