| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0039 |
| --- | --- |

| Title: | Decoupled Luma/Chroma Transform Trees for Intra |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F.Urban, T.Poirier, F.Le Léannec 975 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 37 46 {fabrice.urban, tangi.poirier fabrice.leleannec }@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In this contribution, a Chroma Transform tree decoupled from the luma Transform Tree is proposed for Intra Coding Units. A Chroma split transform flag is added, which indicates if current chroma block is further split into 4 transform units. In addition, the chroma transform tree cannot be split in case luma Transform tree is not split. It is reported that the proposed method provides 0.14% BD-Rate saving in luma and around 1.5% gain in chroma in AI and RA, with encoding and decoding time identical to those of the JEM2.0.

# Introduction

In JEM-2.0 [1], the Transform tree is shared between both Luma and Chroma components. In this contribution, it is proposed to separate the Transform tree for Luma and Chroma. It is shown this brings some BD rate gain, while keeping the coding/decoding times unchanged compared to the JEM2.0.

# Technology description

For intra mode, a Chroma Transform tree decoupled from the luma transform tree is proposed. A Chroma split transform flag is added to indicate whether or not the Chroma Transform Tree is split. The Chroma Transform tree can’t be more split than the Luma Transform tree. In other words, the chroma transform tree ‘follows’ the luma transform tree, but can stop splitting TUs before the luma transform tree.

First, a bug fix for chroma RDCost estimation is proposed. Indeed, the coding of emt_cu_flag was estimated also for chroma even if it is coded only for luma. Table 1 shows the impact of this bug fix onto the overall performances of the JEM-2.0 in AI configuration. The detailed AI results of this modification can also be found in attached file “JVET-C0039-bugfix_EMT_CU_Flag_r1.xlsm”.

Table 1: JEM2 performance change linked to the emt_cu_flag bug fix in the rate distortion optimization process

This proposition is built on top of a new RDO process where CABAC contexts are updated after each Transform Unit coding estimation in Chroma. The impact of this modified RDO process onto the JEM2 performances is illustrated by Table 2. This RDO process is only used for Intra blocks. The RDO for inter CUs is unchanged. The detailed AI and RA results of this modification can also be found in attached file “JVET-C0039-bugfix_EMT_CU_Flag+new_RDO_r1.xlsm”.

To make the implementation more practical, in the Transform tree syntax, Luma and Chroma are no more interlaced, Luma is now coded before Chroma, for Intra blocks. The inter block coding process is unchanged.

Table 2: JEM2 performance change linked to new RDO process where CABAC contexts are updated after each Transform Unit coding estimation in Chroma

# Simulation results

The proposal was implemented on top of JEM-2.0 and simulations were performed following the common test conditions [2]. The anchor and the tested method were run on the same platform, to make coding/decoding times comparison as consistent as possible.

As shown by Table 3, in AI, 0.14% BD-Rate saving in luma and around 1.5% gain in chroma is obtained, with encoding and decoding time identical to those of the JEM2.0. In RA, around 1.5% gain is obtained in chroma as well. The proposed intra tool has slight gain in low-delay configurations.

The detailed AI and RA results of this modification can also be found in attached file “JVET-C0039-Chroma-TU_r1.xlsm”.

Table 3: performances of the proposed tool in all configurations

# Conclusions

This contribution proposed to add an independent Chroma Transform tree in JEM-2.0. Simulation results show some BD rate gains, without any runtime increase on the encoder or on the decoder side. 0.14% gain is obtained in Luma in AI configuration, and around 1.5% gain is obtained in chroma, in AI and RA.

# References

- JEM-2.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 3rd Meeting

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).