| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0041 |
| --- | --- |

| Title: | Proposed test sequences for 1080p class |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Andrey Norkin | Email: | anorkin@netflix.com |
| Source: |  | Netflix Inc. |  |  |

____________________________

# Abstract

This contribution answers to the Call for Test Material issued at the last meeting and proposes test sequences from the Netflix Chimera and El Fuente test sets, downsampled to 1080p resolution as candidates to class B (1080p sequences).

# 1. Introduction

A call for test material has been issued at the last JVET meeting [3]. In particular, JVET is planning to replace part of the 1080p material that was used for the HEVC standard development. At the last meeting, three subsequences from this test material were considered as potential candidates for inclusion to class A by results of discussion in the BoG on selection of test material [2]. In particular, sequences "FoodMarket2", "CrossWalk", "Square and time laps" were selected by the BoG as the next priority sequences in case if using more than the 8 sequences in Class A is found desirable by JVET. In addition to these three sequences, downsampled versions of several other 4K sequences are suggested as candidates for Class B (1080p) sequences.

# 2. Technical characteristics and command lines to obtain the sequences

Technical characteristics of the sequences from El Fuente and Chimera sequences sets (hosted at [4] and [5]) are as follows:

4096x2160p, 10-bit, YCbCr 4:2:0, BT.709, 59.94 fps.

The sequences can be downsampled to 2084x1080p resolution, and cropping applied if necessary, removing 64 samples from left and right of the sequences in order to produce 1920x1080p resolution sequences.

A following example ffmpeg command line can be applied for conversion:

To convert a sequence to 2048x1080p resolution (scaling factor 2):

./ffmpeg -f rawvideo -pix_fmt yuv420p10le -s:v 4096x2160 -i seqIn.yuv -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd -vsync 0 -vf select='between(n\,1\,100)',scale=2048:1080,setsar=1/1 seqOut.yuv

To convert a sequence to 1920x1080p resolution with padding top and bottom:

./ffmpeg -f rawvideo -pix_fmt yuv420p10le -s:v 4096x2160 -i seqIn.yuv -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd -vsync 0 -vf select='between(n\,1\,10)',pad=iw+0:ih+144:0:72,scale=1920:1080,setsar=1/1 seqOut.yuv

To convert a sequence to 1920x1080p resolution with cropping left and right:

./ffmpeg -f rawvideo -pix_fmt yuv420p10le -s:v 4096x2160 -i seqIn.yuv -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd -vsync 0 -vf select='between(n\,1\,100)',crop=iw-256:ih+0:128:0,scale=1920:1080,setsar=1/1 seqOut.yuv

In the examples above, the command select='between(n\,1\,100)', selects frames from 1 to 100. It can be omitted to convert the whole sequence.

Alternatively, the conversion to lower sequences can be done by proponents from the RGB 4:4:4 for El Fuente and YCbCr 4:2:2 in case of Chimera sequences to minimize the distortion.

# 3. List of suggested test sequences

| Sequence | Description |
| --- | --- |
| Food Market 2 | Food market. A loot of fine details, movement. Recommended by BoG as additional candidate sequence [3]. |
| Square and Time Lapse | First part of the sequences shows people on the square, second part shows the time lapse. Recommended by BoG as additional candidate sequence [3]. |
| CrossWalk | People crossing the street. Also a close-up of the face in the first part. Recommended by BoG as additional candidate sequence [3]. |
| Aerial | A scene captured by a camera mounted on a drone containing trees, soils and people with drone controller. A scene when the drone is flying is recommended (e.g. from frame 450) |
| Driving PoV | A scene captured by a camera on a car that shows driver’s point of view of the road Zooming motion, camera moving forward |
| Bas Scene (part 1) | A woman playing shuffleboard in a room with a dim light. Camera focus shift, static camera, sensor noise |
| Bar Scene (part 2) | People talking in a Bar with the window in the background. A typical movie scene |
| Winds and Nature | Many windmills moving as the wind blows, then switches to the grass with a cross-fade Irregular motion of windmills, static camera, according to some observers, there still seems to be room for further improvement (recommend) |
| Pier Side | Scene of pier and the seaside (water). Diagonally moving camera, still seems to be room for further improvement (e.g. coding of the sea). |

# 4. Copyright information

The proposed sequences will be given the same license as their 4K versions, namely the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International  License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/. The license also allows hosting the sequences at another site, provided that the license terms (http://creativecommons.org/licenses/by-nc-nd/4.0/) are fulfilled.

# 5. Conclusions

It is proposed to consider the sequences listed above as candidates for the 1080p class material in JVET test conditions.

# 6. References

[1] A. Norkin, I. Katsavounidis, A. Aaron, J. De Cock, "Netflix test sequences for next generation video coding", MPEG document m36685, Warsaw, Poland, July 2015.

[2] T. Suzuki, J. Chen, "Report of BoG on selection of test material", JVET-B0076, February 2016, San Diego, CA.

[3] A. Norkin, H. Yang, J.-R. Ohm, G. J. Sullivan, T. Suzuki, "Call for test material for future video coding standardization", JVET-B1002, San Diego, February 2016.

[4] https://media.xiph.org/video/derf/

[5] ftp://ftp.ient.rwth-aachen.de in the directory /testsequences/candidates/netflix