| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0032 |
| --- | --- |

| Title: | GoPro virtual reality test sequences evaluation report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seungsu Jeon Namuk Kim Dinh Quoc Khanh Byeungwoo Jeon Sungkyunkwan Univ, Cheoncheon-dong, Jangan-gu, Suwon-si, Gyeonggi-do, Korea | Tel: Email: | +82-31-290-7186 jhsl0812@skku.edu namukss@skku.edu dqkhanh@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University |  |  |

_____________________________

- Abstract

This contribution presents encoding results of 10sec GoPro Virtual Reality (VR) equirectangular format test sequences reported in JVET-C0021 to evaluate the contents and their coded data objectively and compare with three class A sequences reported in JVET-B0076. This contribution uses the HM-16.9 reference software for evaluation. PSNR is used for objective evaluation, and all evaluation results are in attached excel table.

Characteristic of original sequences

This part pre-investigates the original 10 seconds 7 GoPro Virtual Reality (VR) sequences subjectively to see some characteristics which may look complicated or hard to encode spatially and temporally. As following, the 7 sequences are grouped to 2 sets depending on camera movement:

Set 1: With camera movement

- abyss_great_shark (3840x1920_30)

- bicyclist (3840x1920_25)

- glacier (3840x1920_24)

Set 2: Without camera movement

- paramotor_training (3840x1920_25)

- skateboarding (4096x2048_60)

- timelapse_basejump (3840x1920_25)

- timelapse_building (3840x1920_25)

Set 1: With camera movement

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Abyss_great_shark (3840x1920_30) |  | ㆍThere are camera movement. ㆍThere are many fishes moving in various direction. ㆍOn the upper stretched area water is rolling. ㆍUpper stretched area is saturated due to sunshine. ㆍOn the bottom stretched area objects are twisted. |
| Bicyclist (3840x1920_25) |  | ㆍThe camera has fast unstable linear movement. ㆍThere are many complex textures (grass, tree, asphalt, cloud). ㆍOn the bottom stretched area objects are twisted. ㆍThe sequence has wild camera motion. |
| Glacier (3840x1920_24) |  | ㆍThe camera has slow unstable linear movement. ㆍSome objects appear alternately at left and right sides ㆍThe sequence has mild camera motion. ㆍShadows are seen on the snow. ㆍThere are complex textures (snow, stone mountain). |

Set 2: Without camera movement

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Paramotor_training (3840x1920_25) |  | ㆍThe focus is unchanged and stable. ㆍThe objects' movement is on the middle of the frame. ㆍOn the stretched area, complex textures (grass, cloud) are stretched. |
| Skateboarding (3840x1920_60) |  | ㆍThe focus is unchanged. ㆍThere are a lot of fine edges (due to scratches on the floor). ㆍThe objects' movement is on middle of the frame. ㆍSome objects appear alternately at left and right sides. |
| Timelapse_basejump (3840x1920_25) |  | ㆍThe focus is unchanged. ㆍThe objects movement is on middle of the frame. ㆍSmokes appear on the middle of frame. ㆍObjects' movement is fast (note it is timelapse sequence). ㆍThere are complex textures (grass, tree, cloud). |
| Timelapse_building (3840x1920_25) |  | ㆍThe focus is unchanged. ㆍThere are small and fast vehicle movements on the middle of frame. ㆍThere are fast movement of clouds on the upper stretched area. ㆍBrightness changes on middle of sequence. ㆍLuminance changes at the rooftop (contour change). |

Results and analysis

The tested GoPro Virtual Reality (VR) sequences are 420@8bits. Experiment is carried out using HM-16.9 software under the Common Test Condition (CTC). AI and RA main profiles are used. Quantization Parameter (QP) is from 22 to 42 with 5 steps (22, 27, 32, 37 and 42). In the R-D curve, x-axis represents ‘log10(bitrate)’. The experimental results are in the attached ‘JVET-D0032.xls’.

Set 1: With camera movement

Figure 1 Set 1 R-D Curve under RA

Figure 2 Set 1 R-D Curve under AI

In Figures 1 and 2, every sequence in set 1(with camera movement) shows typical R-D curves. PSNR increases steeply as QP goes lower. ‘glacier’ needs the smallest amount of bits under RA condition and ‘abtss_great_shark’ needs the smallest amount of bits under AI condition. ‘bicyclist’ needs the largest amount of bits under both RA and AI conditions.

Set 2: Without camera movement

Figure 3 Set 2 R-D Curve unver RA

Figure 4 Set 2 R-D Curve under AI

In Figures 3 and 4, every sequence in set 2(without camera movement) shows typical R-D curves. ‘skateboarding’ needs the largest amount of bits under RA test condition, and ‘timelapse_basejump’ needs the largest amount of bits under AI test condition. ‘paramotor_training’ needs the smallest amount of bits under both RA and AI conditions.

Total R-D Curves

Figure 5 Total R-D Curve under RA

Figure 6 Total R-D Curve under AI

In Figures 5 and 6, one can compare R-D performance of the 7 GoPro sequences. The results show that set 1 (with camera movement) sequences need more bits than set 2 (without camera movement) under RA test condition. Under AI test condition, performance does not seems to depend heavily on existence of camera movement. Generally, ‘bicyclist’ shows low PSNR and high bitrate under both test conditions.

Comparison with Class A

This contribution compares the three low performing VR sequences (‘abyss_great_shark’, ‘bicyclist’, ‘glacier’) with three class A sequences (‘CampfireParty-Class A1’, ‘TrafficFlow-Class A2’, ‘Runners-2nd higher priority’) under RA main profile.

Figure 7 Comparison R-D Curve under RA

In Figure 7, VR sequences need more bits and show low PSNR at high QP. But the gradient increases steeply by QP change. Class A sequences need less bits and show high PSNR at high QP. But the gradient increases gently by QP change.

Conclusion

The pre-investigation of GoPro sequences, Virtual Reality (VR) equirectangular sequences in this document reveals that the sequences have their own characteristics which are not well represneted in current HEVC test materials. The encoding results using HM-16.9 also showed that the VR sequences have higher compression efficiency than current class A sequences at low QP. But at high QP, its  compression efficiency still needs to be improved. This contribution recommends more works to be done to improve compression efficiency of VR equirectangular sequences.

- References

[1] JVET-C0021, “GoPro test sequences for Virtual Reality video coding”, JVET, Geneva, May. 2016.

[2] JVET-B0076, “Report of BoG on selection of test material”, JVET, San Diego, Feb. 2016.

[2] HM-16.9, https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-16.9

[3] GoPro test sequences, ftp://ftp.ient.rwth-aachen.de