| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0046 |
| --- | --- |

| Title: | High precision FRUC with additional candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, T. Poirier 75 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 34 41 {antoine.robert, fabrice.leleannec, tangi.poirier}@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a High precision FRUC tool with additional candidates. The FRUC list of candidates is enriched with AMVP motion vector predictors when dealing with AMVP mode, and with motion vectors of spatial neighbors. The best candidate refinement is then performed through the finest precision for motion vectors. Using these changes, average coding gains of 0.20% (Luma and Chroma) are reported in Random Access configuration compared to the unchanged JEM-3.0. It is reported that in Low Delay B configuration, average coding gains go through 0.51% (Luma and 0.65% Chroma), and in Low Delay P configuration, through 0.75%. The encoding time modification is reportedly negligible except in Low Delay B configuration where an increase of 10% is reported. Decoding time is reported to be reduced by 6%. It is suggested that the High precision FRUC tool is integrated into the JEM.

# Introduction

The goal of this contribution is to investigate improvements of the FRUC tool [2] for better coding and decoding efficiencies. More precisely, the full precision of motion compensation is currently not used by this tool, and AMVP candidates are not used even in AMVP mode.

In this contribution, the final FRUC refinement precision has been adjusted to fit the motion compensation precision, along with some code optimizations. Furthermore, AMVP candidates and motion vectors of spatial neighbors have been added to the FRUC list of candidates.

Next section describes the technology developed to support such enhanced FRUC tool. Corresponding BD-rate performance are also provided.

# Technology Description

This contribution is presented in three parts corresponding to (i) several optimizations of the FRUC code insertion, (ii) precision of the motion compensation fitting and (iii) candidate addition in the FRUC list.

BD-rate performance is also presented and discussed for each part on top of the previous one. All provided results have been obtained with the JEM-3.0 [1] in the QTBT test3 configuration chosen during the Geneva BoG on QTBT [3].

## FRUC code optimizations

The first optimization of the FRUC code takes place during the best candidate selection for blocks and refinement of motion vector predictor. Indeed, the overall RD cost of a candidate FRUC MV is given by:

frucRdCost=frucMvRate+frucDist

The rate component is evaluated first, then template or bilateral matching is used to compute the distortion part.

If it happens that for a candidate FRUC MV being evaluated, the rate cost is higher or equal (instead of strictly higher for initial JEM-3.0 implementation) to the best frucRdCost found so far, then the evaluation of current FRUC candidate MV is stopped without computing the distortion part, and the process loops to next candidate.

A second optimization aims at removing, at a given precision, the direction opposed to the previously selected best one in the FRUC motion vector refinement steps, i.e. it intends to avoid already tested positions of refinement.

Moreover, the loops collecting the unilateral motion vector candidates for blocks have been rewritten to be more efficient and several unnecessary calculations have been removed.

These FRUC code optimizations allow saving more than 10% decoding time without any change in the results, as shown in Table 1. The encoding runtime saving is more limited (2 to 3% saved).

Table 1: Impact of the optimized code of FRUC

## High precision FRUC

The original FRUC refinement process aims at refining motion vector at the initial precision, i.e. at 1/4-pel accuracy, recursively to the best minimal cost. Then a final single refinement step is performed at a finer precision, i.e. at 1/8-pel accuracy, if the precision of the motion compensation is finer than the initial used precision, i.e. 1/4-pel.

In this proposal, the final single refinement step has been updated to exactly fit the motion compensation precision. This step has been replaced by a loop of single refinement steps at a finer precision to the motion compensation one as presented on Figure 1. For now, the motion compensation precision has been extended to 1/16-pel accuracy [4], meaning this final step performs a refinement at 1/8-pel then at 1/16-pel accuracy.

…

Refine motion vector recursively

1/4-pel accuracy

Refine motion vector

n = n/2 pel accuracy

Set n=1/4

n ≠ MC precision?

n ≠ MC precision?

yes

no

yes

no

Figure 1: High precision refinement until the motion compensation precision

As shown in Table 2, High precision refinement provides 0.12% BD-rate gain in Luma for Low Delay P configuration. The impact is negligible in Random Access and Low Delay B configurations.

This feature also does not significantly impact the encoding and decoding runtimes.

Table 2: Impact of high precision refinement in FRUC over optimizations

## Candidate addition in FRUC lists

The original FRUC tool first collects several motion vector candidates (independently of the matching mode (template or bilateral)) to construct the FRUC list of motion vector predictors. From this list, the best candidate, which is the first one with the smallest matching cost, is extracted for refinement.

For blocks, the FRUC list is constructed in the same way whatever the coding mode is used, AMVP or merge, and contains unique motion vectors candidates from:

- merge candidates, and

- FRUC unilateral candidates.

Then for sub-blocks of a merge block, other FRUC lists are constructed based on the results of the corresponding block and contain unique motion vectors candidates from:

- the best candidate of the corresponding block (the whole CU),

- a zero motion vector,

- scaled versions of collocated motion vectors,

- scaled version of a FRUC unilateral candidate, and

- top and left neighboring motion vectors.

This contribution adds several candidates into these FRUC lists.

For entire CUs, AMVP candidates are added at the beginning of the list if AMVP mode is used, then motion vectors of five spatial neighbors (top, left, top-left, top-right and bottom-left) are also added but at the end of the list and in the same way as for those already present for sub-blocks. Then, the FRUC list is pruned so as to obtain a list of unique motion vectors (the proposed additional candidates are in italics):

- AMVP candidates if current CU is in AMVP mode,

- merge candidates,

- FRUC unilateral candidates, and

- top, left, top-left, top-right and bottom-left neighboring motion vectors.

And, for sub-blocks of a CU, the three missing spatial candidates (top-left, top-right and bottom-left) are also added at the end of the list, which becomes a list of unique motion vectors taken from the following ordered set:

- the best candidate of the corresponding entire CU,

- a zero motion vector,

- scaled versions of collocated motion vectors,

- scaled version of a FRUC unilateral candidate,

- top and left neighboring motion vectors, and

- top-left, top-right and bottom-left neighboring motion vectors.

Results are reported in Table 3. Adding candidates to the FRUC list (on top of high precision and optimizations) provides coding gains of 0.20% in Luma and around 0.20% in Chroma in Random Access configuration. It gives 0.51% gain in Luma and around 0.65% in Chroma in Low Delay B configuration. Finally, 0.75% gain in Luma and around 0.74% gain in Chroma are obtained in Low Delay P configuration.

This feature impacts slightly the encoding time (increase by 2%) in Random Access and Low Delay P configurations, more in Low Delay B configuration (increase of 10%). The decoding time is not impacted whatever the used configuration.

Table 3: Impact of adding candidates in the FRUC list over high precision and optimizations

# Conclusion

In AMVP mode of FRUC, this proposal consists in adding the AMVP MV predictor candidates at the beginning of the FRUC list. Moreover, five motion vectors from spatial neighbors are added to the end of the FRUC list of motion vector predictor candidates. It also increases the motion vector refinement precision to fit the motion compensation precision. BD-rates saving of 0.20% in Random Access, 0.51% (Luma and 0.65% Chroma) in Low Delay B and 0.75% in Low Delay P are obtained.

With regards to complexity, by cumulating high precision management, candidate MV predictor addition and some optimizations in the FRUC implementation, this contribution does not significantly impact the overall encoding time and allows saving about 6% decoding time in all configurations.

It suggested that the proposed FRUC improvement are integrated into next version of the JEM.

# References

[1] JVET-C1001_v3, “Algorithm Description of Joint Exploration Test Model 3”, Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, Jill Boyce,  Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 3rd Meeting: Geneva, CH, 26 May – 1 June 2016

[2] VCEG-AZ07, “Further improvements to HMKTA-1.0”, Jianle Chen, Wei-Jung Chien, Marta Karczewicz, Xiang Li, Hongbin Liu, Amir Said, Li Zhang, Xin Zhao, ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: Warsaw, PL, 19–26 June 2015

[3] JVET-C0101, “Report of BoG on QTBT configuration setting”, K. Choi, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 3rd Meeting: Geneva, CH, 26 May – 1 June 2016

[4] JVET-B0058, “Modification of merge candidate derivation”, Wei-Jung Chien, Jianle Chen, Sungwon Lee, Marta Karczewicz,  Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2nd Meeting: San Diego, USA, 20–26 February 2016

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).