| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0082-r2 |
| --- | --- |

| Title: | VMAF - video quality metric alternative to PSNR (AHG9 related) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Li, Andrey Norkin, Anne Aaron Netflix, Inc. 100 Winchester Circle, Los Gatos CA 95032, USA | Tel: Email: | +1 650 868 8155 zli@netflix.com |
| Source: | Netflix, Inc. |  |  |

# Abstract

This document contains the description of Video Multi-method Assessment Fusion (VMAF) – a full-reference objective video quality metric that aims to predict the human-perceived quality of impaired videos. VMAF is a machine-learning based approach where a prediction model is trained using subjective data collected from user study. For evaluation, VMAF is compared to other objective video quality metrics, including PSNR, SSIM, multi-scale SSIM and PSNR-HVS on a number of video datasets. It is reported that compared to other metrics, VMAF provides higher correlation with subjective opinion scores. The authors claim that VMAF is more suitable than PSNR for reporting performance of video codecs, such as in BD-rate calculation, and propose to conduct further evaluation on VMAF in a context of using it as an additional objective quality metric in the future video coding work.

# Introduction

In video codec development and research, two methods have been generally used to evaluate video quality: 1) visual inspection (or subjective testing), and 2) use of objective quality metrics. Visual inspection is operationally expensive. As a result, objective quality metrics have been used as an alternative or complement. Peak-signal-to-noise-ratio (PSNR) and Structural Similarity Index (SSIM) [1] are examples of metrics originally designed for images and later extended to video.

In video codec standardization work, PSNR has been extensively used. In the context of in-loop optimization, PSNR’s equivalent is sum of squared differences (SSD), which is used for its computational simplicity and easily tractable derivation. In reporting the overall performance of video codecs, PSNR is also used for Bjontegaard delta (BD) rate calculation [13].

In this contribution, we describe an alternative objective video quality metric, called Video Multi-method Assessment Fusion (VMAF). VMAF is a machine-learning based approach where a prediction model is trained using subjective data collected from a user study. The selection of training data and extracted elementary metrics specifically targets for two types of artifacts: lossy compression artifacts and scaling artifacts. Scaling artifacts refer to the distortion introduced when a video is down-sampled frame by frame to a lower spatial resolution before the lossy compression and later up-sampled back. This often happens in an application where the encoder can automatically select the encoding resolution to yield optimized perceptual quality, such as in dynamic adaptive streaming over HTTP (DASH).

For evaluation, we compare VMAF to its alternatives, including PSNR, SSIM [1], multi-scale SSIM [2] and PSNR-HVS [5]. The correlation results with subjective opinion scores are reported on a number of video datasets. Based on the results, we argue that VMAF is more suitable in reporting overall performance of video codecs, and propose the following use case for VMAF:

- In BD-rate calculation, when comparing bitrate savings across codecs.

- In BD-rate calculation, when comparing tools that may affect subjective video quality.

- As an alternative to visual inspection and subjective testing, when the latter is too expensive to conduct.

We also propose to study how VMAF performs in evaluating small differences in BD-rate, such as when evaluating coding tools with close performance.

# The VMAF Algorithm

The VMAF metric is a full-reference video quality metric, which means that when predicting the quality of a distorted video, it needs the original reference video as input. VMAF predicts subjective quality by combining multiple elementary quality metrics. The premise is that each elementary metric may have its own strengths and weaknesses with respect to the source content characteristics, type of artifacts, and degree of distortion. Using machine-learning regression, the elementary metrics are fused into a “meta-metric” in hope that the final prediction could yield a higher correlation with subjective opinion scores.

Figure 1: The VMAF training and prediction process

The training and prediction process of VMAF is demonstrated in Figure 1. The current version of VMAF metric (denoted as VMAF 0.3.2), included in the VMAF Development Kit open source software (link), uses the following elementary metrics:

- Visual Information Fidelity (VIF) [8]. VIF is an image quality metric based on the premise that quality is complementary to the measure of information fidelity loss. On a high level, VIF models the quality assessment problem as a measure of mutual information, as demonstrated in Figure 2. VIF measure both I(C; E), the mutual information between a reference natural image C and the version E after being processed by a human visual system (HVS) model, and I(C; F), the mutual information between C and a distorted image F after being processed by the HVS model. The final VIF score is the ratio between the two terms. In its original form, the VIF score is measured as a loss of fidelity combining four scales. In VMAF, a modified version of VIF is adopted where the loss of fidelity in each scale is included as an elementary metric.

- Detail Loss Metric (DLM) [9]. DLM is an image quality metric based on the premise that the visual quality can be better assessed by separating the measure of detail loss that affects the content visibility, and the redundant impairment that distracts viewer attention. Refer to Figure 3 for a demonstration of the idea. A wavelet-domain decoupling algorithm is used to separate the two types of distortions, the contrast sensitivity function, and the contrast masking effect are taken into account in measuring the distortion of each type. The original metric combines both DLM and additive impairment measure (AIM) to yield a final score. In VMAF, only the DLM is adopted as an elementary metric. Particular care is taken for special cases, such as black frames, where numerical calculations for the original formulation break down.

VIF and DLM are both image quality metrics. To account for the temporal characteristics of video, the following feature is further introduced:

- Temporal Information (TI). This is a simple measure of the temporal difference between adjacent frames. This is accomplished by calculating the average absolute pixel difference for the luminance component on the reference video.

These elementary metrics and features were chosen from amongst other candidates through iterations of testing and validation.

Figure 2: High-level view of VIF – modeling quality assessment as a communication problem [8]

Figure 3: Demonstration of separating the measure of detail loss and additive impairments [9]

To train the VMAF model, a video dataset was built, which includes multiple reference/distorted video pairs, and the differential mean opinion scores (DMOS) on the distorted videos collected from user studies. For details of the video dataset, refer to Section 3. On each reference/distorted video pair in the training dataset, elementary metric extraction was performed, and the extracted metric scores were vectorized and then passed to the SVM training module together with the corresponding DMOS score as input. The SVM training module took these inputs and output a prediction model.

The SVM training module has a number of hyper-parameters, which controls bias/variance tradeoff and was tuned using standard K-fold cross validation technique. When dividing the video dataset into K folds (with K = 5), the video pairs were grouped by the genres of their source content, to create the most challenging scenario for cross-validation.

Once the model is trained, to predict the human perceived quality of a distorted video with respect to its lossless reference, the same elementary metric extraction is performed on the pair, and the extracted metric score vector is passed to the SVM prediction module, which outputs a predicted VMAF score.

# Dataset for Training and Validation

To train the VMAF model, as well as to validate its performance, we built a video dataset that contains 34 reference clips, each 6 seconds long, from popular TV shows and movies from the Netflix catalog and combined them with a selection of publicly available clips (the public part is available under the following link: here). The source clips are 1920x1080 in resolution, and covered a wide range of high-level features (animation, indoor/outdoor, camera motion, face close-up, people, water, obvious salience, number of objects) and low-level characteristics (film grain noise, brightness, contrast, texture, motion, color variance, color richness, sharpness).

Using the source clips, we encoded H.264/AVC video streams at resolutions ranging from 384x288 to 1920x1080 and bitrates from 375 kbps to 20,000 kbps, resulting in 300 distorted videos. This sweeps a broad range of video bitrates and resolutions to be used in a practical video streaming service.

We then ran subjective tests to determine how non-expert observers would score the impairments of an encoded video with respect to the source clip. In standardized subjective testing, the methodology we used is referred to as the Double Stimulus Impairment Scale (DSIS) method [3]. The reference and distorted videos were displayed sequentially on a consumer-grade TV, with controlled ambient lighting (as specified in recommendation ITU-R BT.500-13 [3]). If the distorted video was encoded at a smaller resolution than the reference, it was up-scaled to the source resolution before it was displayed on the TV. The observer sat on a couch in a living room-like environment and was asked to rate the impairment on a scale of 1 (very annoying) to 5 (not noticeable). The scores from all observers were combined to generate a Differential Mean Opinion Score or DMOS for each distorted video and normalized in the range 0 to 100, with the score of 100 for the reference video.

# Test Results

The accuracy of VMAF is compared against a number of other quality metrics, including:

- PSNR for luminance component

- Daala SSIM [1]

- Daala FastSSIM [4] (implementation of Multiscale SSIM [2])

- Daala PSNR-HVS [5]

For SSIM, Multiscale SSIM and PSNR-HVS, the implementation from the Daala codec open source package [6] was used. Besides, we also add the elementary metrics of VMAF, including VIF scale 0 to 3, DLM and TI.

The result is reported in comparison between the predicted quality score and the “ground-truth” DMOS score. For each comparison, three types of scores are reported:

- The Spearman’s rank correlation coefficient (SRCC),

- The Pearson product-moment correlation coefficient (PCC)

- The root-mean-squared-error (RMSE)

Each score is calculated after a non-linear logistic fitting, as outlined in Annex 3.1 of ITU-R BT.500-13 [3].

We report the results on three popular public datasets: the VQEG HD (VQEGHD3 collection only) [10], the LIVE Video Database [11] and the LIVE Mobile Video Database [12]. We also report results on an MPEG-specific dataset: the SHVC verification dataset [14]. For visual intuition, the scatter plot for each of the metric is shown in Figure 5, together with the SRCC, PCC and RMSE reported on top of each plot. For VQEGHD3 and LIVE Video, only a subset of the dataset is used, since the rest of the video contains distortion irrelevant to codec evaluation (for example, a video contains distortion caused by transmission errors). For the SHVC verification dataset, only 1920x1080 videos are used (Session 1, 2 and 4), which include SDR/HDR and 8-bit/10-bit depths. It is reported that among all the datasets and quality metrics evaluated, VMAF yields the highest SRCC and PCC and the lowest RMSE, hence the highest accuracy in predicting the subjective quality.

Figure 4: Scatter plots on the VQEGHD3 dataset. Different color represents different video content. Only streaming-relevant impairments HRC04, HRC07 and HRC16-HRC21 and source content SRC01-SRC09 are reported.

Figure 5: Scatter plots on the LIVE Video Database. Different color represents different video content. Only compression-only impairments (H.264/AVC and MPEG-2 Video) are reported. Note that the true score is reported as the difference between reference’s MOS and distorted’s MOS score, which is different from VQEGHD3’s case.

Figure 6: Scatter plots on the LIVE Mobile Database. Different color represents different video content. Note that the true score is reported as the difference between reference’s MOS and distorted’s MOS score, which is different from VQEGHD3’s case.

Figure 7: Scatter plots on the SHVC Verification dataset. Different color represents different video content. Only 1920x1080 videos are reported (Session 1, 2 and 4), which include SDR/HDR and 8-bit/10-bit depths.

In the tables below we summarize the SRCC, PCC and RMSE for the four datasets.

|  | SRCC | PCC | RMSE |
| --- | --- | --- | --- |
| PSNR | 0.772 | 0.759 | 0.738 |
| Daala SSIM | 0.856 | 0.834 | 0.621 |
| Daala FastSSIM | 0.910 | 0.922 | 0.415 |
| Daala PSNR-HVS | 0.858 | 0.850 | 0.580 |
| VMAF 0.3.2 | 0.928 | 0.939 | 0.374 |
| VIF Scale0 | - 0.799 | - 0.812 | - 0.651 |
| VIF_Scale1 | - 0.908 | - 0.916 | - 0.440 |
| VIF_Scale2 | - 0.908 | - 0.916 | - 0.444 |
| VIF_Scale3 | - 0.910 | - 0.908 | - 0.466 |
| DLM | - 0.910 | - 0.916 | - 0.432 |
| TI | - 0.061 | - 0.066 | - 1.846 |

Table 1: SRCC, PCC and RMSE on the VQEGHD3 dataset. Only streaming-relevant impairments HRC04, HRC07 and HRC16-HRC21 and source content SRC01-SRC09 are reported.

|  | SRCC | PCC | RMSE |
| --- | --- | --- | --- |
| PSNR | 0.416 | 0.394 | 16.934 |
| Daala SSIM | 0.658 | 0.618 | 12.340 |
| Daala FastSSIM | 0.566 | 0.561 | 13.691 |
| Daala PSNR-HVS | 0.589 | 0.595 | 13.213 |
| VMAF 0.3.2 | 0.667 | 0.635 | 12.589 |
| VIF Scale0 | - 0.585 | - 0.591 | - 13.283 |
| VIF Scale1 | - 0.610 | - 0.564 | - 13.894 |
| VIF Scale2 | - 0.601 | - 0.541 | - 14.228 |
| VIF_Scale3 | - 0.587 | - 0.522 | - 14.527 |
| DLM | - 0.625 | - 0.566 | - 13.804 |
| TI | - 0.09 | - 0.169 | - 24.583 |

Table 2: SRCC, PCC and RMSE on the LIVE Video Database. Only compression-only impairments (H.264/AVC and MPEG-2 Video) are reported.

|  | SRCC | PCC | RMSE |
| --- | --- | --- | --- |
| PSNR | 0.632 | 0.643 | 0.850 |
| Daala SSIM | 0.664 | 0.682 | 0.831 |
| Daala FastSSIM | 0.747 | 0.745 | 0.718 |
| Daala PSNR-HVS | 0.703 | 0.726 | 0.722 |
| VMAF 0.3.2 | 0.879 | 0.907 | 0.396 |
| VIF_Scale0 | - 0.617 | - 0.600 | - 0.960 |
| VIF_Scale1 | - 0.732 | - 0.748 | - 0.719 |
| VIF_Scale2 | - 0.764 | - 0.777 | - 0.669 |
| VIF_Scale3 | - 0.790 | - 0.787 | - 0.650 |
| DLM | - 0.835 | - 0.851 | - 0.517 |
| TI | - 0.138 | - 0.076 | - 1.803 |

Table 3: SRCC, PCC and RMSE on the LIVE Mobile Database

|  | SRCC | PCC | RMSE |
| --- | --- | --- | --- |
| PSNR | 0.755 | 0.748 | 2.085 |
| Daala SSIM | 0.626 | 0.674 | 2.180 |
| Daala FastSSIM | 0.514 | 0.504 | 3.111 |
| Daala PSNR-HVS | 0.831 | 0.849 | 1.586 |
| VMAF 0.3.2 | 0.862 | 0.859 | 1.405 |
| VIF_Scale0 | - 0.561 | - 0.556 | - 2.906 |
| VIF_Scale1 | - 0.822 | - 0.843 | - 1.490 |
| VIF_Scale2 | - 0.834 | - 0.845 | - 1.459 |
| VIF_Scale3 | - 0.836 | - 0.842 | - 1.466 |
| DLM | - 0.831 | - 0.832 | - 1.504 |
| TI | - 0.071 | - 0.074 | - 5.450 |

Table 4: SRCC, PCC and RMSE on the SHVC Verification dataset. Only 1920x1080 videos are reported (Session 1, 2 and 4), which include SDR/HDR and 8-bit/10-bit depths.

# Conclusion

This contribution describes VMAF – a full-reference objective video quality metric that aims to predict the human-perceived quality of impaired videos. Using a machine-learning based approach to train a model with respect to subjective data, it is reported that VMAF provides higher accuracy in predicting human-perceived video quality than its alternatives, including PSNR. We argue that VMAF correlates better than PSNR with subjective quality, and propose the following use cases for VMAF:

- In BD-rate calculation, when comparing bitrate savings across codecs.

- In BD-rate calculation, when comparing tools that may affect subjective video quality.

- As an alternative to visual inspection and subjective testing, when the latter is too expensive to conduct.

We propose to conduct further evaluation of VMAF in the context of JVET and future video codec development work, as a tool for reporting performance in the above use cases.

# Supplementary materials

# VMAF Tech Blog: http://techblog.netflix.com/2016/06/toward-practical-perceptual-video.html

# VMAF Open-source Project: https://github.com/Netflix/vmaf

# Patent rights declaration(s)

Netflix Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Z. Wang, A. C. Bovik, H. R. Sheikh, and E. P. Simoncelli, “Image Quality Assessment: From Error Visibility to Structural Similarity,” IEEE Transactions on Image Processing, vol. 13, no. 4, pp. 600–612, Apr. 2004.

- Wang, Z.; Simoncelli, E.P.; Bovik, A.C. (2003-11-01). "Multiscale structural similarity for image quality assessment". Conference Record of the Thirty-Seventh Asilomar Conference on Signals, Systems and Computers, 2004. 2: 1398–1402 Vol.2. doi:10.1109/ACSSC.2003.1292216.

- BT.500 : Methodology for the Subjective Assessment of the Quality of Television Pictures, https://www.itu.int/rec/R-REC-BT.500

- M.-J. Chen and A. C. Bovik, “Fast Structural Similarity Index Algorithm,” Journal of Real-Time Image Processing, vol. 6, no. 4, pp. 281–287, Dec. 2011.

- N. Ponomarenko, F. Silvestri, K. Egiazarian, M. Carli, J. Astola, and V. Lukin, “On Between-coefficient Contrast Masking of DCT Basis Functions,” in Proceedings of the 3 rd International Workshop on Video Processing and Quality Metrics for Consumer Electronics (VPQM ’07), Scottsdale, Arizona, Jan. 2007.

- Daala codec. https://git.xiph.org/daala.git/

- C.Cortes and V.Vapnik, “Support-Vector Networks,” Machine Learning, vol. 20, no. 3, pp. 273–297, 1995.

- H. Sheikh and A. Bovik, “Image Information and Visual Quality,” IEEE Transactions on Image Processing, vol. 15, no. 2, pp. 430–444, Feb. 2006.

- S. Li, F. Zhang, L. Ma, and K. Ngan, “Image Quality Assessment by Separately Evaluating Detail Losses and Additive Impairments,” IEEE Transactions on Multimedia, vol. 13, no. 5, pp. 935–949, Oct. 2011.

- Video Quality Experts Group (VQEG), “Report on the Validation of Video Quality Models for High Definition Video Content,” June 2010, http://www.vqeg.org/

- K. Seshadrinathan, R. Soundararajan, A. C. Bovik and L. K. Cormack, "Study of Subjective and Objective Quality Assessment of Video", IEEE Transactions on Image Processing, vol.19, no.6, pp.1427-1441, June 2010.

- A. K. Moorthy, L. K. Choi, A. C. Bovik and G. de Veciana, "Video Quality Assessment on Mobile Devices: Subjective, Behavioral, and Objective Studies," IEEE Journal of Selected Topics in Signal Processing, vol. 6, no. 6, pp. 652-671, Oct. 2012.

- G. Bjøntegaard, Calculation of average PSNR differences between R-D curves, Technical Report VCEG-M33, ITU-T SG16/Q6, Austin, Texas, USA, 2001.

- Y. He, Y. Ye, Henry, Y.-K. Wang and V. Baroncini, SHVC verification test results, JCTVC-W0095r4, ITU-T SG 16 WP3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd Meeting, San Diego, USA, 19-26 Feb. 2016.