| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0105 |
| --- | --- |

| Title: | AHG8: An alternative arrangement for cube format (Double-cross layout) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hung-Chih Lin, Jian-Liang Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Hung-Chih Lin +886-3-5670766 ext. 24349 {hungchih.lin, jl.lin, shenkai.chang, cc.ju} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution aims to study two layout arrangments for the cube format. One of them is the T-shape layout, which is an unfolded net of a cube. The other is the proposed double-cross layout, which is composed of two hemicubes. In these two layouts, there is no discontinuous edge between any of two connected cube faces. Both two layouts have to fill blank area to form a video frame for encoding. The proposed double-cross layout reduces 33% memory footprint when compared to T-shape layout. Moreover, the proposed double-cross layout is more appropriate for the 360VR streaming.

The compression efficiency between the T-shape layout and the proposed double-cross layout is also examined by using HM16.12. The experimental results reportedly show that the BD-rate difference is insignificant in average.

Introduction

The cube format is one of the main representation formats used in many applications of the 360VR image/video. The cube format is composed of six cube faces. Mathematically, there are eleven distinct nets for a cube. Moreover, as illustrated in Fig. 1, any of two connected faces in each cube net is continuous.

Fig. 1 Cube nets

The PCT360 software [1], provided by InterDigital, supports cube format conversion with flexible frame packing arrangement. For cube format, PCT360 selects one of the eleven nets to provide a default configuration that arranges the six cube faces into a T-shape layout, as shown in Fig. 2(a). To form a rectangular video frame for encoding, this T-shape layout has blank area, in which the area is equal to that of the six faces. The additional blank area results in much more buffer size used in video encoder and video decoder to support this layout. To minimize the extra usage of memory buffer, we rearrange the T-shape layout into a double-cross layout. As illustrated in Fig. 2(b), the blank area is only one-third of that in the T-shape layout. In other words, the area of the proposed layout is about 66% of the T-shape layout. So far, this alternative layout has the following superiority.

- There is no any discontinuous edge inside the six cube faces

- The necessary blank area is minimum

| (a) | (b) |
| --- | --- |

Fig. 2 (a) T-shape layout (b) Double-cross layout

As illustrated in Fig. 3(a), a 360VR video content can be visualized by the combination of two hemispheres. To represent a hemisphere content in cube domain, it is projected to be a hemicube, as shown in Fig. 3(b). After side-by-side arrangement of the two hemicudes, the proposed double-cross layout in Fig. 2(b) is produced.

| (a) | (b) |
| --- | --- |

Fig. 3 360VR content representation. (a) Hemisphere [2] (b) Hemicube

Test Conditions

There are ten 360VR video test sequences provided to JVET by GoPro [3] and Nokia [4]. These test sequences are represented in equirectangular format. We use PCT360 software [1] to convert the test sequences from 8-bit equirectangular format to 10-bit cube format with 75% sampling rate. The detailed information is listed in Table I. Moreover, the reference software is HM16.12 and the test conditions are the Main-10 reference configurations specified in [5].

Table I. List of 360VR Test Sequence and Resolution

| Source | YUV Sequence | Resolution |  |  |
| --- | --- | --- | --- | --- |
|  |  | Equirectangular | Cube |  |
|  |  |  | T-shape layout | Proposed layout |
| GoPro | abyss_great_shark_vr | 3840x1920 | 3840x2880 | 3840x1920 |
|  | bicyclist_vr | 3840x1920 | 3840x2880 | 3840x1920 |
|  | glacier_vr | 3840x1920 | 3840x2880 | 3840x1920 |
|  | paramotor_training_vr | 3840x1920 | 3840x2880 | 3840x1920 |
|  | skateboarding_vr | 4096x2048 | 4096x3072 | 4096x2048 |
|  | timelapse_basejump_vr | 3840x1920 | 3840x2880 | 3840x1920 |
|  | timelapse_building_vr | 3840x1920 | 3840x2880 | 3840x1920 |
| Nokia | BearAttack-equirect-left | 3840x1920 | 3840x2880 | 3840x1920 |
|  | GT_Sheriff-equirect-left* | 4320x2160 | 4352x3264 | 4352x2176 |
|  | LRRH-equirect-left | 3840x1920 | 3840x2880 | 3840x1920 |
| * The resolution of these two layouts is converted to satisfy each cube face is a multiple of 64x64 LCU size. For the resolution of 4320x2160, the converted area is about 76% of the equirectangular format. |  |  |  |  |

Experimental Results

The simulation results of BD-rate comparison are listed in Table II. The PSNR are computed in the cube domain. As shown in Table II, the compression efficiency of these two layouts is rather similar. For the GoPro test set, the bit-rate increase has within 0.5%. On the contrary, more than 0.6% BD-rate reduction could be observed in the low-delay configurations for the Nokia test set.

Table II. Results of double cross layout compared to the T-shape layout

|  | All Intra Main10 |  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| GoPro | -0.12% | -0.26% | -0.32% | GoPro | 0.40% | 0.55% | 0.35% |
| Nokia | -0.07% | -0.08% | 0.01% | Nokia | -0.66% | -0.43% | -0.19% |
| Overall | -0.11% | -0.21% | -0.22% | Overall | 0.08% | 0.25% | 0.19% |
|  | Random Access Main10 |  |  |  | Low Delay P Main10 |  |  |
|  | Y | U | V |  | Y | U | V |
| GoPro | 0.22% | 0.41% | 0.43% | GoPro | 0.35% | 0.40% | 0.28% |
| Nokia | -0.31% | -0.26% | -0.12% | Nokia | -0.64% | -0.50% | 0.01% |
| Overall | 0.06% | 0.21% | 0.26% | Overall | 0.05% | 0.13% | 0.20% |
| Anchor: T-shape layout; Tested: Double-cross layout |  |  |  |  |  |  |  |

Streaming Application

While watching 360VR video contents, users cannot watch the whole 360VR content (front view and rear view) simultaneously. More specifically, the viewed region is less than 50% of the 360VR area. Hence, transmitting entire bit-stream is considerably wasted, especially for the 360VR streaming applications. Moreover, in some scenarios, most users may be only interested in a small range of view area. For example, users are usually interested in the view area that has players and/or singers on the stage. For streaming application on 360VR video content, we need to focus on the following challenges.

- Server side:

    - Transmission bandwidth: How many users can be served while broadcasting?

- User side:

    - Limited network bandwidth: The receiver side may not have sufficient network bandwidth to receive the entire video bit-stream that represents the whole 360VR content.

    - Computing power: The received bit-stream has to be fully decoded for displaying the relatively small view area. Typically, more than 50% decoded region is not displayed.

To address the above requirements, we can partition the proposed double-cross layout with constrained encoding setting such that the encoded bit-stream can be partially sent and received. The partition in the double-cross layout and the encoder constraints are conceptually described as follows.

- Divide the double-cross layout into two frame partitions (such as slice or tile): one partition is the front view and the other is the rear view. Here, we use tile partition for an example, as illustrated in Fig. 4.

- In-loop filter control is disabled when across frame partition (slice or tile) boundary [6].

- Motion compensation: As shown in Fig. 5, the reference area of front view/rear view pointed by a motion vector should not access the other frame partition region (rear view/front view).

- Periodic IDR frame allows user to switch front/rear view at an IDR frame.

Fig. 4 Tile partition for double-cross layout

Fig. 5 Available reference region for front and rear views

With the above encoder setting, the server site can only transmit the desired region (front view or rear view) selected by the user. In other words, the data bandwidth for broadcasting is thus remarkably reduced. Also, the received bit-stream can be independently decoded for display. In Fig. 6, when the user would like to switch to the other view, this action is returned to the server and then the bit-stream of the other view could be sent starting at next IDR frame.

Fig. 6 View switching

Conclusion

The memory usage and the compression efficiency between the T-shape layout and the proposed double-cross layout is analyzed in this contribution. The memory size of the proposed layout is only about 66% of the T-shape layout. The experimental data also evidence that the difference of the compression efficiency between these two layouts is negligible. In addition, this double-cross layout is more appropriate for 360VR streaming applications.

References

- Y. He, B. Vishwanath, and Y. Ye, “AHG8: InterDigital’s projection format conversion tool,” Document of Joint Video Exploration Team, JVET-D0021, October, 2016

- https://www.youtube.com/watch?v=zpuUUxMfaJs

- A. Abbas, “GoPro test sequences for Virtual Reality video coding,” Document of Joint Video Exploration Team, JVET-C0021, May, 2016

- J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, and A. Aminlou, “Nokia test sequences for virtual reality video coding,” Document of Joint Video Exploration Team, JVET-C0064, May, 2016

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations,” Document of Joint Video Exploration Team, JVET-B1010, February, 2016

- Y. Ye, Y. He, Y. He, and M. Horowitz, “ROI tile sections,” Document of Joint Collaborative Team on Video Coding, JCTVC-K0248, October, 2012

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).