| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0023 |
| --- | --- |

| Title: | AHG5: Improved fast encoding setting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshiya Yamamoto Tomohiro Ikai Yukinobu Yasugi | Email: | yamamoto.yoshiya@sharp.co.jp ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | SHARP Corporation. |  |  |

_____________________________

- Abstract

This contribution proposes a fast encoding setting, which changes the threshold for stopping further splitting. The threshold is changed from constant value (3) to adaptive value (2 or 3) which is determined by the distance between current and reference pictures. Large distance picture, which is important for succeeding pictures, uses the value 3 to allow deep splitting, while small distance picture use the value 2 for speed-up.

Simulation results show around 12 % encoding speed up with 0.23 % BD-rate loss.

Introduction

In QTBT, one of the encoding speedup methods is the early CU determination by CU mode and BT depth. If the best CU mode is skip mode and the current BT depth is greater than or equal to SKIP_DEPTH, further QT and BT partitioning is stopped as shown Table 1.

Table 1. Early CU determination in QTBT

if (rpcBestCU->getSkipFlag(0) && (bTestHorSplit ¦¦ bTestVerSplit) && uiBTDepth>=SKIP_DEPTH) { // stop remaining partitioning process bTestHorSplit = bTestVerSplit = bQTSplit = false; }

At the last meeting, Samsung proposed a fast encoding setting for JEM (JVET-D0053) [1]. In JVET-D0053, the SKIP_DEPTH value is changed from 3 to 2 for earlier CU determination. We (JVET-D0095) also reported that the termination control based on Temporal ID, i.e. the distance between pictures, shows better performance complexity balance [2]. In this proposal, we combine these two methods of early termination.

Proposed Method

The proposed method is a simple adjustment of SKIP_DEPTH settings in a manner that depends on the picture distance. A larger value of SKIP_DEPTH is used in frames that refer far frames in order to allow evaluation of a variety of block sizes, while a smaller value is used in frames that refer near frames in order to reduce encoding time.

We use picture distance instead of Temporal ID in JVET-D0095 to apply to LB configuration as well as RA configuration. It is noted in LB configuration TId is always equal to 0 (thus cannot be used). That is, the encoding time will not reduce in LB configuration if SKIP_DEPTH is set to large value in low Temporal Id.

Experimental results

The proposed method was implemented on top of JEM-4.0 reference software. Two options with different SKIP_DEPTH settings at Table2 were performed. The results against JEM4.0 CTC anchor are shown in Table 3, Table 4 and Table 5.

Table 2. SKIP_DEPTH settings

| distance between pictures | SKIP_DEPTH |  |  |
| --- | --- | --- | --- |
|  | D0053 Set1 | TestA | TestB |
| 4 > | 2 | 3 | 3 |
| 3 | 2 | 3 | 3 |
| 2 | 2 | 2 | 3 |
| 1 | 2 | 2 | 2 |

Table 3. Results on TestA

|  | Random Access Main 10 (Parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.04% | 0.11% | 91% | 100% |
| Class A2 | 0.24% | 0.04% | 0.01% | 86% | 88% |
| Class B | 0.27% | -0.13% | -0.14% | 88% | 99% |
| Class C | 0.24% | -0.01% | 0.05% | 89% | 102% |
| Class D | 0.20% | -0.13% | -0.03% | 87% | 104% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.23% | -0.04% | -0.01% | 88% | 98% |
| Class F (optional) |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall (Ref) |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Table 4. Results on TestB

|  | Random Access Main 10 (Parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | -0.09% | -0.07% | 93% | 104% |
| Class C | 0.13% | -0.03% | -0.01% | 92% | 99% |
| Class D | 0.10% | -0.10% | -0.06% | 91% | 104% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.12% | -0.07% | -0.05% | 92% | 102% |
| Class F (optional) |  |  |  |  |  |

Table 5. Results on D0053 Set1

|  | Random Access Main 10 (Parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.41% | -0.26% | -0.29% | 83% | 98% |
| Class C | 0.29% | -0.11% | 0.06% | 86% | 106% |
| Class D | 0.29% | -0.03% | 0.02% | 85% | 105% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.33% | -0.14% | -0.10% | 84% | 102% |
| Class F (optional) |  |  |  |  |  |

The following graph shows the trade-off between encoding time and coding performance of JEM4.0+JVET-D0053 and proposals in class B,C and D for RA configuration. Both test results are better than JEM4.0+JVET-D0053.

Conclusion

This contribution proposes to change the threshold for early CU determination depending on the picture distance. The distance-dependent SKIP_DEPTH shows better encoding time / performance balance than constant SKIP_DEPTH setting. And, TestA setting results show around 12% encoding time reduction with 0.23% luma coding loss in RA configuration. We recommend to include distance-dependent SKIP_DEPTH software in JEM.

References

- Kiho Choi, “AHG5: Fast encoding setting for JEM”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0053, 4th Meeting

- Y.Yamamoto, T. Ikai, “AHG5: Fast QTBT encoding configuration”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0095, 4th Meeting

Patent rights declaration(s)

SHARP Corporation. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).