| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0048 |
| --- | --- |

| Title: | EE3: Cross-check for high precision FRUC with additional candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution verifies performance results of EE3 tests 2 and 3 on high precision FRUC with additional candidates. It is confirmed that major performance improvement 0.2% in RA, 0.7% in LDB and 0,8% in LDP comes from extension MV candidates list (test #2).

# Introduction

Pattern matched motion vector derivation technique (FRUC) is one of the best performing and also consuming the most encoding and decoding of part of JEM4.0. In exploration experiment under verification FRUC was modified as follows [1]:

- Increase MV precision in FRUC

- Addition of Motion Vector Candidates in FRUC

- Removing some candidates for merge sub-blocks (on top of #2)

All three modifications were included into EE3 as tests #1, 2 and 3 correspondently. This contribution verifies tests #2 and #3.

# Test results

The code EE3 tests 1-3 is available at  https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE3-D0046/ and tested under JVET CTC for random access, low-delay and low-delay-P configurations compared to the JEM4.0 as anchor.

Summary of test results is shown in Table 1. Major performance improvement is observed in test #2 due to additional MV candidates in FRUC. AMVP candidates (if current CU is in AMVP mode) are added at the beginning of MV candidates list. Top, left, top-left, top-right and bottom-left neighboring motion vectors are added at the end of MV candidates list at both CU and Sub-PU level. This FRUC MV candidates list extension is also the source of the major run-time increment.

In test #3 de facto original list of MV candidates for FRUC is modified by new one. Namely

Zero MV and scaled interpolated MV are not used for FRUC, no more than 4 TMVP candidates are used.

Verified proposal provides reasonable gain with moderate run-time increment.

Test results perfectly match with data provided by proponent and SW implementation follows description in [1].

Table 1. Test results summary:Test2(on the left), test#3(on the right).

|  | Random Access Main 10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 | -0.3% | -0.3% | -0.4% | 110% | 107% |  | -0.3% | -0.4% | -0.4% | 110% | 112% |
| Class A2 | -0.2% | -0.3% | -0.4% | 111% | 106% |  | -0.3% | -0.2% | -0.4% | 114% | 114% |
| Class B | -0.3% | -0.4% | -0.3% | 107% | 117% |  | -0.2% | -0.3% | -0.3% | 106% | 113% |
| Class C | -0.2% | -0.4% | -0.3% | 102% | 100% |  | -0.2% | -0.4% | -0.3% | 101% | 98% |
| Class D | -0.1% | -0.4% | -0.1% | 102% | 101% |  | -0.2% | -0.3% | 0.0% | 100% | 98% |
| Overall | -0.2% | -0.3% | -0.3% | 106% | 106% |  | -0.2% | -0.3% | -0.3% | 106% | 107% |
| Class F | -0.2% | -0.2% | -0.1% | 101% | 100% |  | -0.2% | -0.2% | -0.1% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class B | -0.7% | -0.9% | -0.9% | 120% | 108% |  | -0.7% | -1.0% | -1.2% | 119% | 105% |
| Class C | -0.5% | -0.7% | -0.9% | 130% | 114% |  | -0.5% | -0.6% | -0.9% | 119% | 103% |
| Class D | -0.6% | -1.3% | -1.1% | 109% | 97% |  | -0.7% | -1.1% | -1.0% | 107% | 94% |
| Class E | -0.9% | -0.4% | -1.4% | 110% | 110% |  | -0.6% | -0.5% | -1.2% | 109% | 100% |
| Overall | -0.7% | -0.9% | -1.1% | 117% | 107% |  | -0.6% | -0.8% | -1.1% | 114% | 101% |
| Class F | -0.4% | -0.5% | -0.5% | 115% | 113% |  | -0.3% | -0.2% | -0.5% | 118% | 110% |
|  | Low delay P Main10 |  |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class B | -0.99% | -1.44% | -1.05% | 113% | 114% |  | -0.98% | -1.57% | -1.30% | 107% | 107% |
| Class C | -0.66% | -1.02% | -1.24% | 106% | 109% |  | -0.70% | -0.82% | -1.15% | 104% | 102% |
| Class D | -0.78% | -1.26% | -0.92% | 105% | 101% |  | -0.76% | -0.61% | -0.88% | 105% | 69% |
| Class E | -0.61% | -1.39% | -1.15% | 103% | 107% |  | -0.65% | -1.01% | -1.60% | 101% | 101% |
| Overal | -0.78% | -1.28% | -1.08% | 107% | 108% |  | -0.79% | -1.04% | -1.21% | 104% | 94% |
| Class F | -0.56% | -0.20% | -0.92% | 105% | 102% |  | -0.64% | -0.63% | -1.16% | 105% | 102% |

Run time is from our cluster, so it is not highly reliable.

# Conclusions

Verified proposal provides reasonable gain with moderate run-time increment. The major source of the performance improvement (0.2% in RA, 0.6% in LDB and 0,8% in LDP) is extension of FRUC MV candidates list.

# References

[1] A. Robert, F. Le Léannec, T. Poirier “High precision FRUC with additional candidates,” JVET-D0046, Oct. 2016, Chengdu

# Patent rights declaration

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).