| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0056 |
| --- | --- |

| Title: | AHG 8: An improvement on the compact OHP layout |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hung-Chih Lin, Chao-Chih Huang, Chia-Ying Li, Ya-Hsuan Lee, Jian-Liang Lin, Shen-Kai Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Hung-Chih Lin +886-3-5670766 ext. 24349 {hungchih.lin, georg.huang, erica.li, louise.lee, jl.lin, shenkai.chang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an improvement on the compact octahedron projection (OHP) layout by rotating 90-degree on the octahedron. As compared to the original compact OHP layout in the previous contribution JVET-D0142, the experimental results reportedly demonstrate that the BD-rate reduction provided by the proposed layout achieves 6.0%, 5.8%, 6.8% and 6.6% in S-PSNR-NN, WS-PSNR, S-PSNR-I, and CPP-PSNR, respectively, while the memory footprint of the line buffer is also reduced.

Introduction

In our previous contribution JVET-D0142 [1], a compact layout for the octahedron projection (OHP) was provided to maximally achieve content continuity between faces. When producing pixel samples from sphere to the equilateral triangle faces of the octahedron, the relative position of the sphere and octahedron is located as shown in Fig. 1(a). However, the video content around the equator area is split and projected to the triangle boundary. Since objects in 360VR video contents usually move around the equator area, the moving objects may be segmented into several small pieces. These pieces dispersed to different faces may result in inefficient predictive coding.

| (a) | (b) |
| --- | --- |

Fig. 1 (a) Relative location of octahedron and sphere in JVET-D0142 [1] (b) Relative location of octahedron and sphere for the proposed rotated layout.

To overcome this drawback, by rotating 90-degree on the octahedron, the area around the equator will be projected inside the eight triangle faces as shown in Fig. 1(b). The frame packing scheme for the proposed projection is shown in Fig. 2, where the dashed line represents the equator. In order to pack the eight triangle faces into a rectangular shape, face 4 and face 8 are bisected into two parts; one is placed at the top of the rectangle and the other one is placed at the bottom of rectangle. An image example for the proposed compact layout is shown in Fig 3.

For hardware design, video encoder and video decoder need the line buffer to temporarily keep the data of previous LCUs for use in intra/inter prediction and in-loop filtering of the current LCU row. The memory size of the line buffer is proportional to the width of the video frame. Hence, as compared to the previous COHP layout [1], the line buffer could also be reduced.

Fig. 2 The frame packing scheme for the proposed OHP projection.

Fig 3. An example image of the proposed layout.

Experimental Results

The experiments on the proposed 90-degree rotated compact OHP layout are conducted based on the JVET common test conditions and evaluation procedures for 360° video [2][3]. The averaged BD-rate reduction over the ten test sequences with various quality metrics for the 90-degree rotated compact OHP compared to the original compact OHP (COHP1) in JVET-D0142 [1] is shown in Table I. The simulation results of the proposed compact OHP layout compared to COHP2 [4] is also listed in Table II.

Table I. BD-rate performance (Anchor: COHP1; Test: Proposed)

| Quality Metric | Y | U | V |
| --- | --- | --- | --- |
| S-PSNR-NN | -6.0% | -6.8% | -6.9% |
| WS-PSNR | -5.8% | -5.3% | -5.8% |
| S-PSNR-I | -6.8% | -7.0% | -7.3% |
| CPP-PSNR | -6.6% | -6.2% | -6.8% |
| WS-PSNR (End to End) | -3.2% | -5.5% | -6.1% |
| PSNR_Viewport0 | -2.8% | -5.0% | -6.0% |
| PSNR_Viewport1 | -7.5% | -6.5% | -6.3% |

Table II. BD-rate performance (Anchor: COHP2; Test: Proposed)

| Quality Metric | Y | U | V |
| --- | --- | --- | --- |
| S-PSNR-NN | -5.6% | -9.4% | -9.7% |
| WS-PSNR | -5.6% | -11.7% | -11.3% |
| S-PSNR-I | -6.3% | -8.7% | -9.3% |
| CPP-PSNR | -6.5% | -9.1% | -9.6% |
| WS-PSNR (End to End) | -3.5% | -8.4% | -8.9% |
| PSNR_Viewport0 | -3.2% | -7.0% | -8.1% |
| PSNR_Viewport1 | -6.4% | -7.9% | -7.0% |

Conclusion

This contribution proposes an improvement on the compact OHP layout by rotating 90-degree on the octahedron. As compared to the COHP1 layout, the simulation results report that the BD-rate saving provided by the proposed layout accomplishes 6.0%, 5.8%, 6.8% and 6.6% in S-PSNR-NN, WS-PSNR, S-PSNR-I, and CPP-PSNR, respectively, while the memory footprint of the line buffer is also reduced. We thus recommend adopting this modification on the octahedron projection to improve the coding performance.

References

- H.-C. Lin, C.-Y. Li, J.-L. Lin, S.-K. Chang, and C.-C. Ju, “AHG8: An efficient compact layout for octahedron format,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0142, Oct. 2016

- E. Alshina, J. Boyce, A. Abbas, and Y. Ye, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D1030, Oct. 2016

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, Feb. 2016

- Y. He, B. Vishwanath, X. Xiu, and Y. Ye, “AHG8: InterDigital’s projection format conversion tool,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0021, Oct. 2016

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).