| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0061 |
| --- | --- |

| Title: | AHG8: Yaw-roll-pitch orientation for VR360 video content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hung-Chih Lin, Cheng-Hsuan Shih, Jian-Liang Lin, Shen-Kai Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Hung-Chih Lin +886-3-5670766 ext. 24349 {hungchih.lin, ch.shih, jl.lin, shenkai.chang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution evaluates the performance of yaw-roll-pitch orientation for the VR360 video contents. The simulations results reportedly show that with appropriate orienation on VR360 video content, the coding performance could be improved remarkably. Since the optimal orientation is highly content-dependent and format-dependent, a syntax design is also proposed to indicate the orientation information.

# Introduction

As shown in Fig. 1, a set of commonly used rotation axes is the yaw-roll-pitch orientation [1]. The pitch axis is usually the horizontal axis and the yaw axis usually represents the vertical axis. Hence, the remained axis is the roll axis that is perpendicular to the pitch-yaw plane. Moreover, the rotation order along these three axes is pitch-axis first, then roll-axis, and finally yaw-axis. With this orientation order, the range of rotation along each axis is not the same. In general, the rotation degree along pitch axis ranges from -90o to 90o while the rotation degree along roll and yaw axes is in the interval of [-180o, 180o).

Fig. 1 Yaw-Roll-Pitch axes

As shown in Fig. 2, the yaw-roll-pitch orientation has already been realized in the 360Lib [2] by the following steps:

- Step #1: Determine the corresponding sphere coordinate s=s0,s1,s2 for each integer coordinate x,y of the output format

- Step #2: The sphere coordinate s multiplies by rotation matrix R to obtain the oriented coordinate s': s'=Rs where R=RYawRRollRPitch and RYaw/RRoll/RPitch are the rotation matrix representing orientation along yaw/roll/pitch axis, respectively.

- Step #3: Determine the coordinate xi',yi' of the input format according to s'

- Step #4: Calculate pixel value representing the integer coordinate x,y of the output format by 2 two-dimensional interpolation filter, such as Lanczos filter

Fig. 2 Implementation of yaw-roll-pitch orientation

# Experimental Results

The simulations on the yaw-roll-pitch orientation for the CMP projection are conducted on 360Lib-1.0-rc1 [2] and HM version 16.9 [3] followed by the JVET common test conditions and evaluation procedures for 360° video [4][5]. With various quality metrics, the BD-rate reductions on Y component provided by different rotations are shown in Table I-Table IV. The experimental results on Y component based on ERP projection for Chairlift sequence with 90-degree rotation are also listed in Table V.

Table I. BD-rate performance in S-PSNR-NN (Anchor: CMP without orientation)

| Sequence | Rotation along yaw-axis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15o | 30o | 45o | 60o | 75o |
| Tain | -4.52% | -8.77% | -4.65% | -2.05% | -1.13% |
| Skateboarding_trick | -0.69% | -1.83% | -0.86% | 0.90% | 2.07% |
| Skateboarding_in_lot | 1.02% | 0.84% | -0.47% | -0.47% | 0.25% |
| Chairlift | 0.65% | -0.04% | -0.59% | -0.21% | -0.34% |
| KiteFlite | -0.24% | 0.82% | 1.11% | 1.62% | 1.36% |
| Harbor | 1.76% | 1.19% | 2.13% | 3.13% | 1.77% |
| PoleVault | 0.85% | 1.11% | 1.52% | 0.09% | -0.42% |
| AerialCity | -1.18% | -2.59% | -4.69% | -3.69% | -2.37% |
| DrivingInCity | -3.41% | -0.31% | 2.51% | 4.02% | 2.98% |
| DirvingInCountry | -2.80% | -0.29% | 3.81% | 6.19% | 5.56% |

Table II. BD-rate performance in WS-PSNR (Anchor: CMP without orientation)

| Sequence | Rotation along yaw-axis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15o | 30o | 45o | 60o | 75o |
| Tain | -4.28% | -8.47% | -4.75% | -2.26% | -1.18% |
| Skateboarding_trick | -0.75% | -2.51% | -1.50% | 0.29% | 1.01% |
| Skateboarding_in_lot | 1.09% | 0.93% | -0.32% | -0.29% | 0.35% |
| Chairlift | 0.64% | 0.01% | -0.63% | -0.28% | 0.06% |
| KiteFlite | 0.06% | 0.94% | 1.31% | 1.69% | 0.97% |
| Harbor | 1.83% | 0.97% | 1.60% | 2.77% | 1.81% |
| PoleVault | 0.58% | 1.47% | 1.41% | 0.60% | 0.22% |
| AerialCity | -0.98% | -2.01% | -3.72% | -2.54% | -1.47% |
| DrivingInCity | -3.16% | 0.28% | 2.99% | 4.58% | 3.63% |
| DirvingInCountry | -2.75% | -0.61% | 3.44% | 6.14% | 6.14% |

Table III. BD-rate performance in S-PSNR-I (Anchor: CMP without orientation)

| Sequence | Rotation along yaw-axis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15o | 30o | 45o | 60o | 75o |
| Tain | -4.50% | -8.76% | -4.75% | -2.11% | -1.15% |
| Skateboarding_trick | -0.55% | -1.61% | -0.67% | 0.94% | 2.12% |
| Skateboarding_in_lot | 0.92% | 0.76% | -0.43% | -0.23% | 0.40% |
| Chairlift | 1.01% | 0.55% | -0.19% | -0.19% | -0.42% |
| KiteFlite | -0.08% | 0.74% | 0.92% | 1.44% | 1.28% |
| Harbor | 2.12% | 1.55% | 2.30% | 2.90% | 1.37% |
| PoleVault | 1.25% | 1.79% | 2.08% | 0.35% | -0.46% |
| AerialCity | -1.31% | -3.19% | -5.67% | -4.48% | -2.64% |
| DrivingInCity | -3.51% | -0.57% | 2.17% | 3.82% | 3.02% |
| DirvingInCountry | -3.31% | -1.12% | 3.00% | 5.90% | 5.66% |

Table IV. BD-rate performance in CPP-PSNR (Anchor: CMP without orientation)

| Sequence | Rotation along yaw-axis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15o | 30o | 45o | 60o | 75o |
| Tain | -4.26% | -8.32% | -4.56% | -2.17% | -1.07% |
| Skateboarding_trick | -0.59% | -2.27% | -1.24% | 0.42% | 1.00% |
| Skateboarding_in_lot | 1.04% | 0.88% | -0.27% | -0.06% | 0.53% |
| Chairlift | 0.99% | 0.56% | -0.31% | -0.27% | -0.09% |
| KiteFlite | 0.27% | 1.29% | 1.19% | 1.51% | 0.89% |
| Harbor | 2.07% | 1.35% | 2.15% | 2.53% | 1.55% |
| PoleVault | 0.84% | 2.00% | 1.77% | 0.60% | 0.05% |
| AerialCity | -1.00% | -2.52% | -4.68% | -3.36% | -1.76% |
| DrivingInCity | -3.20% | 0.06% | 2.83% | 4.39% | 3.45% |
| DirvingInCountry | -3.24% | -1.44% | 2.65% | 5.80% | 6.21% |

Table V. BD-rate performance (Anchor: ERP without orientation)

| Sequence | 90o Rotation along roll-axis |  |  |  |
| --- | --- | --- | --- | --- |
|  | S-PSNR-NN | WS-PSNR | S-PSNR-I | CPP-PSNR |
| Chairlift | -18.1% | -17.6% | -17.3% | -16.8% |

# Proposed Syntax Signaling Scheme

In the previous section, the experimental results evidence that the appropriate yaw-roll-pitch orientation can offer a noticeable improvement on the compression efficiency. Therefore, in Table V, we propose a syntax signaling scheme to indicate the orientation information in the bit-stream.

Table V. Proposed syntax signaling scheme for yaw-roll-pitch orientation

| vr_content_orientation() { | Descriptor |
| --- | --- |
| pitch_zero_orientation | u(1) |
| if ( !pitch_zero_orientation ) |  |
| pitch_orientation_degree | se(v) |
| roll_zero_orientation | u(1) |
| if ( !roll_zero_orientation ) |  |
| roll_orientation_degree | se(v) |
| yaw_zero_orientation | u(1) |
| if ( !yaw_zero_orientation ) |  |
| yaw_orientation_degree | se(v) |
| } |  |
| pitch_zero_orientation |  |
| pitch_zero_orientation specifies whether the content is oriented along the pitch axis before encoding. |  |
| pitch_orientation_degree |  |
| pitch_orientation_degree specifies the rotation degree along the pitch axis, and the value of pitch_orientation_degree shall be an integer value in the range of [-90o, 90o], inclusive. |  |
| roll_zero_orientation |  |
| roll_zero_orientation specifies whether the content is oriented along the roll axis before encoding. |  |
| roll_orientation_degree |  |
| roll_orientation_degree specifies the rotation degree along the roll axis, and the value of roll_orientation_degree shall be an integer value in the range of [-180o, 180o), inclusive. |  |
| yaw_zero_orientation |  |
| yaw_zero_orientation specifies whether the content is oriented along the yaw axis before encoding. |  |
| yaw_orientation_degree |  |
| yaw_orientation_degree specifies the rotation degree along the yaw axis, and the value of yaw_orientation_degree shall be an integer value in the range of [-180o, 180o), inclusive. |  |

# Conclusion

In this contribution, the yaw-roll-pitch orientation on VR360 video contents is proposed to enhance the compression efficiency. The simulation results show that the the coding performance can be significantly improved with appropriate orientation.

# References

- H.-C. Lin, C.-Y. Li, C.-C. Huang, J.-L. Lin, and S.-K. Chang, “Syntax design of yaw-roll-pitch orientation for 360VR video content,” Document of Immersive Visual Content Coding Standard, IEEE1857.9_M1064/AVSVR_M1064, Dec. 2016

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- HM reference software, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/

- E. Alshina, J. Boyce, A. Abbas, and Y. Ye, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D1030, Oct. 2016

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, Feb. 2016.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).