| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0063 |
| --- | --- |

| Title: | Results cross-check of the EE3-D0042 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, T. Poirier 75 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 34 41 {antoine.robert, fabrice.leleannec, tangi.poirier}@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This report verifies the Exploratory Experiments 3, part D0042 (BIO) results. The observed results are in accordance with the proponent’s ones. The average encoding runtimes are decreased through 92% in RA and 95% in LDB, and decoding runtimes through 84% in RA and 94% in LDB, while the average BD rate is preserved with a loss of only 0.02% in RA and 0.15% in LDB.

# Introduction

This modification of BIO was initially proposed in JVET-D0042 [1] and is tested here as part of the Exploratory Experiments 3, part D0042 [2].

If BIO calculations are restricted to be performed only for the same area with predicted block (w/o block extension) then the memory bandwidth becomes the same as for regular motion compensation. Since prediction and gradients outside of the block are not needed computational complexity is reduced and so reduction of both encoder and decoder run time is observed. Memory access before and after suggested modification is depicted on figure below. Here M=2 and T is the length of LumaMC interpolation filter.

Figure 1: BIO modifications to be tested

The associated tests are described in the following table.

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4 | BIO w/o block extension | Samsung | Technicolor |

# Test #4

The proposed enhanced BIO method has been implemented on JEM4.0 [3], and results of Random Access and Low Delay B configurations are summarized in the Table 1 as required by the EE3.

BD-rate performance of these BIO improvements are presented below.

Table 1: EE3 D0042: BIO improvements inJEM4.0

# References

[1] JVET-D0042, “AHG6: On BIO memory bandwidth”, A. Alshin, E. Alshina, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

[2] JVET-D1011_r2_v2, “Description of Exploration Experiments on Coding Tools”, Elena Alshina, Jill Boyce, Li Zhang, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

[3] JVET-D1001_v3, “Algorithm Description of Joint Exploration Test Model 4”, Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, Jill Boyce, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).