| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0065 |
| --- | --- |

| Title: | AHG8: Unrestricted Motion Compensation for 360 Video in ERP Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | minhua.zhou@broadcom.com |
| Source: | Broadcom Limited |  |  |

_____________________________

# Abstract

This contribution reports results of doing unrestricted motion compensation for 360 video in equirectangular projection (ERP) format by leveraging the fact that there is no discontinuous edge between the left and right reference picture boundaries. Experimental results revealed that in RA configuration doing the unrestricted motion compensation in “wrapped-around” fashion in horizontal direction provides on average about 0.2% BD-rate reduction by using HM16.14, and 0.3% BD-rate reduction by using JEM4.1, respectively.

# Introduction

In the previous video compression standards such as MPEG/ITU-T AVC/H.264 and HEVC/H.265 the unrestricted motion compensation is realized by replacing the reference samples outside the reference picture boundaries with the nearest reference picture boundary samples. In 360 video of equirectangular projection (ERP) format, however, there is actually no discontinuous edge between the left and right reference picture boundaries, if a picture is stitched perfectly. Fig.1 provides an example of 360 video picture in ERP format.

Fig. 1 An example of 360 video picture in ERP format

Instead of replacing the reference samples outside the left and right reference picture boundaries with the nearest reference picture boundary samples as the traditional unrestricted motion compensation does, doing the unrestricted motion compensation in a “wrapped-around” fashion in horizontal direction may improve compression efficiency. As shown in Fig.2, in this option a reference block outside the left reference picture boundary is replaced with the “wrapped-around” reference block along the right reference picture boundary, and vice versa. Note that in vertical direction the traditional unrestricted motion compensation is still employed.

Fig. 2 Illustration of unrestricted motion compensation in “wrapped around” fashion

# Experimental Results

The simulation was carried out by using both HM16.14 and the JEM4.1that were integrated with the 360 video library. The JVET 360 video common test conditions specified in [1] were followed.

Table 1 RA results of using HM16.14

As shown in Table 1, using HM16.14 the unrestricted motion compensation in “wrapped-around” fashion provides on average about 0.2% BD-rate reduction in RA configuration.

Table 3 Summary results of using JEM4.1

Table 3 summarizes the experimental results by using JEM4.1. On average about 0.3% BD-rate reduction is achieved relative to JEM4.1 anchor.

Note that the run time in Table 2 and Table 3 may not be accurate due to the heterogeneous nature of LSF farm.

The detailed results for all the 360 video quality metrics defined in [1] can be found in the attached spreadsheets.

# Summary

The experimental results revealed that doing unrestricted motion compensation in “wrapped around” fashion for 360 video in ERP format provides compression advantage. It is recommended in the future video coding to add this option to unrestricted motion compensation for 360 video in ERP format.

# References

[1] E. Alshina, J. Boyce, A. Abbas, Y. Ye , “JVET common test conditions and evaluation procedures for 360° video”, JVET-B1030, February, 2016.

# Patent rights declaration(s)

Broadcom Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).