| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0089 |
| --- | --- |

| Title: | Cross-check of JVET-E0037 on Signaling for primary transform and transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange |  |  |

_____________________________

# Abstract

This contribution reports the cross check of contribution JVET-E0037 "Signaling for primary transform and transform skip". This proposal addresses the removal of a redundant syntax related to transform skip. It is reported that the correction applied in the software reflects the proposal and that the coding results obtained by the proponents are also correct.

# Introduction

In JEM4 a transform skip flag is parsed for the blocks smaller or equal than 4x4. If enabled, the information previously parsed in emt_cu_flag is discarded and not used. Consequently, there is a useless signalling of emt when skip is enabled. This is what the contribution proposes to solve.

The software applies this small modification in a non ambiguous fashion, as it was checked based on the implementation given by the proponent.

Also coding performance of this proposal was checked according to the common test conditions in both Random Access and All Intra configuration.

The results are presented below. The numbers match those of JVET-E0037. Note that the encoder and decoder runtimes are not reliable. From the software analysis the runtime changes are to be considered unchanged compared to the anchors.

The coding performance is merely not affected by this change.

Some improvement can be noticed especially for the Class F items, although the redundant signalling suppression seems to have a limited impact on coding performance (e.g. +0.13% for class A2 in RA mode).

# Conclusions

This contribution verifies the removal of a redundant signalling. The coding performance is merely not affected with this change.

# Patent rights declaration(s)

Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).