| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0099 |
| --- | --- |

| Title: | EE5 #5: Cross-check of JVET-E0062 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Qiang Yao Kei Kawamura Sei Naito | Email: | qi-yao@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research Inc.) |  |  |

_____________________________

- Abstract

This document is a cross-check report of EE5: Multi direct modes for chroma coding with Supplementary Results (JVET-E0062) on test #5, which allows a chroma block to select one of the modes from an intra prediction mode list that consists of cross-component linear model mode, multiple intra prediction modes derived from co-located luma coding blocks, and chroma prediction modes from spatial neighboring blocks. It is reported that the method shows -0.21%(Y)/-1.18% (Cb)/-1.16% (Cr), -0.03%(Y)/-1.13% (Cb)/-1.02% (Cr), and -0.00%(Y)/-0.16% (Cb)/-0.30% (Cr), BD-rate gain with 99%/100%, 100%/100%, and 100%/100% encoding/decoding time on average in All Intra, Random Access and Low delay B (LDB) configuration, respectively. The simulation results match with those provided by proponents.

Introduction

In this assigned cross check, a multiple direct modes for chroma coding is tested. It is reported that the method shows -0.21%(Y)/-1.18% (Cb)/-1.16% (Cr), -0.03%(Y)/-1.13% (Cb)/-1.02% (Cr), and -0.00%(Y)/-0.16% (Cb)/-0.30% (Cr), BD-rate gain on average in All Intra, Random Access and Low delay B (LDB)  configuration, respectively. There is no increase of encoding run time in Random Access or Low delay B (LDB) configuration, and 1% decrease of encoding run time on average is observed in All Intra configuration. In addition, there is no increase of decoding run time on average in All Intra, Random Access or Low delay B (LDB) configuration. The simulation results match with those provided by proponents.

SW implementation is clear and follows original proposal [1].

Test assignment

The cross check corresponds to #5 test in the following EE test table.

List of tests to be performed

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Multiple linear models, samples are grouped in two sets | Qualcomm | Sharp |
| 2 | Multiple-filter LM | Qualcomm | ETRI |
| 3 | average of angular and LM mode | Qualcomm | Samsung |
| 4 | Test 1+ Test 2+ Test 3 | Qualcomm | KDDI |
| 5 | Multi direct modes for chroma coding | Qualcomm | KDDI |

Simulation results

The source code of the EE5 tests 4 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE5-D0111/, and tested under JVET CTC compared to the JEM4.0 as anchor. Provided software was compiled by GCC 4.8.5 on 64-bit Linux. The experiments were performed on the cluster PCs.

The simulation results are presented in the tables below and match with those provided by proponents.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.74% | -0.77% | 100% | 100% |
| Class A2 | -0.31% | -2.10% | -1.75% | 99% | 100% |
| Class B | -0.18% | -1.05% | -1.21% | 100% | 100% |
| Class C | -0.20% | -1.03% | -0.99% | 99% | 101% |
| Class D | -0.17% | -0.82% | -0.85% | 99% | 100% |
| Class E | -0.32% | -1.45% | -1.44% | 99% | 101% |
| Overall | -0.21% | -1.18% | -1.16% | 99% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | -0.62% | 99% | 100% |
| Class A2 | -0.08% | -1.80% | -1.60% | 99% | 100% |
| Class B | -0.04% | -1.50% | -1.24% | 100% | 101% |
| Class C | -0.02% | -1.16% | -1.08% | 100% | 100% |
| Class D | 0.00% | -0.86% | -0.53% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.03% | -1.13% | -1.02% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.05% | -0.44% | 100% | 100% |
| Class C | 0.09% | -0.39% | -0.41% | 100% | 100% |
| Class D | -0.01% | 0.00% | -0.41% | 100% | 100% |
| Class E | -0.18% | -0.25% | 0.24% | 100% | 100% |
| Overall (Ref) | 0.00% | -0.16% | -0.30% | 100% | 100% |

Conclusion

The simulation results of EE5 #5 has been verified. The encoding / decoding runtime also matched closely to those reported by the proponent.

References

- L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz, “Multiple Direct Modes for chroma intra coding”, JVET-D0111, 4th Meeting: Chengdu, CN, 15–21 October 2016

- L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz, “EE5: Multiple Direct Modes for chroma intra coding”, JVET-E0062, 5th Meeting: Geneva, CH, 12–20 January 2017