| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0118 |
| --- | --- |

| Title: | Cross-check of AHG5: Improved fast encoding setting (test B) (JVET-E0023) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Victorien Lorcy Pierrick Philippe Thibaud Biatek | Email: | victorien.lorcy@b-com.com pierrick.philippe@orange.com thibaud.biatek@tdf.fr |
| Source: | b<>com |  |  |

_____________________________

# Abstract

This contribution verifies the performance of a fast encoding method proposed by Sharp. The proposed method makes a depth threshold, which prevents further partitioning, dependant on the reference frame distance. Those modifications provide up to 12% complexity reduction for a +0.23% Y BD-rate loss in Random Access (RA) configuration over JEM 4.0.

# Introduction

In JEM, when evaluating the binary tree (BT), if the best CU mode is skip mode then a threshold defines the BT depth for which the encoder will stop further partitioning. Currently, this threshold takes two different values depending on the BT split mode: 3 for a vertical split (SKIPHORNOVERQT_DEPTH_TH), 2 for a horizontal split (SKIP_DEPTH). At the last meeting, it was proposed in JVET-D0053 [1] to set the threshold value for both cases to 2 in order to reduce the encoder complexity, providing 23% complexity reduction and +0.51% Y BD-rate in RA over JEM 3.0. In [2], the proponents suggested making early split terminations dependant on the reference frame distance to obtain a better trade-off between complexity reduction and compression loss.

The evaluated contribution, JVET-E0023, proposes to have one single threshold value for the two BT split modes (SKIP_DEPTH_VALUE), as in [1], and to make it dependant on the reference frame distance. The following table describes the macro values used in each scenario.

|  | D0053 Set1 |  | E0023 TestA | E0023 TestB |
| --- | --- | --- | --- | --- |
| distance between pictures | SKIPHORNOVERQT_DEPTH_TH | SKIP_DEPTH | SKIP_DEPTH_VALUE | SKIP_DEPTH_VALUE |
| 4 > | 2 | 2 | 3 | 3 |
| 3 | 2 | 2 | 3 | 3 |
| 2 | 2 | 2 | 2 | 3 |
| 1 | 2 | 2 | 2 | 2 |

Table 1: skip depth settings

# Test results

The evaluated method was implemented on top of JEM 4.0. The tests were performed under the JVET Common Test Conditions as described in [3]. The software was compiled using GCC 4.7.2 on Debian 7.8 and the tests were executed on a linux cluster. In this contribution, only E0023 test B is tested. The coding results compared to JEM 4.0 are shown in the tables below.

|  | Random Access Main 10 (Parallel) over HM-16.6-JEM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | 0.12% | 0.00% | -0.02% | 92% | #NOMBRE! |
| Class B | 0.14% | -0.09% | -0.07% | 93% | #NOMBRE! |
| Class C | 0.13% | -0.03% | -0.01% | 92% | #NOMBRE! |
| Class D | 0.10% | -0.10% | -0.06% | 89% | #NOMBRE! |
| Class E |  |  |  |  |  |
| Overall (Ref) | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class F (optional) | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
|  | Low delay B Main10 over HM-16.6-JEM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class C | 0.31% | 0.22% | 0.07% | 87% | #NOMBRE! |
| Class D | 0.24% | -0.04% | 0.26% | 84% | #NOMBRE! |
| Class E | 0.09% | 1.07% | 0.08% | 77% | #NOMBRE! |
| Overall (Ref) | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class F (optional) | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |

# Code

Code differences between JVET-E0023 source code and HM-16.6-JEM-4.0:

source/Lib/TLibCommon/CommonDef.h |  5 +++++

source/Lib/TLibCommon/TypeDef.h   | 12 ++++++++++++

source/Lib/TLibEncoder/TEncCu.cpp | 25 +++++++++++++++++++++++++

3 files changed, 42 insertions(+)

# Conclusion

The following performances are reported: +XX% BD-rate and XX% complexity decrease in RA, +XX% BD-rate and XX% complexity decrease in LDB, both over JEM 4.0. As shown in section 3, the code modifications are minor. The results reported in JVET-E0053 are thus confirmed in this contribution.

# References

- Kiho Choi, “AHG5: Fast encoding setting for JEM”, Joint Video Exploration Team (JVET) of 	ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0053, 4th Meeting

- Y.Yamamoto, T. Ikai, “AHG5: Fast QTBT encoding configuration”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0095, 4th Meeting

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

# Patent rights declaration(s)

B<>com, Orange, TDF may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).