| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0064 |
| --- | --- |

| Title: | MS-SSIM as an additional mandatory metric to PSNR for future video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson, Rickard Sjöberg, Per Wennersten, Kenneth Andersson, Jacob Ström and Jack Enhorn Färögatan 6 164 80 Stockholm Sweden | Email: | {martin.m.pettersson, rickard.sjoberg, per.wennersten, kenneth.r.andersson, jacob.strom, jack.enhorn } @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In JVET-C0030, it was reported that MS-SSIM correlated better with subjective data compared to PSNR and SSIM. During the presentation of that document, a question was raised of how well MS-SSIM was adapted to correctly identify small changes in a video, e.g. turn a tool on/off. The current contribution attempts to answer this question. BD-rate results for MS-SSIM between recent JEM versions are provided. The contribution claims that the numbers indicate that MS-SSIM is also stable for small changes in subjective quality. The proponents argue that including MS-SSIM during future video coding development can help creating a more efficient standard. Therefore, it is proposed to include MS-SSIM as an additional mandatory metric to PSNR for the CTC and the CfE on future video coding.

# Introduction

PSNR, which has traditionally been used to objectively evaluate video quality during past standardization work, has the benefit that it is easy to calculate and provides numbers which are easy to comprehend. On the other hand, several studies over the years have shown that PSNR does not correlate very well to perceptual quality of image and video data since the metric is not designed with the human visual system (HVS) in mind. This may lead to suboptimal decisions in the standardization process. An objective metric that better matches the perceptual quality is needed. Further, desired requirements for an objective metric to be used for video quality evaluation in video codec development would comprise:

- High correlation with subjective MOS

- Perceptual responsiveness to different types of video coding artifacts

- Ability to differentiate small quality differences

- Thoroughly tested and accepted by the video community

A metric that we believe fulfills these requirements is MS-SSIM. Why MS-SSIM would be a good fit is further described in this document.

## Description of SSIM and MS-SSIM

Multi Scale Structural Similarity (MS-SSIM) index [2] is a more advanced version of the well-known Structural Similarity (SSIM) index used for comparing the similarity between two images. MS-SSIM, developed by the people behind SSIM, is based on SSIM but provides more flexibility in accounting for variations in viewing conditions.

### Single Scale SSIM

Let x and y be two discrete non-negative signals that are aligned with each other, e.g., two image patches. Further let μx, μy, σx2,σy2 and σxy be the mean of x, the mean of y, the variance of x, the variance of y and the covariance of x and y, respectively. Luminance (l(x,y)), contrast (c(x,y)) and structure (s(x,y)) comparison measures may then be calculated according to:

lx,y=2μxμy+C1μx2+μy2+C1,                                                                    (1)

cx,y=2σxσy+C2σx2+σy2+C2,                                                                    (2)

sx,y=σxy+C3σxσy+C3,                                                                          (3)

where C1, C2 and C3 are small constants given by

C1=(K1L)2, C2=(K2L)2 and C3=C22,                              (4)

L is the dynamic range of the pixel values (L = 255 for 8 bits/pixel and 1023 for 10 bits/pixel), and K1=0.01 and K2=0.03 are two scalar constants used to stabilize the function for when the denominators in (1) and (2) are close to 0. The general form of SSIM index between signal x and y is defined as:

(5)

where ,  and  define the relative importance of these three components. Normally, for single scale SSIM, ,  and are set to 1 which leads to the formula:

SSIMx,y= 2μxμy+C1(2σxy+C2)(μx2+μy2+C1)(σx2+σy2+C2)                                   (6)

### MS-SSIM

Whereas the single-scale method may be appropriate only for a specific viewing distance, a multi-scale method is a convenient way to incorporate image details at different resolutions.

The MS-SSIM system works as illustrated in figure 1. The image patches are iteratively low-pass filtered and downsampled by a factor of two. The original image has scale 1 and the highest scale is M, obtained after M-1 iterations. At the j-th scale, contrast and structural comparisons cj(x,y) and sj(x,y) are calculated.

Figure 1:MS-SSIM system. L: low-pass filtering; 2↓: downsample by 2

The overall MS-SSIM evaluation is obtained by combining the measurement at different scales using:

(7)

where αM, βj and γj define the relative importance of these three components. To simplify things, equal weighting between the three components are used, i.e. αj =  βj = γj. The cross-scale settings are also normalized such that j=1Mγj=1.  In [2], MS-SSIM is calculated using five different scales (M = 5) with the weights β1 = γ1 = 0.0448, β2 = γ2 = 0.2856, β3 = γ3 = 0.3001, β4 = γ4 = 0.2363, and α5 = β5 = γ5 = 0.1333.

## MS-SSIM correlations to subjective data

At the 3rd JVET meeting in Geneva, Huawei showed in JVET-C0030 [1] that MS-SSIM correlates significantly better with subjective image and video data than PSNR does for a number of subjective databases. The following table was presented in JVET-C0030 showing correlation numbers to eight subjective databases (five image and three video). It is replicated here for convenience to the reader. It can be seen that MS-SSIM outperforms PSNR in all tests and is significantly better than SSIM on average.

| Image Database | Measure | PSNR | SSIM | MS-SSIM |
| --- | --- | --- | --- | --- |
| LIVE JPEG/JPEG2000 [3] | SCC | 0.901 | 0.936 | 0.966 |
|  | KCC | - | - | - |
| TID2013[4-6] | SCC | 0.640 | 0.637 | 0.787 |
|  | KCC | 0.470 | 0.464 | 0.608 |
| A57[7] | SCC | 0.825 | 0.903 | - |
|  | KCC | - | - | - |
| LIVE2006[8-10] | SCC | 0.876 | - | 0.953 |
|  | KCC | - | - | - |
| VCL@FER[11] | SCC | - | 0.911 | 0.923 |
|  | KCC | - | 0.733 | 0.750 |

| Video Database | Measure | PSNR | SSIM | MS-SSIM |
| --- | --- | --- | --- | --- |
| LIVE [12-13] | SCC | 0.368 | 0.526 | 0.736 |
|  | LCC | 0.404 | 0.544 | 0.744 |
| LIVE Mobile [14-16] | SCC | 0.678 | 0.650 | 0.743 |
|  | LCC | 0.691 | 0.664 | 0.708 |
| MCL-V [17] | SCC | 0.464 | 0.648 | 0.623 |
|  | LCC | 0.472 | 0.650 | 0.621 |

Table 1: Performance Comparison of each metric. SCC: Spearman Correlation Coefficient, KCC: Kendall Correlation Coefficient, LCC: (Pearson) Linear Correlation Coefficient.

This is in line with several other studies [3][4][5][6][7] showing that MS-SSIM correlates better with subjective results than PSNR. IETF is also planning to include MS-SSIM as a mandatory metric together with PSNR for objective evaluation of NetVC[8].

# MS-SSIM comparison tests for JEM and HM

At the Geneva meeting a comment was made during the presentation of JVET-C0030 that “results might be very different depending on whether fundamentally different coding algorithms are compared, and what the quality differences are. If the comparison is just made for a tool-on/tool-off experiment, it might be questionable how much we could conclude from that.”

In this document BD-rate results for MS-SSIM between recent JEM versions (JEM-3.1, JEM-4.0 and JEM 5.0.1) is provided to show how MS-SSIM behaves for small quality differences in the JVET context. The thinking here is that the difference between one version (say JEM-4.0) and the next version (say, JEM-5.0.1) is small, and that MS-SSIM should capture this small improvement if it is working as intended. In addition, PSNR and MS-SSIM results are also provided for comparing HM-16.15 with JEM-5.0.1.

## Encoding configurations for JEM and HM

JEM-3.1, JEM-4.0 and JEM-5.0.1 were encoded using their respective encoding configuration files.

HM-16.15 was encoded using the config files aligned with JEM-5.0.1 found in HM-16.6-JEM-5.0.1\cfg\hm-16.13 in the JEM repository to enable a valid comparison with the JEM-5.0.1 CTC. More specifically, the following parameters were changed compared to the HM-16.15 configuration:

|  | Original HM-16.15 configuration | HM-configuration aligned with JEM-5.0.1 |
| --- | --- | --- |
| All Intra | DecodingRefreshType: 1 ReWriteParamSetsFlag: 1 | DecodingRefreshType : 0 TemporalSubsampleRatio: 8 |
| Random Access | ReWriteParamSetsFlag: 1 ASR: 1 MinSearchWindow: 64 |  |
| Low delay | ReWriteParamSetsFlag: 1 |  |
| Low Delay P | ReWriteParamSetsFlag: 1 |  |

Table 2: Difference in encoding settings between original HM-16.15 configuration and the configuration aligned with JEM-5.0.1 found in HM-16.6-JEM-5.0.1\cfg\hm-16.13.

## Results and Discussions

In tables 3 to 5 below, BD-rates for PSNR and MS-SSIM are given for JEM-3.1 vs JEM-4.0, JEM-4.0 vs JEM-5.0.1and HM-16.15 vs JEM-5.0.1. Note that MS-SSIM is only defined for luminance. MS-SSIM results for chroma is provided for information.

The overall BD-rates for PSNR and MS-SSIM for the luma component are also illustrated in the plots of figures 2 to 4.

Figures 2 and 3: BD-rates for PSNR and MS-SSIM for the luma component when comparing JEM 3.1 to JEM 4.0 and JEM 5.0.1 to JEM 4.0.

Figure 4: BD-rates for PSNR MS-SSIM for the luma component comparing JEM 5.0.1 to HM 16.15.

It can be noted that the improvements in BD-rates for PSNR between JEM versions are not very large, namely in the range of -0.24% to -2.84%. This is in the same order as many of the tools that have been included in JEM so far. As figures 2 to 4 illustrate the BD-rate gains are very similar for PSNR and MS-SSIM indicating that MS-SSIM has a stable behaviour for the small changes between the JEM versions.

It can also be noted that the gains are slightly smaller for MS-SSIM than for PSNR in the comparison between HM-16.15 and JEM-5.0.1.

,

Table 3: BD-rate results for PSNR and MS-SSIM for JEM-3.1 (Reference) vs JEM-4.0 (Test).

Table 4: BD-rate results for PSNR and MS-SSIM for JEM-4.0 (Reference) vs JEM-5.0.1 (Test).

Table 5: BD-rate results for PSNR and MS-SSIM for HM 16.15 (Reference) vs JEM 5.0.1 (Test).

# Conclusion

In this contribution, PSNR and MS-SSIM results have been provided for JEM 5.0.1, JEM 4.0, JEM 3.1 and HM 16.15. BD-rates for PSNR and MS-SSIM between the different JEM-versions show very similar results for the two metrics which indicates that MS-SSIM could be as stable as PSNR for small changes in compression efficiency in the JVET context.

In addition, BD-rates for PSNR and MS-SSIM results comparing JEM 5.0.1 with HM 16.15 show slightly lower gains for MS-SSIM compared to PSNR.

We believe that MS-SSIM is appropriate to use as metric in future video coding development since:

- MS-SSIM has high correlation with perceptual quality. Subjective results show that MS-SSIM outperform PSNR

- MS-SSIM emulates the HVS and it is easy to comprehend. It does not include any hidden features prone to overtraining.

- Our results comparing different versions of JEM indicate that MS-SSIM can differentiate small differences in compression efficiency to the same extent as PSNR

- MS-SSIM has been around since 2003 and has been evaluated and used as benchmark numerous times in the video community.

- Ericsson is one of the leading providers of video compression solutions. During encoder product development, we find MS-SSIM much more useful than PSNR

- We believe using MS-SSIM during FVC development can help creating a more efficient standard. Reaching a goal of 50% bitrate reduction for the same visual quality should be helped by a metric that is better at measuring visual improvements.

We propose to include MS-SSIM as an additional mandatory metric to PSNR for the CTC and the CfE on future video coding. By including MS-SSIM as a mandatory metric for the CfE, the metric could be further evaluated with subjective MOS data in the JVET context.

The zip-file for this contribution contains proposed changes to the draft CfE text on top of JVET-E1002-v1, an excel template for the CTC and excel sheets with results for the tests described in section 2.

# References

- H. Zhang, X. Ma, Y. Zhao, H. Yang, “Perceptual Quality Assessment Metric MS-SSIM”, JVET-C0030, May 2016

- Z. Wang, E. P. Simoncelli, and A. C. Bovik, “Multi-scale structural similarity for image quality assessment”, in IEEE Asilomar Conf. Signals, Systems, and Computers, Nov. 2003.

- M-J. Chen, A.C. Bovik, “Fast structural similarity index algorithm”, in IEEE International Conference on Acoustics Speech and Signal Processing (ICASSP), 2010

- D. Vranješ, D. Žagar and O. Nemčić, “Comparison of objective quality assessment methods for scalable video”, In ELMAR 2012 IEEE Proceedings, Sept 2012

- M. Spyridoula, “MSc Thesis: Objective Image Quality Assessment”, Kingston University, faculty of science, engineering and computing, July 2013

- J. Søgaard, L. Krasula, M. Shahid, D. Temel, K. Brunnström and  M. Razaak, Applicability of Existing Objective Metrics of Perceptual Quality for Adaptive Video Streaming. in IS&T International Symposium on Electronic Imaging 2016: Image Quality and System Performance XIII.

- C. Li, A. C. Bovik, “Three-component weighted structural similarity index”, In Proc SPIE, Image and System Performance VI, January 2009

- IETF draft “Video Codec Requirements and Evaluation Methodology”, Nov 17, 2016, https://tools.ietf.org/html/draft-ietf-netvc-requirements-04#ref-3

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).