| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0001 |
| --- | --- |

| Title: | JVET AHG report: Tool evaluation (AHG1) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Qualcomm Inc. Elena Alshina Samsung Electronics | Email: Email: | martak@qti.qualcomm.com elena_a.alshina@samsung.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 6h JVET meeting at Hobart, Australia (31 March – 07 April 2017) and the 7th Meeting at Turin, Italy (13– 21 July 2017).

# Mandates

At the 6th JVET meeting, AHG1 on Tool evaluation was established with the following mandates:

- Coordinate the exploration experiments.

- Investigate interaction of tools in JEM and exploration experiment branches.

- Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime.

- Study and summarize new technology proposals

# Summary of Activities

All together 50+ e-mails related to AhG1 and EE activities were sent to the JVET reflector including EE tests scheduling and EE summary discussion [1].

# JEM Status

Algorithms included into JEM6.0 are described in [2]. There is a list of tools below. Tools modified at the JVCT-F meeting are marked as bold. The biggest change is addition of division-free bilateral filter after inverse transform. Additionally to the Luma BD-rate gain some performance improvement of JEM5.0 over JEM4.0 is observed for Chroma due to the improvement both Intra mode coding and cross-color prediction for Chroma.

JEM6.0 tools:

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure

- Intra prediction improvements

  - 65 intra prediction directions

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation modified in JEM6.0

  - Bi-directional optical flow (BIO)  modified in JEM6.0

  - Decoder-Side Motion Vector Refinement (DMVR)

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms Signal dependent transform (SDT)  disabled by default

- In-loop filter

  - Bilateral filter

  - Adaptive loop filter (ALF)

  - Content adaptive clipping

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

Performance progress for JEM (HM-KTA) in terms of BD-rate gain vs. encoder time increase in random access test configuration is demonstrated on Figure 1. Results are based on Software Development AHG reports. Some encoder run-time reduction is observed for JEM6.0 compared to JEM5.0, but still encoder run time is much higher compared to HM (>10).

Screen content coding tools were enabled for HEVC at the last meeting for class F (screen content) which is optional ( not included to the averaging) It needs to be noticed that SCM 16.15 outperforms JEM in “all-intra” (19%) and random access (7%) configurations even JEM’s encoder is much slower.

Fig. 1. The progress of JEM performance in RA test configuration.

Table 1: Coding performance compared to HEVC summary.

JEM6.0 (6th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | -20% | -28% | -28% | 63 | 2 |
| Random Access | -29% | -36% | -35% | 12 | 10 |
| Low Delay-B | -22% | -28% | -29% | 10 | 8 |
| Low Delay-B | -26% | -31% | -32% | 7 | 5 |

JEM5.0 (5rd meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | -20% | -28% | -28% | 63 | 2 |
| Random Access | -29% | -35% | -34% | 12 | 10 |
| Low Delay-B | -22% | -29% | -29% | 10 | 8 |
| Low Delay-B | -26% | -31% | -32% | 7 | 5 |

Significant gain is observed in both three color components. In random access test highest gain over HEVC is observed for DaylightRoad test sequence (39.6%), lowest gain JEM shows for ToddlerFountain video (15.2% only).

# CfE test conditions

JVET Common Test Conditions use integer QP settings Qp=22, 27, 32, 37 for all videos in test set. For CfE bit-matching was performed. Table 2 shows BD-rate performance JEM6.0 for video test set form CfE [3].

Table 2. JEM5.0.1 vs HM under CfE test conditions

| Resolution | Seq. | BD-rate |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| 4K | CrossWalk1 | -37.9% | -43.9% | -47.6% |
|  | FoodMarket3 | -34.9% | -46.5% | -48.9% |
|  | Tango1 | -36.4% | -55.0% | -49.5% |
|  | CatRobot1 | -40.2% | -52.4% | -45.5% |
|  | DaylightRoad1 | -40.7% | -53.5% | -38.1% |
|  | BuildingHall1 | -33.3% | -41.4% | -46.5% |
|  | ParkRunning2 | -31.6% | -26.2% | -29.3% |
|  | CampfireParty | -37.9% | -35.7% | -56.6% |
| 2K | BQTerrace | -30.4% | -50.5% | -61.2% |
|  | RitualDance | -27.8% | -37.9% | -41.7% |
|  | TimeLapse | -26.8% | -61.3% | -67.1% |
|  | BasketballDrive | -32.1% | -46.9% | -43.3% |
|  | Cactus | -36.0% | -49.1% | -45.1% |
| 4K All |  | -36.6% | -44.3% | -45.2% |
| 2K All |  | -30.6% | -49.1% | -51.7% |
| All |  | -34.3% | -46.2% | -47.7% |

In SDR category (most relevant to this AhG) 2 CfE responses were submitted, even higher performance than Jem was demonstrated.

# Exploration Experiments activity

At the 2nd JVET meeting Exploration Experiments practice was established. In 6th JVET meeting 4 EEs were created. For each new coding tool under consideration special SW branch was created. After implementation of each tool announcement via JVET reflector was done. For all 4 EEs input contribution for this meeting were submitted. Summary of exploration experiments is provided in [2].

# New tools contributions to this Meeting

Increment for the number of technical contribution is observed. In total 20 contributions on coding tools were submitted in following categories:

- Structure (0)

- Intra (13),

- Inter (4),

- Transform (0),

- Entropy (1)

- In-loop filter (2).

| JVET-VC number | Title | Authors | Category |
| --- | --- | --- | --- |
| JVET-G0060 | Improvements for Intra Prediction Mode Coding | Y. Han, J. An, J. Zheng (HiSilicon) | Intra |
| JVET-G0062 | EE1-Related : Harmonization UW Prediction method with improved PDPC | H.M. Jang, J. Lim, S.-H. Kim (LGE) | Intra |
| JVET-G0065 | Simplification and improvements on FRUC | J. Seo, J. Lee, S.-H. Kim, H.M. Jang, J. Lim (LGE) | Inter |
| JVET-G0068 | Non EE1 : Unified-PDPC : unification of intra filters | M. Philippe, K. Sharman (Sony Europe) | Intra |
| JVET-G0076 | Bilateral filter simplification | R. Vanam, Y. He, Y. Ye (InterDigital) | Filter |
| JVET-G0077 | EE1: UWP&UW66 with PDPC for other intra mode and ARSS off (Test2) | H. M. Jang, J. Lim, S.-H. Kim (LGE), K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris), | Intra |
| JVET-G0078 | EE1 Test 1: UWP+UW66, PDPC off, ARSS on | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris) | Intra |
| JVET-G0079 | EE1 Test 10: UWP+UW66, PDPC on for other modes, ARSS constrained as in F0024 but explicit signaling as in F0055 | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris), H. M. Jang, J. Lim, S.-H. Kim (LGE) | Intra |
| JVET-G0080 | Additional EE1 Tests (Test 2.1 : UWP+UW66, PDPC for other modes, PDPC-L, ARSS off, and Test 10.1 : UWP+UW66, PDPC on for other modes, PDPC-L, ARSS constrained as in F0024 but explicit signaling as in F0055) | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris) | Intra |
| JVET-G0081 | Comparisons between UWP, W66 and Planar, Angular mode 66 under the same coding conditions | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris), | Intra |
| JVET-G0082 | EE2: A block-based design for Bi-directional optical flow (BIO) | H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm) | Inter |
| JVET-G0083 | EE2-related: A simplified gradient filter for Bi-directional optical flow (BIO) | H.-C. Chuang, J. Chen, K. Zhang, M. Karczewicz (Qualcomm) | Inter |
| JVET-G0090 | Unified adaptive search range setting in JEM and HM | T. Ikai, Y. Yasugi (Sharp) | Inter |
| JVET-G0095 | Unified Adaptive Loop Filter for Luma and Chroma | J. An, J. Zheng (HiSilicon) | Filter |
| JVET-G0104 | EE1: Alternative setting for PDPC mode and explicit ARSS flag (tests 3-7) | M. Karczewicz, V. Seregin, A. Said, N. Hu, X. Zhao (Qualcomm) | Intra |
| JVET-G0107 | Non-EE1: PDPC without a mode flag | V. Seregin, M. Karczewicz, A. Said, X. Zhao (Qualcomm) | Intra |
| JVET-G0108 | Non-EE1: Fix for strong intra smoothing filtering | V. Seregin, X. Zhao, M. Karczewicz (Qualcomm) | Intra |
| JVET-G0109 | A modification of fast algorithm in intra mode selection | P.-H. Lin, C.-L. Lin, C.-C Lin (ITRI) | Intra |
| JVET-G0112 | Arithmetic coding with context-dependent double-window adaptation response | A. Said, M. Karczewicz, L. Zhang, V. Seregin (Qualcomm) | Entropy |
| JVET-G0113 | EE1 (Tests 8 and 9) Performance of RASS and PDPC in presence of other tools | E. Alshina | Intra |

# Recommendations

The AHG recommends:

- Consider encoder complexity as one of the criteria when evaluating the tools. Encourage further encoder and decoder complexity reduction.

- To review all the related contribution.

- To continue Exploration Experiments practice.

# References

- E. Alshina, L. Zhang “Exploration Experiments on Coding Tools Report”, JVET-F0010, Turin, July, 2017

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm Description of Joint Exploration Test Model 6”, JVET-F1001, Hobart, March, 2017.

- M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki , Joint Call for Evidence on Video Compression with Capability beyond HEVC, JVET-F1002, Hobart, March, 2017