| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13 – 21 July 2017 | Document: JVET-G0022 |
| --- | --- |

| Title: | CfE response to the HDR category from Technicolor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | E. François, F. Le Léannec, C. Chevance, F. Hiron, M. Kerdranvat | Tel: Email: | edouard.francois@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes the Technicolor response to the Joint Call for Evidence on Video Compression with Capability beyond HEVC, for the HDR category. The proposed technology is directly based on the Exploratory Test Model mapping (a.k.a. reshaping with luma-dependent chroma scaling) previously tested in MPEG and JCT-VC. The proposed mapping, applying directly to the input YCbCr 4:2:0 BT.2100 PQ content, is based on one single scaling table used both for luma and chroma. The scaling table is used to build a mapping 1D look-up-table applied to the luma component, and to perform a luma-based cross-component scaling of the chroma components. One scaling table per shot was used in the reported experiments.

# Introduction

Both CfE HDR HM and JEM anchors are based on the adaptive luma QP adaptation described in [1,2,3]. The approach consists in deriving a local dQP value, per block, based on the average value of the luma prediction signal. The solution is non-normative (the dQP values are explicitly coded in the bitstream). The dQP table, experimentally built from visual testing, is plotted in Figure 1.

Figure 1. Suggested dQP table for PQ video signal.

Some proposals have been made to make the local QP adaptation process normative, by implementing in the decoder the luma-based dQP derivation (using a default dQP table or a table signaled in the bitstream).

During the MPEG and JCT-VC exploration phase on HDR coding technologies, an Exploratory Test Model (ETM) has been developed, based on the mapping of the input HDR signal prior to its encoding [4]. The solution developed in the ETM consists in mapping (or reshaping) the input luma and chroma components prior to the encoding, and performing the inverse mapping at the decoder.

# Description of the proposed solution

The solution proposed in this response is directly based on the ETM design. It is illustrated in Figure 2. The input YCbCr 4:2:0 HDR signal is pre-processed to produce a modified YCbCr 4:2:0 HDR signal that corresponds to the input of the core encoder. The core decoder outputs the reconstructed mapped YCbCr 4:2:0 video. Then, the inverse mapping is performed to reconstruct the YCbCr 4:2:0 HDR signal.

The encoder is a modified version of the JEM, in which the mapping-related metadata are inserted. The JEM decoder is also modified to include the decoding of the mapping-related metadata, and the inverse mapping process using these metadata of the reconstructed video.

Figure 2. Block diagram of the solution.

The proposed mapping is based on a single scaling table used for both luma and chroma, either transmitted in the bitstream, or selected using a coded index among a set of pre-defined tables. The scaling table is used to build the mapping Look-Up-Table (LUT) of the luma component, and to scale the chroma components.

The scaling table is defined per interval [ Yi, Yi+1 – l ], where i denotes the index of the interval between 0 and N-1, N being the total number of intervals. Let’s note Sci the scaling factor for the interval i.

The mapping function map( ) for the luma component is defined as follows, for a value Y in an interval i:

map( Y ) = map( Yi ) + Sci * ( Y – Yi )

The mapping of the luma component is done as follows:

Ymap = map( Yorig )

where Ymap is the remapped version of the original luma value Yorig.

Each chroma component (C = U or V), co-located to the luma Y, is rescaled as follows:

Cmap = offset + Sci * (Corig – offset )

where offset is equal to 2(bitdepth-1)

The decoder performs the inverse operations for the chroma and luma components:

Yinvmap = map-1( Ymap )

Cinvmap = offset + ( 1 ÷ Sci ) * (Cmap – offset )

For a division-free implementation, the scaling can be made as follows:

Umap = ( ( Urec – offset ) * lutScal[ Ymap ] + ( 1 << (B-1) ) + ( offset << B ) ) >> B

where B is the bit depth used for representing the scaling (typically 8 bits), offset is equal to 2bitdepthC-1 and

lutScal[ Y ] = invSci

with invSci  = Round( (1<<B) ÷ Sci ), i being the interval in which Y is included.

# Objective metrics

In the results generated for this response, a very simple process has been applied for deriving the mapping table. Three different tables were used and selected based on the content analysis. Only one table is signaled for all the pictures of a sequence.

The BD-rate variations using the recommended HDR objective metrics, compared to the HM and JEM HDR anchors are reported in Table 1 and Table 2, respectively. The reported wPSNR metrics are computed end-to-end, i.e.  between the source sequence and the inverse mapped sequence.

Table 1. HDR metrics using HM anchors as reference.

Table 2. HDR metrics using JEM anchors as reference.

Note: for wPSNRV of Cosmos1, the RD curves do not overlap.

Figure 3 presents different snapshots of source (PQ) and mapped pictures. It can be observed that the mapping makes the HDR content more “SDR-like”, and could therefore be used for SDR backward compatibility purposes.

Figure 3. Illustration of the mapping on different content (left: PQ version, right: mapped version).

# Comments

In HEVC as well as in the JEM, the scaling operations applied to the luma and the chroma transform coefficients of prediction residues are interdependent, since the chroma QP is additively dependent on the luma QP. In a first approximation, modifying the local luma QP by a value dQP results in an additional scaling of the luma and chroma signals by a factor of 2–dQP/6. This scaling applies per block. Like in the HDR anchors, the dQP, computed per block, is luma-dependent. Hence, there is a cross-component scaling of the chroma, depending on the luma signal.

In case of mapping, as illustrated in section 2, the same luma-dependent scaling factor is also used for building the mapping LUT for the luma (the scaling factor is the 1st derivative of the mapping function) and for scaling the chroma.

The main advantage of the mapping approach is that it applies at pixel-level, whereas the local luma-based QP adaptation applies per block. So a better accuracy can be obtained, as observed in the objective metrics reported in section 3. Another advantage is that the inverse mapping is a simple process, not intrusive in the core decoding process.

# References

[1] J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. J. Sullivan, P. Topiwala, A. Tourapis (editors), Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics (Draft 4), JCTVC-Z1017, Jan 2017, Geneva, CH.

[2] J. Zhao, S.-H. Kim, A. Segall, K. Misra, “Performance investigation of high dynamic range and wide color gamut video coding techniques”, MPEG contribution M37439, Oct 2015, Geneva, CH.

[3] A. Segall, J. Zhao, J. Strom, M. Pettersson, K. Andersson, “AHG on HDR and WCG: Average Luma Controlled Adaptive dQP”, JCTVC-W0054, Feb 2016, Geneva, CH.

[4] K. Minoo, T. Lu, P. Yin, L. Kerofsky, D. Rusanovskyy, E. François, “Description of the Exploratory Test Model (ETM) for HDR/WCG extension of HEVC,” JCTVC-W0092 / m37732, Feb 2016, Geneva, CH.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).