| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0055 |
| --- | --- |

| Title: | InterDigital Test Sequences for Virtual Reality Video Coding |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Eduardo Asbun, Yong He, Philippe Hanhart, Yan Ye 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Email: | Ed.Asbun@InterDigital.com |
| Source: |  | InterDigital Communications, Inc. |  |  |

____________________________

# Abstract

This contribution makes three Virtual Reality (VR) video sequences available to the JVET.  The video sequences are provided in 6K (6144x3072) and 8K (8192x4096) resolutions.

# Introduction

360° video is a rapidly growing format emerging in the media industry. It is enabled by the rising availability of VR devices and able to provide the viewer with an immersive experience.

# Sequences

The following 6K sequences are provided in equirectangular projection (ERP):

| Sequence | Resolution@FPS | Bit Depth | Frames |
| --- | --- | --- | --- |
| balboa360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |
| broadway360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |
| horton360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |

Sequences were re-rendered at 8K (8192x4096) resolution with the same stitching software used to generate the 6K sequences. The following 8K sequences are provided in equirectangular projection (ERP):

| Sequence | Resolution@FPS | Bit Depth | Frames |
| --- | --- | --- | --- |
| balboa360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |
| broadway360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |
| horton360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |

The contributed sequences were captured by InterDigital using GoPro Omni rig, a synchronized camera array with 6 GoPro Hero4 Black cameras. Kolor Autopano Video Pro is used to stitch the video. No encoding is done after stitching; therefore, the stitched content is provided in uncompressed raw YUV format.

# Sequence Thumbnails

- balboa360_6144x3072_60fps_600frames_8bits.yuv

- broadway360_6144x3072_60fps_600frames_8bits.yuv

- horton360_6144x3072_60fps_600frames_8bits.yuv

# Results

The three 6K sequences were encoded at four different QP values using HM-16.15 and JM-6.0. The table below summarizes the bit rate and PSNR values of HM and JM, respectively, for each QP value.  The Excel file that accompanies this document contains the full set of results.

|  |  | HM-16.15 |  |  |  | JEM-6.0 |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V |
| Balboa | 22 | 35402 | 42.76 | 51.36 | 51.48 | 30017 | 43.16 | 51.26 | 51.42 | -33.6% | -37.1% | -41.5% |
|  | 27 | 13901 | 39.93 | 49.62 | 49.85 | 11307 | 40.49 | 49.92 | 50.24 |  |  |  |
|  | 32 | 6252 | 37.32 | 47.80 | 48.13 | 4958 | 37.99 | 48.69 | 49.16 |  |  |  |
|  | 37 | 2881 | 34.77 | 46.19 | 46.60 | 2248 | 35.53 | 47.77 | 48.30 |  |  |  |
| Broadway | 22 | 43919 | 41.91 | 50.74 | 50.87 | 38077 | 42.29 | 50.67 | 50.85 | -33.6% | -39.4% | -42.5% |
|  | 27 | 16270 | 39.24 | 48.89 | 49.18 | 13434 | 39.77 | 49.30 | 49.63 |  |  |  |
|  | 32 | 7378 | 36.76 | 47.04 | 47.46 | 5841 | 37.44 | 48.08 | 48.57 |  |  |  |
|  | 37 | 3506 | 34.17 | 45.44 | 45.99 | 2709 | 34.99 | 47.13 | 47.69 |  |  |  |
| Horton | 22 | 36140 | 42.34 | 50.76 | 50.39 | 31748 | 42.68 | 50.80 | 50.49 | -32.5% | -43.8% | -45.0% |
|  | 27 | 12615 | 39.78 | 49.00 | 48.71 | 10677 | 40.27 | 49.51 | 49.22 |  |  |  |
|  | 32 | 5446 | 37.47 | 47.11 | 46.88 | 4383 | 38.08 | 48.32 | 48.07 |  |  |  |
|  | 37 | 2592 | 35.02 | 45.43 | 45.33 | 2003 | 35.80 | 47.23 | 47.01 |  |  |  |
| All |  |  |  |  |  |  |  |  |  | -33.20% | -40.11% | -42.98% |

# Sequence MD5 Sums

Uncompressed 8-bit YUV420 sequences in 6K and 8K resolution in equirectangular projection will be uploaded to MPEG content repository. The MD5 of each sequence is listed below:

| Sequence | MD5 |
| --- | --- |
| balboa360_6144x3072_60fps_600frames_8bits.yuv | 1457bb109ae0d47265f5a589cb3464d7 |
| balboa360_8192x4096_60fps_600frames_8bits.yuv | 32fa87fe48e0926fad1019e912bdc8f2 |
| broadway360_6144x3072_60fps_600frames_8bits.yuv | 1baf6667ef84bc3534809463faee8e82 |
| broadway360_8192x4096_60fps_600frames_8bits.yuv | 42573904d6662a0e61536e50d40c236d |
| horton360_6144x3072_60fps_600frames_8bits.yuv | 6b01584be2c806db5c06656ec10217b9 |
| horton360_8192x4096_60fps_600frames_8bits.yuv | 99a21fb108a88341e2d50f83be89db94 |

# Copyright Information

The sequences provided in this contribution are owned by InterDigital Communications, Inc.

The following uses are allowed for the contributed sequences:

- Sequences may be published in technical papers, played at technology research and development events.

- Sequences may be used by Standards committees. (e.g., ITU, MPEG, VQEG).

The following uses are NOT allowed for the contributed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.