| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, Italy, 13 – 21 July 2017 | Document: JVET-G0071 |
| --- | --- |

| Title: | AHG8: ACP with padding for 360-degree video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Muhammed Coban, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 geertv@qti.qualcomm.com, mcoban@qti.qualcomm.com, martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Padding is proposed for the adjusted cubemap projection (ACP) for efficient compression of 360-degree video and reduced seam artifacts in the rendered viewports. ACP is an enhancement of CMP by adjusting the sampling on the cube faces to be nearly uniform. Without padding, the RA coding gain is 11.0% (e2e WS-PSNR) compared with the equirectangular projection in accordance with the JVET common test conditions for 360-degree video, while with the proposed padding the gain is 10.7%.

# Introduction

After stitching of multiple input camera pictures, 360-degree video pictures are typically represented in the equirectangular projection format (ERP), which projects the 360-degree environment onto a spherical geometry and maps the sphere onto a 2D projected picture. Another popular projection format is the cubemap projection (CMP). The CMP projects the surface of the sphere onto planes (six faces of the cube) that are tangent to the sphere surface. The adjusted cubemap projection ACP [1][2][3] is an enhancement of CMP, which adjusts the sampling on the cube faces to be nearly uniform.

In ACP, the cubemap lookup vectors are modified to approximate uniform sphere sampling for addressing the non-uniform sampling problem of the CMP while preserving the packing scheme. The function

| f(x)=sgn(x)0.34-0.342-0.09x0.18 |  |
| --- | --- |

is used to modify the lookup vectors in the forward 2D-to-3D conversion. For the inverse 3D-to-2D conversion, its inverse function

| g(x)=sgn(x)(-0.36x2+1.36x) |  |
| --- | --- |

is used to modify the lookup vectors. The mapping functions are illustrated in Figure 1. The mapping enlarges the center of the cubemap face with respect to the CMP projection.

Figure 1. ACP forward and inverse mapping functions.

Video pictures of the ‘PoleVault’ sequence obtained with ERP, CMP, and ACP are illustrated in Figure 2.

| (a) | (b) |
| --- | --- |
| (c) |  |

Figure 2. (a) ERP picture, (b) CMP 3x2 picture, and (c) ACP 3x2 picture from ‘PoleVault’ sequence.

After the projection process, the video pictures are coded, stored or transmitted, and decoded at the receiver side where the renderer generates the viewport that matches the viewer’s head orientation. Depending on this orientation and the field of view, the viewport may cross the picture boundaries or boundaries within the picture between adjacent or non-adjacent regions as illustrated in Figure 3.

In the ERP case, the left and right picture boundaries wrap around the back of the sphere, hence, a viewport that is orientated towards the back of the sphere may cross the left and right picture boundaries as depicted in Figure 3(a). The region in the ERP picture that is required to render the viewport is represented by shaded rectangles for illustrative purposes (generally the shape is not rectangular). The boundaries within the region required for rendering are represented by bold line segments.

In case of 3×2 frame packing of the CMP or ACP cube faces as illustrated in Figure 2(b) and (c), the viewport may cross various boundaries as depicted in Figure 3(b) and (c). Although the left, front, and right cube faces are placed adjacent in the top picture half, there is still a boundary between the left and front faces, and between the front and right faces where, for example, lines are broken as is visible in Figure 2(b) and (c), and similarly for the bottom picture half, where there is a boundary between the top and back faces, and between the bottom and back faces. These boundaries between adjacent cube faces are less discontinuous than the actual picture boundaries and the boundary between the top and bottom halves of the picture. Figure 3(b) and (c) illustrate two viewports (‘evil viewports’) as defined in [4] with regions required for rendering that cross various boundaries. In these examples, a 3×2 packing of the cube faces is chosen; however, alternative packing schemes exist that have different boundaries. The 3×2 packing has one of the fewest boundaries of the packing schemes.

| - ERP |  |
| --- | --- |
| - CMP, ACP: viewport1 (-135°, 35°) | - CMP, ACP: viewport2 (45°, -35°) |

Figure 3. Viewport regions (shaded) crossing projected picture boundaries (bold) and boundaries between adjacent and non-adjacent picture regions.

The picture boundaries and boundaries between adjacent and non-adjacent regions within the picture may result in visible seam artifacts within the rendered viewport. The following is a list of main causes:

- Motion: In case of motion across the picture boundary directed into the region, the motion estimation process may select a block that partially includes the padded samples that the encoder adds to extend the reference pictures. At higher QP values, those padded samples appear on one side of the boundary in the viewport, while the other side of the boundary is different as illustrated in Figure 4(a). This type of seam artifact is strong at higher QP values. A similar problem occurs inside the CMP or ACP picture (3×2 packing), when motion estimation selects a block that is located partially across the middle picture boundary.

- In-loop filtering: Since filtering is disabled on the picture boundaries, a sharp unfiltered seam may be visible in the rendered viewport especially at higher QP values. In the CMP or ACP picture (3×2 packing), the deblocking filtering may occur across the middle picture boundary resulting in visible seam artifacts as illustrated in Figure 4(b). Disabling in-loop filtering across tile boundaries (top and bottom picture halves) is not feasible if the middle picture boundary falls within the CTUs.

- …

| (a) | (b) |
| --- | --- |

Figure 4.  (a) Example of rendered viewport seam due to motion. (b) Viewport seam due to filtering. (‘ChairliftRide’ sequence)

# Proposal

Guard bands or padded regions are proposed for the ACP projection. Guard bands are regions added into the projected picture consisting of additional samples that are not essential for projecting the picture back onto the sphere, but they are helpful in reducing artifacts such as seams on the sphere or in the rendered viewport. The proposed padded regions consist of additional projected samples, i.e., samples obtained around the projected regions with the ACP projection process. The regions are added to the projected picture before encoding, hence, they are available to the video encoding processes for prediction, such as motion compensation, which may improve coding efficiency somewhat.

Figure 5 illustrates the proposed guard bands for 3×2 packed ACP pictures. Figure 5(b) illustrates the case where the guard bands are added along the top and bottom 3×1 halves of the projected picture. The reason is that the cube faces within the top and bottom halves are adjacent to avoid discontinuous edges. The assumption to support this type of guard band regions is that the seam artifacts corresponding with the adjacent cube face edges are less visible in the rendered viewport and, hence, require no padding.

The padding width to completely remove the seams is dependent on the motion intensity in the video. However, for most of the test sequences it was experimentally determined that padding width equal to 4 samples is a reasonable trade-off between quality and coding efficiency (see Section 4).

Figure 5. Proposed guard bands for 3x2 packed ACP. Guard bands are added around the top and bottom 3x1 picture halves.

# ACP with padding

This section describes the proposed ACP padding utilizing projected samples in detail. Denote the dimension of any ACP square face as A×A in sample units. For 2D-to-3D coordinate conversion, given the position (m, n) on a given face f, (u, v) is first calculated as:

| u = (m + 0.5) * 2 / A − 1, 0 ≤ m < A, -1 < u < 1 |  |
| --- | --- |
| v = (n + 0.5) * 2 / A − 1, 0 ≤ n < A, -1 < v < 1 |  |

The following scaling factors are defined:

| padfactor = 1+2 × padmarginA-2 × padmargin apadfactor = AA-padmargin. |  |
| --- | --- |

with padmargin specifying the width of the guard bands in sample units (must be smaller than A/2), for example, 4 samples. Subsequently, the (u, v) coordinates per cube face f are scaled into (u’, v’) per Table 1. The scaling is followed by the forward ACP adjustment functions (Eqn. 1). The 3D coordinates (X, Y, Z) are derived using the position (u’, v’) and the face index f (see Table 3 in [2]).

Table 1. Scaling of (u,v) coordinates in 2D-to-3D conversion of ACP guard bands.

| Face | Index f | 2D-to-3D Conversion |
| --- | --- | --- |
| Front | 0 | u'=u v'=v × padfactor |
| Back | 1 | u'=u × padfactor v'=v |
| Top | 2 | u'=u+1 × apadfactor-1 v'=v × padfactor |
| Bottom | 3 | u'=u+1 × apadfactor-1 v'=v × padfactor |
| Left | 4 | u'=u-1 × apadfactor+1 v'=v × padfactor |
| Right | 5 | u'=u+1 × apadfactor-1 v'=v × padfactor |

In case of 3D-to-2D coordinate conversion, given (X, Y, Z), the (u, v) and face index f is calculated per Table 4 in [2]. Subsequently, the scaling in Table 2 is applied to obtain the final (u’, v’) coordinates. The scaling is performed after the inverse ACP adjustment functions (Eqn. 2).

Table 2. Scaling of (u,v) coordinates in 3D-to-2D conversion of ACP.

| Face | Index f | 3D-to-2D Conversion |
| --- | --- | --- |
| Front | 0 | u'=u v'=v / padfactor |
| Back | 1 | u'=u / padfactor v'=v |
| Top | 2 | u'=u+1 / apadfactor-1 v'=v / padfactor |
| Bottom | 3 | u'=u+1 / apadfactor-1 v'=v / padfactor |
| Left | 4 | u'=u-1 / apadfactor+1 v'=v / padfactor |
| Right | 5 | u'=u+1 / apadfactor-1 v'=v / padfactor |

# Results

## Common test conditions

The proposed ACP with padding is implemented into the JVET 360Lib-3.0 [3] library (HM16.15) for evaluation. The padding width is made configurable and is set to 4 samples. Per common test conditions for 360-degree video [4], the random access (RA) coding gain of ACP without padding and 3×2 packing is enumerated per sequence in Table 3. The ACP face sizes are set equal to cube map face sizes in [4]. Frame sizes are 3552×2368 for 8K test sequences and 2880×1920 for 4K. Results in Table 4 are for ACP with padding (width = 4 samples). The end-to-end coding gain (WS-PSNR) slightly decreases from 11.0% to 10.7%.

Table 3. BD-rates for RA test conditions. ACP without padding.

| ACP vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -7.5% | 2.9% | -1.5% | -7.7% | 2.8% | -1.6% | -7.7% | 2.9% | -1.6% | -7.6% | 3.0% | -1.6% |
| GasLamp | 1184 | 1184 | -4.3% | 2.2% | 7.4% | -4.4% | 1.9% | 7.1% | -4.3% | 2.0% | 7.1% | -4.2% | 2.1% | 7.3% |
| Skateboarding_in_lot | 1184 | 1184 | -16.1% | -16.2% | -16.7% | -16.3% | -16.3% | -16.8% | -16.1% | -16.3% | -16.8% | -16.0% | -16.1% | -16.7% |
| Chairlift | 1184 | 1184 | -25.2% | -18.5% | -18.7% | -25.2% | -18.5% | -18.8% | -25.2% | -18.5% | -18.8% | -25.2% | -18.5% | -18.8% |
| KiteFlite | 1184 | 1184 | -9.9% | -3.2% | -4.6% | -10.1% | -3.3% | -4.7% | -10.0% | -3.3% | -4.6% | -9.8% | -3.1% | -4.5% |
| Harbor | 1184 | 1184 | -9.2% | -3.4% | -2.3% | -9.3% | -3.5% | -2.4% | -9.7% | -3.5% | -2.4% | -9.5% | -3.4% | -2.3% |
| PoleVault | 960 | 960 | -9.6% | -9.6% | -9.7% | -9.2% | -9.5% | -9.7% | -9.2% | -9.5% | -9.8% | -9.6% | -9.6% | -9.7% |
| AerialCity | 960 | 960 | -3.1% | -2.2% | -1.7% | -3.3% | -2.5% | -2.1% | -3.2% | -2.5% | -2.1% | -3.0% | -2.2% | -1.8% |
| DrivingInCity | 960 | 960 | -4.7% | 4.4% | 4.0% | -4.0% | 3.9% | 3.6% | -3.9% | 3.9% | 3.6% | -4.5% | 4.3% | 4.1% |
| DrivingInCountry | 960 | 960 | -21.1% | -18.1% | -18.9% | -20.4% | -18.3% | -19.1% | -20.4% | -18.3% | -19.1% | -21.1% | -18.1% | -18.8% |
| Overall |  |  | -11.1% | -6.2% | -6.3% | -11.0% | -6.3% | -6.4% | -11.0% | -6.3% | -6.5% | -11.0% | -6.1% | -6.3% |

Table 4. BD-rates for padded ACP (padding width = 4 samples).

| ACP with padding vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -6.7% | 3.1% | -1.1% | -6.9% | 2.9% | -1.2% | -6.9% | 2.9% | -1.2% | -6.8% | 3.0% | -1.2% |
| GasLamp | 1184 | 1184 | -3.6% | 2.3% | 7.4% | -3.7% | 2.2% | 7.2% | -3.7% | 2.2% | 7.3% | -3.6% | 2.4% | 7.4% |
| Skateboarding_in_lot | 1184 | 1184 | -16.0% | -16.4% | -17.1% | -16.1% | -16.6% | -17.2% | -16.0% | -16.6% | -17.3% | -15.9% | -16.4% | -17.2% |
| Chairlift | 1184 | 1184 | -25.0% | -18.1% | -18.3% | -25.0% | -18.1% | -18.3% | -25.1% | -18.1% | -18.4% | -25.0% | -18.1% | -18.4% |
| KiteFlite | 1184 | 1184 | -9.3% | -2.5% | -4.2% | -9.5% | -2.7% | -4.3% | -9.4% | -2.7% | -4.3% | -9.2% | -2.6% | -4.2% |
| Harbor | 1184 | 1184 | -8.7% | -3.8% | -2.9% | -8.9% | -3.9% | -2.9% | -9.2% | -3.9% | -2.9% | -9.0% | -3.8% | -2.8% |
| PoleVault | 960 | 960 | -8.9% | -9.9% | -9.8% | -8.5% | -9.7% | -9.7% | -8.5% | -9.7% | -9.9% | -8.8% | -9.8% | -9.8% |
| AerialCity | 960 | 960 | -3.3% | -1.8% | -1.4% | -3.5% | -2.1% | -1.7% | -3.4% | -2.1% | -1.7% | -3.2% | -1.8% | -1.4% |
| DrivingInCity | 960 | 960 | -4.5% | 4.4% | 4.1% | -3.9% | 4.0% | 3.6% | -3.7% | 4.0% | 3.6% | -4.4% | 4.4% | 4.1% |
| DrivingInCountry | 960 | 960 | -20.7% | -18.2% | -19.0% | -20.1% | -18.4% | -19.1% | -20.1% | -18.4% | -19.0% | -20.7% | -18.2% | -18.9% |
| Overall |  |  | -10.7% | -6.1% | -6.2% | -10.6% | -6.2% | -6.4% | -10.6% | -6.2% | -6.4% | -10.7% | -6.1% | -6.2% |

## Viewports

Figure 6 and Figure 7 depict ACP viewport2 (Figure 3b) to compare the seam regions with and without padding for the ‘Chairlift and ‘Harbor’ sequences. Figure 8 illustrates the picture corner seams (viewport1) with the ‘Harbor’ sequence. The test conditions are RA and QP ~ 32. In these cases, the viewport seams are adequately removed by applying the 4-sample padding. In case of high motion intensity some seams may be slightly visible, but they are not objectionable given the overall low quality of the content at QP 32, for example.

(a)

(b)

Figure 6. Viewport2 from ‘Chairlift’ sequence (frame 0). (a) Without padding and (b) with padding.

(a)

(b)

Figure 7. Viewport2 from ‘Harbor’ sequence (frame 0). (a) Without padding, (b) with padding.

(a)

(b)

Figure 8. Viewport1 from ‘Harbor’ sequence (frame 20). (a) Without padding, (b) with padding.

# Conclusion

ACP with padding was proposed. The reported coding gain without padding is 11.0% (e2e WS-PSNR) for RA conditions, while the gain is 10.7% with the proposed padding. It is claimed that the seam artifacts in the rendered viewports are adequately reduced. It is requested to add the ACP padding to 360Lib.

# References

- M. Coban, G. Van der Auwera, M. Karczewicz, “AHG8: Adjusted cubemap projection for 360-degree video,” 6th JVET Meeting, Hobart, AU, March-April 2017, JVET-F0025.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib,” 6th JVET Meeting, Hobart, AU, March-April 2017, JVET-F1003.

- 360Lib-3.0: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-3.0/

- E. Alshina, J. Boyce, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, 6th JVET Meeting, Hobart, AU, March-April 2017, JVET-F1030.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).