| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, July 13-21, 2017 | Document: JVET-G0106 |
| --- | --- |

| Title: | EE3: Adaptive QP for 360° video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This contribution reports the results of EE3 [1] for Test 2 (Adaptive QP with F0072 weighting for ERP in HM-360Lib), test 2.1 (Adaptive QP with F0072 weighting for rotated ERP in HM-360Lib) and test 5 (Adaptive QP (as in F0072) for CMP in HM-360Lib (with signaling)). Test results show that the adaptive method yields 4.4%, 8.4%, and 2.9% in BD-rate (using codec WS-PSNR) reduction on average for ERP, rotated ERP and CMP respectively in comparison to coding with fixed QP.

Introduction

To improve coding efficiency of 360 video, areas where weights of coding errors are larger should be assigned with more bits and less coding errors. It can be solved by using finer quantization to encode areas with larger weights. We propose an adaptive quantization method implemented in CTU level. Each CTU is encoded with a QP offset based on the WS-PSNR [2] weights. The adaptive QP used for different projection formats are derived through the following three steps.

- Step 1: Quantization step derivation according to WS-PSNR weights

Assuming residual distribution is uniform within each quantization interval, the relationship between Qstep(i) and spherical distortion Di (WMSE) of i-th CTU is

Di=wi*Qstep(i)212                (1)

where wi is the average WS-PSNR weight inside the i-th CTU.  Qstep(i) is the distance of the quantization interval in i-th CTU.

In order to use finer quantization on CTU position where average weight wi is larger, we adjust Qstep(i) to keep spherical distortion of each CTU in picture equal. The Qstep(i) is adjusted based on slice quantization step Q'step according to (1):

Qstepi=Q'stepwi.         (2)

- Step 2: QP offset for each CTU

In HEVC, the relationship between Qstep and QP is

Qstep≈2QP-46.      （3）

Then slice QP can add a delta QP so that spherical distortion of each CTU is equal. Considering QP is an integer, delta QP for each CTU can be obtained by

∆QPi=3*log21wi+0.5,         (4)

where (6) is derived according to (2) and (3). ∆QPi represents delta QP of the i-th CTU.

As all the WS-PSNR weights are between 0 to 1, which results in all the delta QPs are not negative. To make average QP of all CTUs close to original slice QP, average delta QP is subtracted from delta QP of each CTU, which is equals to normalize WS-PSNR weights. Thus, final QP offset of the i-th CTU is calculated as

QP_offseti=∆QPi-∆QPave,         (5)

where ∆QPave is average delta QP which can be derived using average WS-PSNR weights of all CTU.

ERP and CMP are two typical projection formats of 360 video. For an ERP with resolution 3328*1664, the QP offsets of 52*26 CTUs range from -2 to 10. Figure 1 illustrates the QP offsets of the ERP, where the QP offset is -2 in equator and 10 near the pole. Weight distributions of WS-PSNR on all faces are the same for CMP, therefore only QP offsets on one face are shown on Figure 2. QP offset of each CTU on one 960*960 face ranges from -3 to 4.

Figure 1. QP offset of each CTU on the 3328*1664 ERP

Figure 2. QP offset of each CTU on one 960*960 face of CMP

- Step 3: Coded QP of each CTU

After obtaining the QP offset of each CTU for one specific projection format, the coded QP of i-th CTU can be updated within the range from 0 to 51:

QPi=0 ,        sliceQP+QP_offseti<0      51,  sliceQP+QP_offseti>51  sliceQP+QP_offseti,                else.            (6)

Simulation results

The tests are conducted according to the JVET common test conditions [3] and on HM 16.15-360Lib with RA configuration. For test 2.1, rotation of (0, 0, 90) is performed to rotate complex content near the equator to the pole, which is shown in Fig.3. BD-rate using codec WS-PSNR is reported as this is a coding tool related method. And End to End WS-PSNR is also listed below. Other metrics’ results can be seen in attached excel.

Table 1 BD-Rate for coded sequence in ERP

| DeltaQP（signaling)_vs_FixedQP | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -1.0% | -2.2% | -5.2% | -0.9% | -2.2% | -5.2% |
| GasLamp | -1.3% | -1.2% | 3.2% | -1.5% | -1.2% | 3.2% |
| Skateboarding_in_lot | -8.2% | -17.9% | -15.6% | -8.8% | -18.0% | -15.6% |
| Chairlift | -9.1% | -9.1% | -10.2% | -9.3% | -9.1% | -10.2% |
| KiteFlite | -2.9% | -3.8% | -4.4% | -2.9% | -3.8% | -4.4% |
| Harbor | -0.6% | -4.6% | -3.8% | -0.7% | -4.5% | -3.7% |
| PoleVault | -5.9% | -9.6% | -10.5% | -6.2% | -8.5% | -9.5% |
| AerialCity | -4.0% | -5.1% | -5.3% | -4.3% | -4.9% | -5.2% |
| DrivingInCity | 0.4% | 2.8% | 2.6% | 0.4% | 2.8% | 2.6% |
| DrivingInCountry | -9.3% | -13.6% | -15.3% | -9.4% | -13.6% | -15.2% |
| Overall | -4.2% | -6.4% | -6.4% | -4.4% | -6.3% | -6.3% |

Figure 3. Illustration of rotated ERP (DrivingInCity sequence)

Table 2 BD-Rate for coded sequence in rotated ERP

| DeltaQP（signaling)_vs_FixedQP | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -6.6% | -9.8% | -8.9% | -7.0% | -9.7% | -8.9% |
| GasLamp | -6.0% | -7.5% | -11.1% | -6.1% | -7.4% | -11.0% |
| Skateboarding_in_lot | -7.7% | -8.0% | -8.4% | -7.6% | -8.0% | -8.4% |
| Chairlift | -8.3% | -14.1% | -12.6% | -8.4% | -14.1% | -12.6% |
| KiteFlite | -4.6% | -7.4% | -6.6% | -4.8% | -7.4% | -6.6% |
| Harbor | -7.2% | -6.9% | -6.2% | -7.5% | -6.8% | -6.1% |
| PoleVault | -6.9% | -9.3% | -7.9% | -7.5% | -8.9% | -7.6% |
| AerialCity | -8.7% | -15.9% | -15.9% | -9.7% | -15.9% | -15.9% |
| DrivingInCity | -14.7% | -27.2% | -26.8% | -15.1% | -27.2% | -26.8% |
| DrivingInCountry | -10.3% | -14.9% | -16.0% | -10.6% | -14.8% | -15.9% |
| Overall | -8.1% | -12.1% | -12.0% | -8.4% | -12.0% | -12.0% |

Table 3 BD-Rate for coded sequence in CMP

| DeltaQP（signaling)_vs_FixedQP | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -1.6% | -3.9% | -3.1% | -2.3% | -3.8% | -3.1% |
| GasLamp | -1.6% | -4.5% | -5.3% | -2.1% | -4.4% | -5.2% |
| Skateboarding_in_lot | -2.8% | -6.9% | -7.3% | -3.5% | -7.0% | -7.4% |
| Chairlift | -3.0% | -6.8% | -6.8% | -3.5% | -6.8% | -6.7% |
| KiteFlite | -0.8% | -4.2% | -2.8% | -1.3% | -4.2% | -2.7% |
| Harbor | -1.3% | -0.7% | -0.5% | -2.0% | -0.6% | -0.4% |
| PoleVault | -2.8% | -4.7% | -5.6% | -3.5% | -4.5% | -5.4% |
| AerialCity | -2.7% | -5.1% | -4.8% | -3.4% | -5.0% | -4.7% |
| DrivingInCity | -2.1% | -3.2% | -4.4% | -2.5% | -3.3% | -4.4% |
| DrivingInCountry | -3.8% | -8.8% | -10.2% | -4.5% | -8.8% | -10.1% |
| Overall | -2.2% | -4.9% | -5.1% | -2.9% | -4.8% | -5.0% |

Issues in subjective viewing: (for ERP, pole on the pole with high QP)

Figure 4. Viewport of North Pole (AerialCity: coded with fixed QP 37)

Figure 5. Viewport of North Pole (AerialCity: coded with adaptive QP 37)

Figure 5. Viewport of North Pole (DrivingInCountry: coded with fixed QP 37)

Figure 6. Viewport of North Pole (DrivingInCountry: coded with adaptive QP 37)

Additional information: compared with signaling adaptive QP method, the results of without signaling method are provided.

Table 4 BD-Rate for coded sequence in ERP

| DeltaQP NoSignaling_vs_DeltaQP Signaling | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| GasLamp | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% | 0.1% |
| Skateboarding_in_lot | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Chairlift | -0.1% | 0.1% | 0.2% | -0.1% | 0.1% | 0.2% |
| KiteFlite | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% |
| Harbor | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| PoleVault | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| AerialCity | -0.3% | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% |
| DrivingInCity | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | 0.1% |
| DrivingInCountry | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |

Table 5 BD-Rate for coded sequence in rotated ERP

| DeltaQP NoSignaling_vs_DeltaQP Signaling | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |
| GasLamp | -0.1% | -0.2% | 0.3% | -0.1% | -0.2% | 0.3% |
| Skateboarding_in_lot | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% |
| Chairlift | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| KiteFlite | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Harbor | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| PoleVault | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| AerialCity | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% |
| DrivingInCity | -0.2% | 0.0% | -0.4% | -0.2% | 0.0% | -0.5% |
| DrivingInCountry | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |

Table 6 BD-Rate for coded sequence in CMP

| DeltaQP NoSignaling_vs_DeltaQP Signaling | WS-PSNR (End to End) |  |  | WS-PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Trolley | -0.3% | -0.3% | -0.5% | -0.2% | -0.3% | -0.5% |
| GasLamp | -0.3% | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% |
| Skateboarding_in_lot | -0.5% | -0.2% | -0.2% | -0.5% | -0.2% | -0.2% |
| Chairlift | -0.3% | -0.3% | 0.0% | -0.3% | -0.3% | 0.0% |
| KiteFlite | -0.2% | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% |
| Harbor | -0.4% | -0.1% | -0.4% | -0.4% | -0.1% | -0.4% |
| PoleVault | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |
| AerialCity | -0.5% | -0.3% | -0.6% | -0.5% | -0.3% | -0.6% |
| DrivingInCity | -0.5% | -0.7% | -0.6% | -0.4% | -0.7% | -0.6% |
| DrivingInCountry | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% | 0.0% |
| Overall | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |

Conclusion

Adaptive quantization method is presented to improve coding efficiency for 360 videos. Different CTUs in each coded frame are assigned with different QP offsets according to WS-PSNR weights. Lower QP is applied to the CTUs with larger average weights. Experimental results show that the proposed method yields 4.4%, 8.4%, and 2.9% in BD-rate (codec WS-PSNR) reduction on average for ERP, rotated ERP and CMP respectively in comparison to coding with fixed QP.

References

[1] E. Alshina, and L. Zhang, “Description of Exploration Experiments on Coding Tools,” JVET-F1011, April 2017.

[2] Y.Sun, A. Lu and L.Yu, " Weighted-to-Spherically-Uniform Quality Evaluation for Omnidirectional Video." IEEE Signal Processing Letters. Link: http://ieeexplore.ieee.org/document/7961186/ .

[3] J. Boyce, E. Alshina, A. Abbas, and Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video,” JVET-F1030, Apr 2017

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).