| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0125 |
| --- | --- |

| Title: | EE3: Cross-check for JVET-G0106 Test2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document presents the cross-verification results of JVET-G0106 on adaptive quantization of 360-degree video coding for the ERP format, proposed by Zhejiang University. It is verified that all the simulation results are matched with that provided by the proponent and the software implementation is consistent with the description of JVET-G0106.

# Introduction

In the Hobart meeting, one adaptive quantization method was proposed in [1] to improve the coding efficiency of the equirectangular projection (ERP) format and the cubemap projection (CMP) format based on HM. The method is included in the current EE3 for further evaluation [2]. Compared to the scheme using a fixed quantization parameter, the method can improve the uniformity of the spherical distortion of the reconstruction distortion by adjusting the QP value at the CTU level based on the weight derived by the WS-PSNR, as shown as

| ∆QPi=3*log21wi+0.5 | (1) |
| --- | --- |

Additionally, one delta weight normalization method is applied by subtracting the average of the delta QP of all the CTUs from delta QP of each CTU. Correspondingly, final QP offset of one CTU is calculated as

| QP_offseti=∆QPi-∆QPave | (2) |
| --- | --- |

This document cross-checks the EE3 Test2 on adaptive quantization on the ERP format. The software used in this cross-check was downloaded from the current EE3 branch on the svn server [3]. The software implementation is consistent with the contribution description and all the simulation results are perfectly matching the results provided from the proponents.

# Simulation results

The detailed results are presented and bundled in the same archive.

# References

- Yule Sun, Lu Yu, “AHG8: stretching ratio based adaptive quantization for 360 video”, JVET-F0072, Mar. 2017, Hobart, AU.

- E. Alshina, L. Zhang, “Description of Exploration Experiments on Coding Tools”, JVET-F1011, Mar. 2017, Hobart, AU.

- https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/branches/360Lib-3.0-EE3-F0072/