| Joint Video Exploration Team (JVET) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: Macao, CN, 18–24 October 2017 | Document: JVET-H0047 |
| --- | --- |

| Title: | Perceptually optimized QP adaptation and associated distortion measure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET (AHG10) |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sebastian Bosse Christian Helmrich Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

The peak signal-to-noise ratio (PSNR) measure has been reported to show a lower correlation with results of corresponding subjective evaluations than some other measures.  Moreover, it is also known that constant quantization parameter (QP) assignments frequently lead to visually suboptimal results.  To reduce this divergence between objective measurements and subjective quality assessments, a generalization of PSNR is introduced, called perceptually weighted PSNR (WPSNR) together with a corresponding block-wise adaptive QP selection method that is optimized for subjective quality.

The suggested block-wise QP adaptation at CTU granularity provides significant improvements in subjective quality compared to a constant QP assignment for each picture as illustrated by a subjective test and examples. These subjective improvements result in PSNR decreases, but in corresponding WPSNR increases. Moreover, WPSNR shows high correlation with results of subjective quality assessments similar to other perceptually motivated measures such as (MS-)SSIM [9], VIF [10], or FSIM [11].

The weights of the WPSNR measure are derived via the impact of the QP adaptation method on the block-wise Lagrange parameters used in the Lagrangian bit allocation. Hence, as for the combination of a picture-wise constant QP assignment and PSNR, the combination of adaptive QP assignment and the corresponding WPSNR measure lead to operating points on the convex hull of the rate distortion space. Therefore, when due to a coding tool an improvement can be determined for the combination constant-QP/PSNR, a similar improvement can be determined for the combination adaptive-QP/WPSNR.

Given the advantages of WPSNR over PSNR and the strong similarity between the two methods (one is the generalization of the other), it is suggested that JVET considers test conditions for their standardization work that use an adaptive QP assignment (for improved subjective quality) and an associated WPSNR measure.

# Introduction

It is common practice in video coding standardization to use fixed quantizer settings [1] for evaluating the coding efficiency of proposed coding tools or comparing the coding efficiency of different codec designs.  While the quantization parameter (QP) is typically cascaded among hierarchy levels in hierarchical coding structures (such as hierarchical B frames), it is kept constant inside each video frame.  However, such a constant QP strategy often leads to perceptually suboptimal results.  For a given fixed QP, coding artefacts are often only perceivable in some regions of an image or video.  For example, it can often be observed that the subjective quality of low-contrast image regions (i.e., regions with low visual activity) is noticeably degraded while at the same time no coding artefacts are visible in high-contrast image regions (i.e., regions with high visual activity).

The perceptual video quality can often be improved if adaptive QP assignment strategies are used within video frames. Examples for such QP assignment strategies are described in [3][4].  Even though suitable QP adaptation concepts often yield an improved subjective quality, these quality improvements are not reflected in the peak signal-to-noise ratio (PSNR) measure, which is used in video coding standardization.  Hence, for enabling the objective measurement of small coding efficiency improvements, we use fixed QP assignments in the standardization process, even though it is well known that more advanced QP selection schemes can substantially improve the perceptual video quality (some evidence is presented in sec. 4.1).

In this document, we discuss a perceptually weighted PSNR (WPSNR) metric which enables the measurement of small coding efficiency improvements for a block-adaptive QP selection strategy. The underlying concept is as follows: Block-based weighting factors, which can be interpreted as visual sensitivity weights [22], are calculated based on properties of the original samples in a block.  Instead of calculating the sum of squared errors (SSE) as distortion measure for a picture, a weighted SSE is calculated, where the SSE for each block is scaled with a corresponding weighting factor. WPSNR is then computed using the weighted SSE instead of the conventional SSE.  The same weighting factors are used for determining the block-based QP selection.  Note that for a given concept of determining the block-based weights (and a corresponding QP assignment rule), WPSNR can be used in exactly the same way as the conventional PSNR measure in determining Bjøntegaard delta rate or delta PSNRs as well as delta WPSNRs [21]. On the one hand, an improved perceptual quality of adaptive QP assignment schemes is measurable in the corresponding WPSNR, and on the other hand, small coding efficiency improvement of various coding tools are still measurable using WPSNR.

We suggest that the usage of an appropriate block-based adaptive QP assignment together with a quality measurement using the associated WPSNR will allow to investigate video coding tools in a more realistic encoder setting. For the standardization process, the JVET could specify the block-based adaptive QP assignment together with the WPSNR measure in its test conditions.  Given these definitions, the gains of individual coding tools can be measured using the well-understood Bjøntegaard delta rates for the new WPSNR measure.

Even though a block-based weighting of the SSE distortion is unlikely to provide the best objective measure for subjective quality, it has some advantages:

- Easy computation in both encoder and a corresponding WPSNR measurement;

- All block-based encoder decisions are conducted using the conventional SSE measure:

  - The block encoding process does not change;

  - Simple quantization, since SSE can be calculated in transform domain.

The usefulness of a block-based adaptive QP assignment and the corresponding WPSNR is highly dependent on the algorithm for deriving block-wise weighting factors.  We suggest a simple low-complexity algorithm, which showed significant improvements in perceptual quality in our viewing tests.

It should be noted that the discussed weighted SNR metric is conceptually similar to the wPSNR specified in the JVET common test conditions and evaluation procedures for HDR/WCG video [2].  In the HDR setting, an improved QP assignment is derived based on the average luma value in a block using a look-up table (subjective evaluations showed that such QP assignment leads to improvements in perceptual quality).  For obtaining a quality measure that reflects the improvement in visual quality, a corresponding weighted PSNR metric is introduced (which weights the squared error for each sample using the same look-up table).

While working on the WPSNR method, two aspects were considered to allow for efficient implementations:

- Low complexity.  The WPSNR algorithm is specified such that the computationally expensive operations can be fully implemented in fixed-point arithmetic and, due to integer-multiple-of-four memory address offsets, with microprocessor extensions like Single Input Multiple Data (SIMD). Long filters, transforms like DCTs, and similar algorithmically complex operations were avoided.

- High perceptual relevance.  The WPSNR algorithm is designed to deliver perceptually meaningful results which correlate well with corresponding subjective data when the reference input and test output are viewed on (a) a typical display for the given image/video size (e. g., a television) from (b) a typical distance using (c) a typical display luminance (ranging from around 100 ~ 101 nits and mesopic vision to 103 ~ 104 nits and photopic vision).  Region-of-interest modeling is not included.

A detailed description of the WPSNR algorithm and the choice of its parameters is given in the following.

# Block-Based Weighted SSE Distortion Measures

Let DkSSE denote the sum of squared errors (SSE) for a block Bk, where k represents the block index. With s and s' being the original and the reconstructed signal for the considered picture, it is given by

| DkSSE=x,y∈Bks[x,y]-s'[x,y]2. | (1) |
| --- | --- |

We assume that a distortion measure DkwSSE which better reflects the subjective quality for a block Bk (in comparison to other blocks) can be defined according to

| DkwSSE=wk⋅DkSSE, | (2) |
| --- | --- |

where the weighting factors wk for the blocks Bk of a picture can be basically any measures that only depend on the original samples of the considered video picture.  The weighting factors wk can be interpreted as visual subjective sensitivity measures for the blocks.  For blocks Bk with a large visual sensitivity measure wk, a given SSE distortion has a higher impact on perceptual quality than for blocks with a small visual sensitivity measure wk.

The overall distortion Dpic for a picture shall be given by the sum of the distortions for the individual blocks.  Hence, for the SSE distortion, we have

| DpicSSE=kDkSSE=x,ys[x,y]-s'[x,y]2. | (3) |
| --- | --- |

For the weighted SSE distortion, we obtain

| DpicwSSE=kDkwSSE=kwk⋅DkSSE=kwkx,y∈Bks[x,y]-s'[x,y]2. | (4) |
| --- | --- |

Note that for wk=1, the weighted SSE distortion measure DpicwSSE is equal to the SSE distortion DpicSSE.  Hence, a simple weighted distortion measure defined according to (2) and (4) can be considered as a generalization of the SSE distortion.

## PSNR, WPSNR, and Bjøntegaard Delta Rates / Delta PSNRs

### WPSNR for a single picture

Given the SSE distortion DpicSSE for a color component of a picture, the corresponding PSNR (as used in video coding standardization) is defined by

| PSNR=10⋅log10W⋅H⋅2552⋅22B-16DpicSSE, | (5) |
| --- | --- |

where W and H specify the number of samples in horizontal and vertical direction, respectively, and B represents the bit depth for the samples of the considered color component.  If we utilize the weighted SSE distortion DpicwSSE instead of the conventional SSE distortion DpicSSE, we obtain a very similar measure

| WPSNR=10⋅log10W⋅H⋅2552⋅22B-16DpicwSSE, | (6) |
| --- | --- |

which represents a block-based, activity weighted variant of the conventional PSNR, hence the abbreviation perceptually weighted PSNR (WPSNR).  If all blocks have weight wk=1, the WPSNR is identical to the PSNR.

### WPSNR for video sequences

It is common practice in standardization that the average PSNR of all video frames is used as quality measure for a video sequence.  Let us consider a particular color component.  With N being the number of frames in a video sequence and PSNR[n] representing the PSNR of the considered color component for the n-th frame, the average PSNR for the video sequence is given by

| PSNRseq=1NnPSNR[n], | (7) |
| --- | --- |

The same concept can be used for the proposed WPSNR measure.  With WPSNR[n] representing the WPSNR of the considered color component for the n-th frame, the average WPSNR for the video sequence is given by

| WPSNRseq=1NnWPSNR[n], | (8) |
| --- | --- |

In addition, we want to present an alternative for calculating an average WPSNR measure.  If we use hierarchical coding structures, the reconstruction quality of the frames of the different hierarchy levels typically contribute in a different way to the overall subjective impression.  The frames at the lowest hierarchy level have typically the largest impact on the overall video quality, while the impact of the frame at the highest hierarchy level is noticeably smaller.  For additionally considering this aspect in an average WPSNR measure, we present the following alternative:

| WPSNRseq=10⋅log10W⋅H⋅2552⋅22B-16⋅nfnnfnDpicwSSE[n], | (9) |
| --- | --- |

where W and H are number of samples in horizontal and vertical direction, B is the bit depth of the samples of the considered color component, DpicwSSE[n] is the weighted SSE distortion measure for the n-th picture as defined in (4), and fn>0 represents a weighting factor for the n-th picture.  In hierarchical coding structures, the weighting factor fn should be larger for low hierarchy levels (e.g., key pictures) and it should be smaller for higher hierarchy levels (e.g., non-reference pictures).

Using the arguments later given in sec. 2.2, the picture weighting factors fn can be chosen based on the QP cascading (or the Lagrange parameter cascading).  With QPn and λn being the QP and the Lagrange parameter that are used for the n-th frame, we suggest to chose

| fn=2- QPn3 (or fn=λn-1). | (10) |
| --- | --- |

A disadvantage of this measure is that it values depend on encoder parameters such as the QP cascading.

### Bjøntegaard delta Rates and delta SNRs

When replacing the PSNR with the WPSNR, the well-known and understood Bjøntegaard delta rates and delta PSNRs (which should then be called delta WPSNRs) can be calculated in exactly the same way as usual.  Since Bjøntegaard delta rates and delta PSNRs are relational measures, they basically keep the same meaning.  In other words, a coding tool that yields a bit rate saving of x% using PSNR can be expected to also yield a bit rate saving of approximately x% using WPSNR (if the encoder control remains the same).

For obtaining PSNR and WPSNR values which have approximately the same range of values (which would make it easier to interpret the WPSNR values), the average of the visual sensitivity measures for a picture, or a set of pictures or video sequences, should be close to 1.  This can be ensured by an appropriate scaling during the calculation of the visual sensitivity measures.  This aspect is described in greater detail in sec. 3.

## Implications on Encoder Control

### Review of Lagrangian Bit Allocation

In the following, we briefly review the concept of Lagrangian bit allocation that is used in our reference encoder implementations.  At this point, we do not make any particular assumptions about the actually used distortion measure.  We only assume that the applied distortion measure is block-additive, i. e., the distortion for a picture is equal to the sum of the distortions for the individual blocks.  This property is valid for both the conventional SSE distortion and the above-introduced weighted SSE distortion.

Let pk be the vector of encoding decisions for the block Bk. The main objective of an encoder control is to determine the block coding parameters pk in a way that the overall distortion Dpic is minimized while the associated rate Rpic does not exceed a given rate budget.  Using the concept of Lagrangian multipliers, we obtain the optimization problem [5][6]

| minpkDpicpk+λ⋅Rpic(pk). | (11) |
| --- | --- |

If we use any block-additive distortion measure, we can write the minimization as

| minpk∀kDk(pk)+λ⋅∀kRkpk, | (12) |
| --- | --- |

where Rkpk represents the rate that is required for transmitting all decisions pk for a block Bk and Dk(pk) is the resulting distortion for the block Bk.

For obtaining a feasible encoding algorithm, we partly ignore the dependencies between blocks (actually, dependencies on previous block are taken into account in calculating the rate and distortion terms, but the impact on future blocks is ignored). Then, the overall optimization problem (12) can be solved approximately by separate optimizations (in coding order) for the individual blocks

| ∀k, minpkDkpk+λ⋅Rk(pk) . | (13) |
| --- | --- |

In case of independent blocks, the solution of the overall optimization problem (11) is obtained when all decisions for the individual blocks are conducted with the same value of the Lagrange multiplier λ.  For non-independent blocks (which we have in a video codec), still the same Lagrange multiplier λ should be used for all block decisions.

### Lagrangian Bit Allocation for Weighted SSE Distortion

If we use the conventional SSE distortion measure Dk=DkSSE, the coding decision for each block can be written as

| minpkDkSSEpk+λpic⋅Rk(pk) . | (14) |
| --- | --- |

One particular advantage of using the SSE distortion in the encoder decision is that (since unitary transforms preserve the l2-norm) the distortion can be directly calculated in the transform domain.  This makes it possible to consider the block distortion in the quantization of transform coefficients in a straightforward way.

If we now use the weighted SSE distortion measure DkwSSE=wk⋅DkSSE, we obtain

| minpk wk⋅DkSSEpk+λ⋅Rk(pk) , | (15) |
| --- | --- |

which is equivalent to

| minpk DkSSEpk+λk⋅Rk(pk) , with λk=λwk . | (16) |
| --- | --- |

Hence, for each individual block Bk, we have the same optimization problem as with the conventional SSE distortion.  Only the Lagrange multiplier λk is modified on a block basis.  We still have the main advantage of using the SSE distortion in an encoder control (straightforward quantization).

Experiment:

For demonstrating the effect of using weighted distortion measures and changing the Lagrangian multiplier on a block basis, we demonstrate a simple experiment.

Given are 5 different quantizers, which should represent 5 independent entities (such as blocks in a picture).  Each quantizer provides 6 operation points (i.e., 6 available rate-distortion points that can be selected by an encoder).  The corresponding rate-distortion curves for SSE distortion are depicted in Figure 1(a) and weighted SSE distortion in Figure 1(b).

Further given are the weighting factors wk={1, 0.3, 1.5, 0.4, 3.1}.  We consider the two overall distortion measures

Dm=kDk              and               Dw=kwk Dk.

Overall rate-distortion plots for the measure Dm and Dw are shown in Figure 1(b) and Figure 1(c), respectively.  The blue points represent all (overall) rate-distortion points that can be achieved by selecting one of the available rate-distortion points for each of the 5 quantizers (which represent different blocks).  There are 65=7776 possibilities.  Now, we consider two variants of Lagrangian optimization (with a given value of λ):

Distortion measure Dm:		minDk+λ⋅Rk,

and

Distortion measure Dw:		minDk+λk⋅Rk,      with    λk=λwk.

For varying the Lagrange multiplier λ, the first optimization (for the distortion measure Dm) yields the rate-distortion points that are marked with red circles; the second optimization (weighted distortion Dw) yields the rate-distortion points that are marked with green circles.

(a)       (b)

(c)        (d)

Figure 1: Rate-distortion curves for 5 quantizers (which should represent independent blocks), each with 6 selectable rate-distortion points for (a) conventional SSE distortion measure; (b) weighted SSE distortion measure;  (c) Obtained rate-distortion points for the conventional SSE distortion measure;  (d) Obtained rate-distortion points for the weighted SSE distortion measure.  The blue points represent the set of all available rate-distortion points; the red circles represent the rate-distortion points obtained by a separate Lagrangian optimization with SSE distortion and a fixed Lagrange parameter λk=λ; the green circles represent the rate-distortion points obtained by a separate Lagrangian optimization with SSE distortion and a weighted Lagrange parameter λk=λ/wk.

As can be observed in Figure 1(c,d), a Lagrangian optimization with the SSE distortion and a constant Lagrange multiplier λk=λ yields rate-distortion points that lie on the convex hull for the conventional distortion measure Dm.  On the contrary, a Lagrangian optimization with the SSE distortion and weighted Lagrange multipliers λk=λ/wk yields rate-distortion points that lie on the convex hull for the weighted distortion measure Dw.

### QP Selection

In general, the quantization parameter QPk for a block Bk can be considered as an element of the block decision parameter vector pk, in which case the QP selection for a block is part of the block encoding process.  However, for speeding up the encoding process, it is common practice to select the quantization parameter QPk for a block Bk at the beginning of the block encoding process.  Then, the block decision process can be conducted given the Lagrange multiplier λk and the pre-selected quantization parameter QPk.

Using high-rate approximations for the SSE distortion and the operational rate-distortion curve of a block, the following relationship between the Lagrange multiplier λk and the associated quantization step size Δk can be derived [7][8]:

| λk∝Δk2. | (17) |
| --- | --- |

This relationship has also been experimentally verified [7][8] and is used in the reference video encoder implementations.  Since we have the approximate relationship

| Δk∝2QPk6 | (18) |
| --- | --- |

in AVC, HEVC, and JEM, by combining (17) and (18), we obtain

| QPk-3log2λk=const. | (19) |
| --- | --- |

If we assume that the overall Lagrange parameter λ for a picture is associated with the picture quantization parameter QP, we obtain the QP assignment rule

| QPk=QP-3log2λλk=QP-3log2wk. | (20) |
| --- | --- |

At this point, it must be noted again that it is preferable to scale the weighting factors wk in a way that the average over a picture, or a set of pictures or video frames, is close to 1.  In that case, the same relationship between the picture/set Lagrange parameter λ and the picture/set QP as for SSE distortion can be employed.

### Summary of the Encoding Algorithm

The encoding algorithm for the weighted SSE distortion measure can be summarized as follows:

- Calculate weighting factors wk for the blocks Bk (such as CTUs) of a video picture;

- Choose a picture QP and a corresponding picture Lagrange parameter λ;

- For each block Bk (in coding order):

  - Set the block quantization parameter QPk and the Lagrange multiplier λk according to

QPk=QP-3log2wk      and      λk=λwk;

  - Choose the coding parameters pk for the block Bk according to

minpkDkSSEQPk,pk+λk⋅RkQPk,pk.

# Perceptually Relevant Block Weighting Factors

In the following, we propose a simple measure for characterizing the visual sensitivity wk of image block Bk at block index k.  This measure can be used to calculate WPSNR values and to adapt the encoder control as described in sec. 2.

It is frequently observed that when using the same quantization parameter for a picture, regions with rather smooth (low-frequency) content, i. e., low visual activity, are perceptually more degraded than regions with strongly varying (high-frequency) content, i. e., high activity.  To reduce this effect, we devise a very simple visual sensitivity measure wk that is based on a local energy measure of high-pass filtered original samples.

Let sx,y, with 0≤x<W and 0≤y<H, represent the original luma samples of a picture, with width W and height H as previously.  We first obtain a high-pass filtered version of the picture’s luma component:

| hx,y=4⋅sx,y-sx-1,y-sx+1,y-sx,y-1-s(x,y+1) | (21) |
| --- | --- |

for 0<x<W-1 and 0<y<H-1.  Then for each block Bk, a local activity ak is calculated according to

| ak=max1; 1¦Bk*¦x,y∈Bk*h(x,y)2. | (22) |
| --- | --- |

At this, Bk* represents the considered block Bk without the border samples (with  x=0, y=0, x=W-1, or y=H-1) for which we do not have high-pass samples h(x,y).  The term |Bk*| specifies the number of samples in the block Bk*.  The clipping max⁡(1;..) is required for avoiding divisions by very small values.

In order to obtain visual sensitivity values wk with an average equal to 1, we weight the reciprocals of the local activities ak with the picture’s arithmetic mean given by

| apic=1Kpick∈Kpicak , | (23) |
| --- | --- |

where Kpic specifies the number of blocks {Bk} in the considered picture. This effectively shifts the WPSNR value for each picture near the corresponding PSNR value for the same picture.  Alternatively, apic can be computed as the “global” arithmetic mean of all ak across a large set of pictures (or, in the case at hand, video sequences).  We found that, averaging across, for example, all UHD and HD sequences contained in JVET’s call for evidence [12], apic can be closely approximated as follows:

| apic=1Ksetk∈Ksetak≈2∙2B, | (24) |
| --- | --- |

where B is the bit depth for the samples of the picture’s luma component as in sec. 2.1 and Kset specifies the total number of considered blocks.  It is important to note that, when computing average Bjøntegaard delta WPSNRs between two codecs across a set of video sequences, it does not matter whether the WPSNR values are computed using (23) or (24), i. e., the results will be the same.  Furthermore, apic in (24) can be chosen such that even the average absolute WPSNR values across said set of sequences will be the same as those obtained using (23).

Note that, in principle, one may also determine the geometric mean of all ak in (23) or (24), but to simplify software implementations, we chose the arithmetic mean. Finally, the visual sensitivity weight wk for each block Bk at index k is calculated according to

| wk=apicakβ, | (25) |
| --- | --- |

where β controls the impact of deviating block sensitivity values.  Note that for β=0, we obtain wk=1, in which case the WPSNR reduces to the conventional PSNR and the encoder control will utilize fixed QP settings inside a picture.  Based on our visual experiments described in sec. 4, we chose

| β=23 . | (26) |
| --- | --- |

For various test sequences, a QP adaptation using this simple visual sensitivity measure wk yields noticeable improvements in subjective quality compared to fixed QP settings, particularly in low-rate coding scenarios.

It should be noted that even though it appears reasonable to utilize the same block sizes for determining the visual sensitivity values wk in an encoder and during WPSNR calculation, it is also possible to use picture dependent block sizes.  In an encoder, the blocks Bk represent CTUs or DQP groups, but during WPSNR measurement, it is also possible to use smaller block sizes Bk, which could be reasonable for small pictures. In fact, in one of our implementations, the block size depends on the product of the picture width W and height H in relation to the HD luma-component picture dimensions of 1920×1080 samples and is given by

| Bk=WBk∙HBk, WBk=HBk=minBmax;4∙round16∙W∙H1920∙1080 , | (27) |
| --- | --- |

where upper limit Bmax equals 64 for 4:2:0 downsampled chroma components and 128 otherwise. Note that Bk is an integer multiple of 4×4 to simplify SIMD software implementations, as noted in sec. 1.  Moreover, for HD images, we obtain Bk=64×64.  Finally, for very small images with fewer than 2025 luma samples, Bk=0 and a block-wise weighting can be considered meaningless.  Hence, in such cases, we recommend resorting to traditional “unweighted” PSNR measurements.

# Experimental Results

## Subjective Assessment of Visual Sensitivity Derived QP Adaptation

The performance of a CTU-wise perceptual QP and lambda adaptation according to sec. 2.2, reflecting the visual sensitivity measure wk of sec. 3, was evaluated using both informal and formal subjective tests.  The informal studies intended to identify a suitable value for β as in (26) using still images as well as to verify the subjective quality gains on video material. The formal experiments comprised A/B tests of HEVC coded still images at two operating points.  More specifically, the first frames (I-slices) of 11 high definition (HD, 1920×1080) and ultra-high definition (UHD, 2560×1600) 8-bit YUV video sequences utilized during the HEVC standardization [16][17][18] (classes A and B) were coded and decoded with and without an early variant of the visual QP adaptation (QPA). The global slice QP was set to 32 and 37 for good overall quality.

The subjective evaluation was carried out using a formal visual comparison test following the methodology of ITU-R Rec. BT.500 [19].  Specifically, a degradation category rating (DCR) experiment with a sequence-wise simultaneous ten-second presentation of the uncoded reference, on the left, and the coded version, on the right, assessed on a nine-grade numerical degradation scale according to Annex C and Appendix V, respectively, of ITU-T Rec. P.910 [20] was performed.  Each uncoded/coded image pair was, if necessary, cropped to HD resolution and shown side-by-side horizontally on top of a medium-gray background on a color-calibrated 65-inch LG E6 OLED television.  An uncoded/uncoded “hidden reference” pair was shown for each sequence at random points during the test for verification purposes. Fourteen observers experienced in detecting traditional image and video coding artifacts, aged between 24 and 35 (incl. two females), took part in this study.  The voting period after each ten-second presentation of stimulus pairs was not limited in duration in order to allow the participants to take voluntary short breaks.  The viewing distance for all 14 subjects was limited to the range 3H–3.5H, with H denoting the stimulus height (0.4m in this case, i. e., half the height of the 3840×2160 panel). Very low-luminance room lighting was used while the test was running.

The mean opinion score (MOS) and associated 95% confidence interval (computed with the assumption of a student-t distribution) for each codec configuration (QPA, no QPA) is illustrated in Figure 2.  Note that all depicted values were determined using data from only 12 out of the 14 observers since the scores of two subjects had to be post-screened (one due to an excessively low MOS < 6 for the hidden reference and one due to an excessively high MOS > 7.5 when averaged across all stimuli).  For these 12 observers, the hidden reference reaches a MOS of 8.22 and a confidence interval of ±0.18, indicating stable and accurate grading (a score of 8 is considered the threshold of artifact perception [20]).  Moreover, the MOS values for QP 32 lie between grade 7 (perceptible, but not annoying) and grade 5 (slightly annoying), which implies sufficient overall image reconstruction quality after coding.  Finally, a subjective quality increase can be attributed to the QPA design at the cost of an only minor increase in the average (per-image and overall) coding bit-rate.

Figure 3 illustrates the effect of the QPA algorithm on the input image for which the largest MOS gain of all images under test was observed (BQTerrace, target QP 32, first frame, top-right crop for better visibility).

Figure 2:  Results of the formal subjective QPA evaluation on HEVC coded still images; QP 37 (left points), QP 32 (right points).

Figure 3:  Visual effect of sensitivity based perceptual QP adaptation. HEVC without QPA (top), with QPA (bottom).

## Comparison of WPSNR with PSNR and Other Measures

The performance of the WPSNR algorithm was compared with other well-known psychovisually motivated measures. Specifically, we compared the overall Spearman rank-order correlation between several measurement algorithms and the MOS values collected through formal subjective tests on the JPEG and JPEG 2000 distorted still-image sets of the LIVE database [13].  Table 1 presents the rank-order correlation coefficients computed for each pair of subjective MOS value set (the ground truth) and objective measure (the algorithm under test).  Regarding the WPSNR, two conclusions can be drawn.  First, as expected, the WPSNR version using only picture-wise ak averaging to obtain apic according to (23) yields significantly lower correlation values than the WPSNR version where apic is determined as a “global” across-picture average as in (24). Second, when using WPSNR and (24), the results approach the rank-order correlations attained by the other state-of-the-art measures assessed and are significantly better than the correlations obtained for the PSNR.

In document [14], the results of comparative benchmarks of algorithmic complexity were presented, showing that the majority of the psychovisual measures is at least an order of magnitude more complex than the PSNR method.  Our own evaluations indicate that the WPSNR algorithm is a bit less complex than the SSIM method. This makes the WPSNR design the least complex of all alternatives to the PSNR reference listed in Table 1.

It must be noted that the evaluation of the WPSNR measure in Table 1 only serves informative purposes. As noted earlier, it should be obvious that better psychovisually motivated measures can easily be derived. However, the promising performance of the WPSNR on this MOS database particularly in comparison with PSNR indicates that our approach towards a block-based QP selection and distortion measure is reasonable.

Table 1:  Spearman rank order correlations between subjective and objective data on LIVE database [13].
               * data taken from [14], other data computed on luma component in YCbCr domain. See also [15].

| Distortion Type | PSNR* | SSIM* | MSSSIM* | FSIM* | VSI* | HaarPSI* | WPSNR with (23) | WPSNR with (24) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JPEG 2000 JPEG | 0.8954 0.8809 | 0.9614 0.9764 | 0.9627 0.9815 | 0.9724 0.9840 | 0.9604 0.9761 | 0.9684 0.9832 | 0.8746 0.9005 | 0.9562 0.9685 |

## QP Adaptation and WPSNR

In a next experiment, we want to demonstrate the effect of a perceptually motivated QP adaptation on both the conventional PSNR and the proposed WPSNR measure(s).

We used three base encoder configurations:

- HEVC Main 10

- QTBT (without any additional tools)

- QTBT+Tools (QTBT with most JEM coding tools enabled)

In the QTBT configuration, the HEVC block partitioning is replaced with the QTBT concept; all other aspects are the same as in HEVC (i.e., no additional JEM coding tools are enabled). For the QTBT+Tools configuration, most JEM coding tools are enabled in addition to the QTBT block partitioning concept.

For each of the base encoder configurations, we started two encodings in two configurations:

- Fixed QP configuration (as specified in common test conditions);

- QP adaptation inside pictures according to (20) and (25).

Both configurations differ only in the QP selection on a CTU basis, all other encoding parameters are the same.  For all configurations, the quality was measured using

- PSNR (averaged over all pictures according to (7));

- WPSNR (averaged over all pictures according to (8));

- Frame-weighted WPSNR (calculated according to (9)).

Based on these quality measures, the Bjøntegaard delta rates between the fixed QP setting and the adaptive QP assignment were calculated. All experiments were performed for the common test conditions (Intra slice QPs 22, 27, 32, 37) using Fraunhofer HHI’s custom version of a JEM/HEVC encoder.

Table 2, Table 3, and Table 4 list the component-wise BD-rates for the adaptive QP selection versus the same encoder with fixed QP assignment for the different encoding scenarios. In all cases, a positive percentage indicates a loss (rate-distortion increase) and a negative percentage reveals a gain. Three observations can be made from the data:

- The PSNR measure, as noted earlier, strongly favors a locally invariant, input invariant QP (fixed QP) over any perceptual optimizations (QPA enabled).  The average loss in PSNR due to the QP adaptation inside pictures is approximately 10-12%.

- For both variants of the WPSNR (frame average and frame weighted WPSNR), the QP adaptation yields gains of approximately 6-7% for intra and 3-5% for random access.

- The WPSNR gains due to the proposed QP adaptation are virtually independent of the underlying codec (HEVC, QTBT).

Table 2:  Comparison between fixed QP and the proposed QP adaptation for the HEVC Main 10 configuration and different quality measures.

Table 3:  Comparison between fixed QP and the proposed QP adaptation for the QTBT configuration and different quality measures.

Table 4:  Comparison between fixed QP and the proposed QP adaptation for the QTBT+Tools configuration and different quality measures.

## Gains of Coding Tools in WPSNR

In a last experiment, we want to demonstrate that the gains of individual coding tools are similarly reflected in both of the following settings:

- Common setting:  PSNR quality measure and fixed QP assignment inside pictures;

- Proposed setting:  WPSNR quality measure and adaptive QP assignment inside pictures.

As examples we tested the following codecs according to the common test conditions:

- HEVC Main 10;

- QTBT (replaces HEVC block partitioning; no additional tools enabled);

- QTBT with additionally enabling 65 angular intra modes (Intra65);

- QTBT with additionally enabling sub-PUs (subPU);

- QTBT with most JEM coding tools enabled (QTBT+Tools).

The measured gains are summarized in the following tables:

- Table 5:  QTBT vs HEVC;

- Table 6:  QTBT+Intra65 vs QTBT (intra only);

- Table 7:  QTBT+SubPU vs QTBT (random access only);

- Table 8:  QTBT+Tools vs HEVC.

The data in the tables show that the measured coding gains for the individually tested coding tools are very similar with both settings (fixedQP+PSNR and QPA+WPSNR).

Table 5:  Comparison of coding gains of QTBT vs HEVC for the following configurations: (left) Fixed QP assignment with PSNR measure; (center) Adaptive QP assignment (QPA) with WPSNR measure; and (right) Adaptive QP assignment (QPA) with frame-weighted WPSNR measure.

Table 6:  Comparison of coding gains of QTBT with 65 intra modes enabled vs QTBT only for the following configurations: (left) Fixed QP assignment with PSNR measure; (center) Adaptive QP assignment (QPA) with WPSNR measure; and (right) Adaptive QP assignment (QPA) with frame-weighted WPSNR measure.

Table 7:  Comparison of coding gains of QTBT with subPUs enabled vs QTBT only for the following configurations: (left) Fixed QP assignment with PSNR measure; (center) Adaptive QP assignment (QPA) with WPSNR measure; and (right) Adaptive QP assignment (QPA) with frame-weighted WPSNR measure.

Table 8:  Comparison of coding gains of QTBT+Tools vs HEVC for the following configurations: (left) Fixed QP assignment with PSNR measure; (center) Adaptive QP assignment (QPA) with WPSNR measure; and (right) Adaptive QP assignment (QPA) with frame-weighted WPSNR measure.

In Figure 4 and Figure 5, we additionally present a visual comparison of the coding efficiency of HEVC, QTBT, and QTBT+Tools for two example sequences. The top diagrams show RD curves for a fixed QP assignment and the PSNR quality measure; the bottom diagrams show RD curves for the adaptive QP assignment (QPA) and the associated WPSNR quality measure

Figure 4:  Coding efficiency comparison for Kimono (random access):  (top) Fixed QP encoding and PSNR quality measure; (bottom)  Adaptive QP encoding and WPSNR quality measure.

Figure 5:  Coding efficiency comparison for BasketballDrive (random access):  (top) Fixed QP encoding and PSNR quality measure; (bottom)  Adaptive QP encoding and WPSNR quality measure.

# Conclusions

We introduced a generalization of the PSNR measure, called perceptually weighted PSNR (WPSNR), which was shown to allow perceptually much more relevant assessments than PSNR.  We further presented a block-wise QP adaptation (QPA) method implementing the principles of the WPSNR measure in order to achieve visual coding quality improvements.  It was shown that the WPSNR measure correlates much better with subjective MOS values than PSNR.  Moreover, significant visual improvements were observed in a formal subjective test when using the correspondingly implemented block-wise QP adaptation (QPA).  Finally, the results of numerous Bjøntegaard-delta (BD) evaluations conducted on HEVC/JEM codec configurations show that we can achieve both:

- A perceptually motivated QP adaptation results not only in improvements of subjective quality, but also in BD-rate gains when using the WPSNR measure (while the PSNR measure indicates losses);

- For the evaluation of individual coding tools, using the WPSNR with adaptive QP selection provides very similar gains in BD-rate as the conventional PSNR with a fixed QP selection.

Given this evidence, we suggest the adoption of the WPSNR as an additional objective measure in future work and literature created within the JVET.  For the upcoming standardization towards a video codec with compression capability exceeding that of HEVC [12], we particularly recommend test conditions in which a block-adaptive QP assignment an associated WPSNR-based Bjøntegaard delta-rate analysis is used.

# References

K. Sühring and X. Li, “JVET common test conditions and software reference configurations,” JVET, doc. JVET-G1010, July 2017.

A. Segall, E. François, and D. Rusanovsky, “JVET common test conditions and evaluation procedures for HDR/WCG video,” JVET, doc. JVET-G1020, July 2017.

C.-W. Tang, C.-H. Chen, Y.-H. Yu, and C.-J. Tsai, “Visual sensitivity guided bit allocation for video coding,” IEEE Trans. Multimedia, vol. 8, no. 1, pp. 11–18, February 2006.

Z. Chen and C. Guillemot, “Perceptually-friendly H.264/AVC video coding based on foveated just-noticeable-distortion model,” IEEE Trans. Circuits and Systems for Video Technology, vol. 20, no. 6, pp. 806–819, June 2010.

H. Everett III, “Generalized Lagrange multiplier method for solving problems of optimum allocation of resources,” Operations Research, vol. 11, no. 3, pp. 399–417, Jun. 1963.

Y. Shoham and A. Gersho, “Efficient Bit Allocation for an Arbitrary Set of Quantizers,” IEEE Trans. Acoustics, Speech, and Signal Process., vol. 36, no. 9, pp. 1445–1453, Sep. 1988.

G. J. Sullivan and T. Wiegand, “Rate-Distortion Optimization for Video Compression,” IEEE Signal Processing Magazine, vol. 15, no. 6, pp. 74-90, November 1998.

T. Wiegand, B. Girod, “Lagrange multiplier selection in hybrid video coder control,” in Proc. IEEE Intl. Conf. on Image Processing, Oct. 2001.

Z. Wang, E. P. Simoncelli, and A. C. Bovik, “Multi-scale structural similarity for image quality assessment,” in Proc. IEEE 37th Asilomar Conf. on Signals, Systems, and Computers, Nov. 2003.

H. R. Sheikh and A. C. Bovik, “Image Information and Visual Quality,” IEEE Trans. Image Process., vol. 15, no. 2, pp. 430–444, Feb. 2006.

L. Zhang, L. Zhang, X. Mou, and D. Zhang, “FSIM: A Feature Similarity Index for Image Quality Assessment,” IEEE Trans. Image Process., vol. 20, no. 8, pp. 2378–2386, Aug. 2011.

V. Baroncini et al., “Results of the Joint Call for Evidence on Video Compression with Capability beyond HEVC,” JVET, doc. G1004-v2 (ISO/IEC JTC1/SC29/WG11 N17054), Jul. 2017.

H. R. Sheikh, Z. Wang, L. Cormack, and A. C. Bovik. “LIVE Image Quality Assessment Database,” release 2, 2005. http://live.ece.utexas.edu/research/Quality/subjective.htm

R. Reisenhofer, S. Bosse, G. Kutyniok and T. Wiegand, “A Haar Wavelet-Based Perceptual Similarity Index for Image Quality Assessment,” 2017. http://arxiv.org/abs/1607.06140, http://www.haarpsi.org

T. K. Tan, R. Weerakkody, M. Mrak, N. Ramzan, V. Baroncini, J.-R. Ohm, and G. J. Sullivan, “Video Quality Evaluation Methodology and Verification Testing of HEVC Compression Performance,” IEEE Trans. Circuits and Systems for Video Technology, vol. 26, no. 1, pp. 76–90, Jan. 2016.

J. Lainema, F. Bossen, W.-J. Han, J. Min, and K. Ugur, “Intra Coding of the HEVC Standard,” IEEE Trans. Circuits Systems for Video Technology, vol. 22, no. 12, pp. 1792–1801, 2012.

V. Sze, M. Budagavi, and G. J. Sullivan, High Efficiency Video Coding (HEVC) – Algorithms and Architectures, Cham, Switzerland: Springer International Publishing, 2014.

F. Bossen, “Common test conditions and software reference configurations,” JCTVC-L1100/m28412, Geneva, 2013.

ITU-R, Recommendation BT.500, “Methodology for the subjective assessment of the quality of television pictures,” Geneva, Switzerland, Jan. 2012.

ITU-T, Recommendation P.910, “Subjective video quality assessment methods for multimedia applications,” Geneva, Switzerland, Apr. 2008.

G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

S. Bosse, M. Siekmann, W. Samek, and T. Wiegand, “A Perceptually Relevant Shearlet-Based Adaptation of the PSNR,” in Proc. IEEE Intl. Conf. on Image Processing, Sep. 2017.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).