| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0031 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by University of Bristol |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Bull, Fan Zhang, and Mariana Afonso The department of Electrical and Electronic Engineering University of Bristol, UK | Tel: Email: | +44 (0) 117 954 5195 {dave.bull,fan.zhang, mariana.afonso}@bristol.ac.uk |
| Source: | University of Bristol |  |  |

_____________________________

# Abstract

This contribution describes University of Bristol’s response to the Joint Call for Proposals on Video Compression with Capability beyond HEVC (CfP) issued by ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 (MPEG). In this proposal, a resolution adaptation approach (ViSTRA), based on the JVET JEM 7.0 software, is proposed as a coding tool for future standard development.  This adaptively determines the optimal spatial resolution and bit depth for input videos during encoding, and reconstructs full video resolutions at the decoder using a deep CNN-based up-sampling method. This approach has been tested on the SDR test dataset in the CfP, and is found to achieve average bitrate savings (BD-rate) of -4.54% (-8.69% for SDR-A and -0.39% for SDR-B) and -0.52% for Constraint Set 1 and Constraint Set 2 respectively against the JEM anchor.

# Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Encoder algorithm description	1

2.1	Quantisation-resolution Optimisation	1

2.1.1	Spatial resolution optimisation	2

2.1.2	Bit depth optimisation	4

2.2	Resolution down-sampling	4

2.2.1	Spatial down-sampling	5

2.2.2	Bit-depth down-sampling	5

2.3	Low resolution video encoding	5

2.3.1	Base QP adjustment	5

2.3.2	Interim solution for resolution variations	5

2.4	Side information	6

3	Decoder algorithm description	6

3.1	Bitstream structure	6

3.2	Resolution up-sampling	7

3.2.1	Up-scaling	8

3.2.2	Normalization	9

3.2.3	Block splitting	9

3.2.4	Convolutional Neural Network	9

4	Compression performance	11

4.1	SDR: Constraint set 1 configuration relative to HM anchor	11

4.1.1	Class SDR-A (RD-curves)	11

4.1.2	Class SDR-B (RD-curves)	12

4.1.3	Overall (BD-rate summary tables)	13

4.2	SDR: Constraint set 2 configuration relative to HM anchor	14

4.2.1	Class SDR-B	14

4.2.2	Overall	15

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	15

4.3.1	Class SDR-A	15

4.3.2	Class SDR-B	16

4.3.3	Overall	17

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	18

4.4.1	Class SDR-B	18

4.4.2	Overall	19

5	Decoder complexity analysis	19

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16 and JEM 7.0	19

5.1.1	Constraint Set 1	19

5.1.2	Constraint Set 2 – Low Delay	20

5.2	Description of computing platform used to determine decoding times reported in section 5.1	21

5.3	Memory usage of decoder	21

5.4	Complexity characteristics of additional decoder tools	21

5.5	Degree of capability for decoder parallel processing	21

6	Encoder complexity analysis	22

6.1	Encoding time and measurement methodology	22

6.1.1	Constraint Set 1	22

6.1.2	Constraint Set 2 – Low Delay	23

6.2	Description of computing platform used to determine encoding times reported in section 6.1	23

6.3	Memory usage of encoder	23

6.4	Complexity characteristics of additional encoder tools	24

6.5	Degree of capability for encoder parallel processing	24

7	Algorithmic characteristics	24

7.1	Random access characteristics	24

7.2	Delay characteristics	24

7.3	Compression performance discussion	24

8	Software implementation description	24

9	Closing remarks	25

10	Patent rights declaration(s)	25

11	References	25

12	Annex: Questionnaire	27

12.1	Codec level Q&A	27

12.2	Tool level Q&A	28

12.2.1	Category 1. Partitioning structure	28

12.2.2	Category 2. Entropy coding	28

12.2.3	Category 3. Control data derivation process	28

12.2.4	Category 4. Decoder-side motion refinement	29

12.2.5	Category 5. Intra prediction	29

12.2.6	Category 6. Inter prediction	30

12.2.7	Category 7. Quantization	30

12.2.8	Category 8. Transforms	30

12.2.9	Category 9. In-loop filtering	31

12.2.10	Category 10. “tool-off” and “tool-on” tests	31

# Introduction

Due to the significantly increased demands on bandwidth for more immersive video content (high spatio-temporal resolutions, high dynamic range and 360 degree), significant challenges remain for video coding. This document describes a resolution adaptation approach (ViSTRA) which was submitted as an SDR video coding technology proposal in response to the CfP. This approach dynamically determines the optimal spatial resolution and bit depth at the encoder, and reconstructs each frame to its original resolution (spatial resolution and bit depth) at the decoder using a deep CNN-based super resolution method.

The JEM-VISTRA codec is based on the JVET JEM software version 7.0 with the addition of a resolution adaptation tool (ViSTRA). Unless otherwise noted, the algorithm details of the proposal are identical to the tools found in the original JEM software.

# Encoder algorithm description

Figure 1 shows the proposed JEM-ViSTRA encoder, in which the optimal spatial resolution and bit depth are firstly determined by the Quantisation-Resolution Optimisation (QRO) module according to initial QP values used in compression and video content. The original video frames may then down-sampled and compressed using the host encoder (JEM). This decision is made for each frame or a number of frames depending on the GOP configuration used - one frame for Low Delay mode (CS2) and 16 frames for Random Access mode (CS1). This ensure that the temporal delay remains the same as that for host/anchor encoder.

Figure 1: Diagrammatic illustration of the JEM-ViSTRA encoder.

## Quantisation-resolution Optimisation

The figure below shows the Quantisation-resolution Optimisation module in the JEM-ViSTRA encoder, in which optimum spatial resolution and bit depth are determined according to the original resolution video frame(s) and the initial quantisation parameters. The determination algorithms are described in the following subsections.

Figure 2: Diagrammic illustration of the Quantisation-resolution optimisation module.

### Spatial resolution optimisation

The spatial resolution optimisation module is responsible for deciding whether it would be beneficial to apply spatial resampling prior to encoding. This is determined by a machine learning approach using low-level spatio-temporal features from the uncompressed video frames. In the current version, these features are computed at frame level, on the Y (luma) channel only and are summarised below.

Spatial feature - SRQM

SRQM [1] is computed per frame as a spatial feature. The first step is to down-sample and subsequently up-sample the uncompressed high-resolution frame by two (both vertically and horizontally) using a Lanczos3 filter, as described in detail in Sections 2.2.1 and 3.2.1.1.

Then, a 2-D Haar Discrete Wavelet Transform (DWT) [2] is applied to both the original resolution (HR) and resampled video frames (LR), decomposing the image into 4 subbands, DC (LL), horizontal (LH), vertical (HL) and diagonal (HH) high frequency:

BLLHR, BHLHR, BLHHR, BHHHR=DWT(YHR)

BLLLR, BHLLR, BLHLR, BHHLR=DWT(YLR)

Then, the absolute difference between the corresponding high frequency subbands of HR and LR frames is calculated and averaged as follows:

∆B= 13BHLHR- BHLLR+ BLHHR- BLHLR+ BHHHR- BHHLR

After that, the average subband difference (ASD) is computed for all 8×8 non-overlapping blocks:

ASDi= ∀x,y∈Blocki∆B(x,y)64

Here i is the block index.

The frame level distortion, DSRQM is then computed as the maximum value of ASD between all blocks:

DSRQM=max{ ASD1, ASD2,…,ASDK}

Here, K is the number of 8×8 non-overlapping blocks in a frame. Finally, frame level SRQM is then calculated using the equation below.

SRQM=20 ∙log102B-1DSRQM

Here B is the bit depth in the original frame.

Temporal Feature 1 and 2 – Mean peak and Kurtosis of Normalized Cross Correlation (NCC):

NCC [3] calculates the normalized cross correlation between successive frames It and It+1 using a sliding template window T of size h2×w2 size from the reference frame It+1, where h and w correspond to the height and the width of the original frames.

NCCu,v=x,yItx,y-Itu,v[Tx-u,y-v-T]x,y[Itx,y-Itu,v)2∙x,yTx-u,y-u-T20.5

Here u and v are all the positions where the sliding window was calculated, Itu,v corresponds to the average pixel value at the sliding window position and T corresponds to the average pixel value of the template. NCCu,v contains values between 0 and 1. Two statistics from NCC are used here - mean peak and kurtosis. The mean peak is the computed as the difference between the maximum NCC and the average NCC around a border of K pixels around the location of the maximum (peak). A fixed value of 50 was used for parameter K. The detailed calculation of NCC can be found in [4].

Temporal Feature 3 - Temporal Coherence (TC):

TC [3] is computed using the magnitude-squared coherence which estimates how well the current frame corresponds to the subsequent frame at each temporal frequency f, calculated using the Power Spectral Density (PSD) of the Fast Fourier Transform (FFT). This feature takes values between 0 and 1. It is calculated using the following equation:

TC(f)=PIt It+1(f)2PItIt(f)PIt+1It+1(f)

Where PIt It+1 is the cross-spectral density between the two frames, PItIt is the auto-spectral density of the current frame and PIt+1It+1 is the auto-spectral density of the subsequent frame. The statistic of this feature used in the spatial resolution optimisation module is the skewness of the TC. The detailed calculation of TC can be found in [5].

For both NCC and TC, in the special case of the last frame in a sequence, instead of using the subsequent frame as reference, the previous frame is used.

Linear SVM regression

The above features were computed for a training dataset, consisting of 50 selected sequences from various UHD and HD databases (not including the test sequences in the CfP) and, using a similar procedure to that described [6], a linear Support Vector Machine (SVM) regressor was trained offline to predict a spatial adaptation QP threshold for the training sequences, based on the trained parameters: scale (s), beta (β) and bias (b). Hence, by computing the same set of features for the input video frame, we are able to determine the predicted QP threshold for each frame, QPt,SR, following the equation below:

QPt,SR=zstandardizeds ×β+b

Here zstandardized is the standardised version of the combined four features:

z=[SRQM, NCCmeanpeak,NCCkurtosis,TCskewness]

zstandardized= z-μσ

Here μ and σ are constant vectors which are pre-defined in the training, and their values are given as follows:

μ=[30.5228,0.3670,3.1016,0.3688]σ=[5.0198,0.1404,1.2520,0.8289]

The values of other trained parameters are:

s = 2.6879

β=[-5.7625, 4.0495, -1.1725, -3.8476]T

b= 36.7192

Decision making

The initial base QP value (QPi) is then compared with the average QPt,SR value for a number of frames - one GOP for Random Access mode (CS1) or one frame for Low Delay mode (CS2). If QPi is greater than QPt,SR, the spatial resolution of the original frame(s) will be reduced by 2 (both vertically and horizontally). Otherwise, encoding is performed using the original resolution. This decision applies to all frames assessed (one GOP for CS1, and one frame for CS2).

### Bit depth optimisation

The optimal bit depth is determined according the initial base QP value (QPi) and the distortion introduced due to bit depth down-sampling (DBD). The latter is calculated by taking the mean absolute error between the original frame and its bit depth re-sampled frame.

The bit depth re-sampling frame is obtained by firstly bitwise right-shifting every pixel value of the original frame by 1 bit, and then shifting back to their original bit depth. For each frame, another QP threshold (QPt,BD) is defined as a staircase function of the re-sampling distortion DBD. Here the RD-characteristics of the sequences from the BVI-HomTex database [7] (these sequences are different from the test sequences included in the CfP) were observed for the model development.

QPt,BD=   45, when 1.0≥DBD≥0.52539, when 0.4<DBD<0.52533, when 0.0≤DBD≤0.400

The initial QP value (QPi) is then compared with the average QPt,BD value for a number of frames - one GOP for Random Access mode (CS1) or one frame for Low Delay mode (CS2). If QPi is greater then QPt,BD, the bit depth of the original frame(s) will be reduced by 1 bit. Otherwise, the original bit depth remains the same. Again, this decision applies to all frames assessed (one GOP for CS1, and one frame for CS2).

## Resolution down-sampling

In this proposal, down-sampling is performed for spatial resolution and bit-depth using a single ratio of 0.5 for spatial and 1 bit for bit-depth down-sampling. In the future algorithm extensions, multiple sampling ratios may be adopted to achieve higher compression efficiency. The next sections explain how both down-sampling operations are performed.

### Spatial down-sampling

Spatial resolution down-sampling using a ratio of 0.5 is performed by applying an anti-aliasing modified separable Lanczos3 resampling filter [8], as represented by the following equation:

Lanczos3downk= 0.5×Lanczos3(0.5×k)

The equation for Lanczos3 kernel can be found in Section 3.2.1.1. The kernel width for down-sampling is 12 which means the closest 12 pixels are used to interpolate each pixel value in the down-sampled image. The value of k is:

k=[-5.5, -4.5, -3.5, -2.5, -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5]

Then, by calculating the down-sampling kernel, the normalized filter weights (64-bit precision) correspond to (rounded to 3 decimal places):

[0.004, 0.015, -0.034, -0.067, 0.136, 0.446, 0.446, 0.136, -0.067, -0.034, 0.015, 0.004]

### Bit-depth down-sampling

For bit-depth down-sampling, a bitwise right shift operation by a factor of 1 bit is applied:

Yupx,y= Yinput(x,y)≫1

## Low resolution video encoding

### Base QP adjustment

When a lower spatial resolution or bit depth is selected as the optimal resolutions for encoding, identical configurations as for JEM are used during encoding except base QP, which is adjusted in order to achieve similar bit rate as encoding the original video with the initial QP. By default, the new base QP is calculated based on the table below.

| Initial QP (QPi) | Spatial Resolution Down-sampling | Bit-Depth Down-sampling | New Base QP (QPlow) |
| --- | --- | --- | --- |
|  | Yes | Yes | QPi-12 |
|  | Yes | No | QPi-6 |
|  | No | Yes | QPi-6 |
|  | No | No | QPi |

In order to meet the target bit rates for the test sequences in the CfP, the new base QP values were iteratively adjusted around their default values until the output bitrates became sufficiently close to the target bitrates.

### Interim solution for resolution variations

It should be noted that, due to the possibility of significant content variations within video content (although not very common in the SRD test set), our Quantisation-Resolution Optimisation module may lead to input resolution variations for video encoder. This should not cause any problems if resolution changing is allowed at this level in the encoding loop. Since the development of this approach is still at an early stage and resolution changing is not fully enabled for ViSTRA, video sequence segmentation is currently executed when different resolutions have to be used during encoding. This may affect the overall coding efficiency. When a video sequence has to be segmented into more than one segments, their bitstreams are generated sequentially by the host encoder JEM.

In order to avoid frequent sequence segmentation, the following criteria were employed to decide if a video sequence should be segmented.

- Each video segment should be longer than 1 second.

- The difference between the average QP thresholds (for both spatial resolution and bit depth) for this GOP or frame (depending on the encoding mode) and that for the previous GOP or frame should be greater than 5.

It should be noted that this segmentation approach is an interim solution, which was used in the current version of the JEM-ViSTRA encoder for generating the submitted results. It will be removed when resolution changing is fully enabled for ViSTRA in the JEM encoder.

## Side information

JEM-ViSTRA requires flag bits to be incorporated in the bitstream to indicate resolution change. Ideally, these flag bits would be inserted just after the header of the first frame in a GOP. As an interim solution employed for resolution variation in the current version of JEM-ViSTRA, the flag bits were inserted after the header of the bitstream for each video segment.

The flag bits include one byte for spatial resolution resampling ratio, and one byte for the bit depth resampling ratio. Currently, they are only two possible values for each flag, 1 or 2, indicating without or with resolution (spatial resolution and bit depth) resampling respectively. Other possible values are reserved for future use.

# Decoder algorithm description

The proposed decoder is illustrated in Figure 3. In the decoder, the flag bits and JEM bitstream are firstly extracted from the input bitstream. Resolution down-sampled frame(s) are then reconstructed by the Host Decoder (JEM 7.0), and up-sampled to their original resolutions (spatial resolution and bit depth) using a deep CNN-based super resolution method.

Figure 3: Diagrammic illustration of the proposed decoder.

## Bitstream structure

The bitstream structure remains almost the same as for JEM, except for the inclusion of the flag bits used to indicate resolution change. The current version of JEM-ViSTRA insert two additional bytes after the bitstream header – the first one indicates the spatial resolution re-sampling ratio and the second indicates the bit depth re-sampling ratio.

It should be noted that this is the bitstream structure for the current version of the encoder, which has not been finalised. In future versions, resolution flag bits may be used for each frame or GOP when adaptive resolution changing is fully enabled for ViSTRA in both JEM encoder/decoder.

See Section 2.4 for further details regarding side information in bitstream.

## Resolution up-sampling

Two types of up-sampling are incorporated in the JEM-ViSTRA decoder: spatial resolution up-sampling and bit-depth up-sampling. The up-sampling ratio is currently constrained to two for spatial resolution adaptation, for example, from 1920×1080 to 3840×2160. For bit depth, the current codec only supports 1 bit up-sampling, e.g. from 9 bit (0-511) to 10 bit (0-1023). In both cases, a deep CNN-based super-resolution model is employed to effect the upsampling in JEM-ViSTRA.

Depending on the flag bits received at the decoder, four possible operations may be performed, as listed in the table below.

| Case | Spatial flag byte | Bit depth flag byte | Spatial up-sampling | Bit-depth up-sampling |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | No | No |
| 1 | 2 | 1 | Yes | No |
| 2 | 1 | 2 | No | Yes |
| 3 | 2 | 2 | Yes | Yes |

For each case where up-sampling is enabled (1, 2 or 3), the same CNN architecture is used but the model parameters vary. In total, there are five model parameters are used in the decoder, which are given as follows:

Case 1: SR.caffemodel

Case 2: BD.caffemodel

Case 3: SRBD_QPset1.caffemodel, SRBD_QPset2.caffemodel, SRBD_QPset3.caffemodel

For Case 3, where both spatial and bit-depth up-sampling is applied, a single CNN model that combines both contributions is used (SRBD). In this case, in order to further improve the reconstructed video quality, one of three different model parameters is selected according to the base QP value that is used to encode the low resolution video frames, QPlow (described in Section 2.3.1.) The following equation describes the use of these three models:

CNN model parameters for Case 3=SRBD_QPset1,  &QPlow<29.5SRBD_QPset2,  &QPlow<34.5 and QPlow≥29.5SRBD_QPset3,  &QPlow≥34.5

Here QPlow is the base QP used for encoding the low resolution video frames.

The overall procedure of the up-sampling operation in Cases 1, 2 and 3 is described in Figure 4. First, the decoded low resolution (spatial and/or bit depth) is up-sampled through simple filtering and/or bit-shifting. The pixel values of the Y channel are then normalized to the range from 0 to 1, and the whole frame is split into 128×128 overlapping blocks. These blocks are processed by the CNN model using model parameters trained, and the results for all blocks are aggregated and then inversely normalised to the original spatial resolution and/or bit depth. Finally, the U and V color channels of the frame, which are generated from the up-sampling operation are concatenated with the Y channel and form the final output frame. Each of these operation is described in the following sections.

Figure 4: Diagram of the overall proposed up-sampling operation at the decoder.

### Up-scaling

#### Spatial up-scaling

For spatial resolution adaptation, up-scaling is performed by applying a separable Lanczos3 resampling filter, which is represented by the following equation [8]:

Lanczos3x=sinπ×x×sinπ×x3π2 ×x23,  &|x|<30                       ,  &|x|≥3

Here x corresponds to the relative position of the nearest pixels of each interpolated pixel. For this kernel, the closest 6 pixels are used to interpolate one pixel value in the interpolated image.

Since only up-sampling ratio of 2 is currently applied in the proposal, the value of x is:

x=[-2.25, -1.25, -0.25, 0.75, 1.75, 2.75],  &for odd pixel positions[-2.75, -1.75, -0.75, 0.25, 1.25, 2.25],  &for even pixel positions

Then, by calculating the Lanczos3 kernel, the normalized filter weights, W, (64-bit precision) correspond to:

W=[0.030,-0.133, 0.893, 0.271, -0.068, 0.007],  &for odd pixel positions[0.007,-0.068, 0.271, 0.893, -0.133, 0.030],  &for even pixel positions

In addition, symmetric padding is applied to handle the borders. This operation is applied in both dimensions of the image, first the height and then the width.

#### Bit-depth up-scaling

For bit-depth up-scaling, a bit shift operation by a factor of 1 bit is applied:

Yupx,y= Yinput(x,y)≪1

In Case 3, the spatial resolution up-scaling is firstly performed, and the bit-depth up-scaling is then applied to the output of the spatial up-scaling.

### Normalization

Normalization is applied to the Y channel only prior to using it as the input to the CNN operation. This is used to facilitate the deployment of the CNN. The input frame Yup(x,y) is normalized according to the following equation:

Yupnorm(x,y) = Yup(x,y)2B-1

Where B corresponds to the original bit-depth of the video sequence. For example, if the original video has a bit-depth of 10 bit, the denominator would be 1023. This will guarantee that the output frame Yupnorm is between 0 and 1.

### Block splitting

In order to reduce the memory consumption required to process large frames using the CNN, each frame is split into 128×128 overlapping blocks. The overlap size is 4 pixels. This is also suitable for potential parallel processing.  The following figure illustrates how to split a frame into blocks:

Figure 5: Illustration of splitting the frame into 128×128 overlapping block.

### Convolutional Neural Network

The architecture of the current network has been adapted from VDSR [9], and is presented in the diagram below. The network contains 20 convolutional layers, each followed by ReLU layers. Each layer contains 64 filters of size 3 × 3. In addition, the final layer performs a summation operation between the input frame and the result of the cascaded convolutional layers. Therefore, instead of predicting the output directly, this architecture predicts the residual between the LR input and the HR output, as shown in the diagram.

Figure 6: CNN architecture based on VDSR.

#### Convolutional Layers

Convolutional Neural Networks (CNN) apply a cascade of convolutions to the input image usually with non-linearity layers or other layer types in between. Each layer i contains Ki filters of size mi ×mi that are convolved with an input of size (Wi-1, Hi-1,Ki-1) and generates an output of size (Wi-mi+1, Hi-mi+1,Ki), where Wi, Hiand Ki are the width, height and depth of the features after layer i.

The architecture used contains 20 cascaded convolutional layers all with 64 filters of size 3 ×3. The input to the CNN are the blocks of size 128 ×128 pixels. The dimensionality of the parameters required for all layers is as follows:

Weights:

- Layer 1: 3 ×3 ×1× 64

- Layers 2 to 19: 3 ×3×64 ×64

- Layer 20: 3 ×3 ×64 ×1

Biases:

- Layers 1 to 19: 64×1

- Layer 20: 1×1

Therefore, in total 664704 floating point weights and 1217 floating point biases are required to perform the CNN operations for this architecture.

#### Zero Padding

If cascaded layers are applied consecutively, the dimensionality of each feature is reduced due to the convolutional operation at the borders. For example, in this case, if the input is size 128 ×128, the output of the first layer will be 126 ×126 ×64. Therefore, to keep the size of the features constant throughout the network (the same as the input block), zero padding is applied. This consists of adding zeros to the border of each convolutional layer output.

#### ReLU layer

The Rectified Linear Unit (ReLU) is an elementwise activation function that has the following equation:

ReLUx=0,  &x<0x,  &x≥0

This layer is applied in between convolutional layers as shown in the diagram of Figure 2. In addition, this layer does not change the feature size.

#### Summation layer

This layer takes two inputs with the same dimension and outputs their sum elementwise. This process does not change the size of the features. This layer performs the addition between the input low-resolution block and the result of the convolutional and ReLU layers (a residual) to obtain the final high-resolution output block.

#### Inverse normalisation

The output of the CNN is between 0 and 1 following the previous normalization operation. In order to return to the original range, the inverse normalisation procedure is applied to Yout in order to obtain the final high resolution Y channel frame YHR as follows, where B corresponds to the original bit-depth of the video sequence:

YHRx,y= Youtx,y×(2B-1)

#### Training

The CNN network was trained using SSE as the loss function, which is equivalent to optimizing for PSNR. A total of 50 sequences from public video databases including Harmonic [10] and Tampere  [11] databases were used to train the network (It should be noted that these sequences do not include the CfP test sequences). Training sequences included both UHD and HD resolutions, with either 10 bit originals or up-sampled 10 bit versions from 8 bit originals.

# Compression performance

As this proposal is based on the JEM 7.0, the coding configuration used is identical to the JEM anchors for the respective constraint set.

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

### Class SDR-B (RD-curves)

### Overall (BD-rate summary tables)

| Sequence | BD-PSNR [dB] | BD-Bitrate [%] |
| --- | --- | --- |
| FoodMarket4 | 1.77 | -36.61 |
| CatRobot1 | 1.56 | -40.32 |
| DaylightRoad2 | 1.12 | -40.34 |
| ParkRunning3 | 1.51 | -45.69 |
| Campfire | 1.48 | -50.77 |
| Average for SDR-A | 1.49 | -42.75 |
| BQTerrace | 0.91 | -30.26 |
| RitualDance | 1.46 | -27.86 |
| MarketPlace | 1.07 | -30.94 |
| BasketballDrive | 1.12 | -31.72 |
| Cactus | 1.44 | -35.68 |
| Average for SDR-B | 1.20 | -31.29 |
| Average | 1.34 | -37.02 |

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

### Overall

| Sequence | BD-PSNR [dB] | BD-Bitrate [%] |
| --- | --- | --- |
| BQTerrace | 0.78 | -26.93 |
| RitualDance | 1.14 | -24.63 |
| MarketPlace | 0.70 | -21.90 |
| BasketballDrive | 0.67 | -26.13 |
| Cactus | 1.13 | -29.12 |
| Average | 0.88 | -25.74 |

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

### Class SDR-B

### Overall

| Sequence | BD-PSNR [dB] | BD-Bitrate [%] |
| --- | --- | --- |
| FoodMarket4 | 0.12 | -2.94 |
| CatRobot1 | 0.02 | -0.69 |
| DaylightRoad2 | 0.00 | 0.18 |
| ParkRunning3 | 0.52 | -18.45 |
| Campfire | 0.47 | -21.53 |
| Average for SDR-A | 0.23 | -8.69 |
| BQTerrace | 0.00 | 0.00 |
| RitualDance | 0.01 | -0.23 |
| MarketPlace | 0.05 | -1.85 |
| BasketballDrive | 0.00 | 0.00 |
| Cactus | 0.00 | 0.11 |
| Average for SDR-B | 0.01 | -0.39 |
| Average for SDR | 0.12 | -4.54 |

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

### Overall

| Sequence | BD-PSNR [dB] | BD-Bitrate [%] |
| --- | --- | --- |
| BQTerrace | 0.00 | 0.00 |
| RitualDance | 0.04 | -0.74 |
| MarketPlace | 0.05 | -1.60 |
| BasketballDrive | 0.01 | -0.20 |
| Cactus | 0.00 | -0.05 |
| Average | 0.02 | -0.52 |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16 and JEM 7.0

### Constraint Set 1

#### SDR-A

|  |  | ViSTRA GPU vs HM 16.16 | ViSTRA GPU vs JEM 7.0 | ViSTRA CPU vs HM 16.16 | ViSTRA CPU vs JEM 7.0 |
| --- | --- | --- | --- | --- | --- |
|  | Rate point | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio |
| FoodMarket4 | R1 | 65.28 | 8.93 | 2522.08 | 344.85 |
|  | R2 | 71.39 | 9.58 | 2681.44 | 359.98 |
|  | R3 | 6.01 | 1.03 | 6.85 | 1.17 |
|  | R4 | 6.86 | 0.98 | 8.31 | 1.19 |
| CatRobot1 | R1 | 88.77 | 11.18 | 2502.38 | 315.09 |
|  | R2 | 8.44 | 1.04 | 8.68 | 1.07 |
|  | R3 | 8.53 | 1.26 | 8.48 | 1.26 |
|  | R4 | 6.74 | 1.06 | 7.34 | 1.16 |
| DaylightRoad2 | R1 | 57.87 | 8.60 | 2707.62 | 402.59 |
|  | R2 | 7.92 | 1.06 | 6.99 | 0.94 |
|  | R3 | 6.54 | 1.47 | 6.67 | 1.50 |
|  | R4 | 6.32 | 1.09 | 7.30 | 1.26 |
| ParkRunning3 | R1 | 59.71 | 9.34 | 2209.92 | 345.76 |
|  | R2 | 67.18 | 8.57 | 2942.87 | 375.63 |
|  | R3 | 47.45 | 7.38 | 2217.66 | 345.09 |
|  | R4 | 9.50 | 1.10 | 9.07 | 1.05 |
| Campfire | R1 | 67.01 | 9.77 | 2968.50 | 432.66 |
|  | R2 | 59.20 | 9.91 | 3307.26 | 553.93 |
|  | R3 | 50.83 | 8.67 | 2951.87 | 503.31 |
|  | R4 | 4.77 | 1.01 | 6.49 | 1.38 |

#### SDR-B

|  |  | ViSTRA GPU vs HM 16.16 | ViSTRA GPU vs JEM 7.0 | ViSTRA CPU vs HM 16.16 | ViSTRA CPU vs JEM 7.0 |
| --- | --- | --- | --- | --- | --- |
|  | Rate point | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio |
| BQTerrace | R1 | 6.10 | 1.64 | 5.36 | 1.44 |
|  | R2 | 5.64 | 1.12 | 8.39 | 1.67 |
|  | R3 | 8.54 | 1.48 | 10.60 | 1.83 |
|  | R4 | 7.69 | 1.23 | 11.02 | 1.77 |
| RitualDance | R1 | 63.84 | 8.39 | 3651.25 | 479.93 |
|  | R2 | 10.36 | 1.31 | 9.42 | 1.19 |
|  | R3 | 4.99 | 0.94 | 6.17 | 1.16 |
|  | R4 | 10.02 | 1.82 | 9.82 | 1.79 |
| MarketPlace | R1 | 72.10 | 11.62 | 3579.88 | 577.02 |
|  | R2 | 50.02 | 8.04 | 3095.34 | 497.31 |
|  | R3 | 9.90 | 1.22 | 9.29 | 1.14 |
|  | R4 | 6.54 | 1.10 | 9.56 | 1.61 |
| BasketballDrive | R1 | 75.17 | 9.80 | 2961.07 | 385.86 |
|  | R2 | 14.14 | 2.87 | 5.70 | 1.16 |
|  | R3 | 7.51 | 1.34 | 6.57 | 1.18 |
|  | R4 | 6.86 | 1.16 | 8.30 | 1.41 |
| Cactus | R1 | 42.51 | 7.52 | 2620.89 | 463.50 |
|  | R2 | 10.31 | 1.41 | 9.73 | 1.34 |
|  | R3 | 9.64 | 1.22 | 12.00 | 1.51 |
|  | R4 | 9.00 | 1.17 | 8.89 | 1.15 |

### Constraint Set 2 – Low Delay

#### SDR-B

|  |  | ViSTRA GPU vs HM 16.16 | ViSTRA GPU vs JEM 7.0 | ViSTRA CPU vs HM 16.16 | ViSTRA CPU vs JEM 7.0 |
| --- | --- | --- | --- | --- | --- |
|  | Rate point | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio | Dec. Time ratio |
| BQTerrace | R1 | 5.23 | 1.26 | 5.23 | 1.26 |
|  | R2 | 4.84 | 1.17 | 4.84 | 1.17 |
|  | R3 | 5.02 | 1.15 | 5.02 | 1.15 |
|  | R4 | 3.70 | 1.04 | 3.70 | 1.04 |
| RitualDance | R1 | 33.15 | 7.11 | 2200.48 | 472.05 |
|  | R2 | 6.68 | 1.47 | 6.68 | 1.47 |
|  | R3 | 5.06 | 1.00 | 5.06 | 1.00 |
|  | R4 | 6.63 | 1.22 | 6.63 | 1.22 |
| MarketPlace | R1 | 38.17 | 8.80 | 2506.98 | 577.78 |
|  | R2 | 36.98 | 7.27 | 2753.19 | 541.31 |
|  | R3 | 6.26 | 1.22 | 6.26 | 1.22 |
|  | R4 | 6.42 | 1.23 | 6.42 | 1.23 |
| BasketballDrive | R1 | 50.73 | 9.31 | 2951.35 | 541.63 |
|  | R2 | 5.72 | 1.08 | 5.72 | 1.08 |
|  | R3 | 7.47 | 1.02 | 7.47 | 1.02 |
|  | R4 | 7.56 | 1.28 | 7.56 | 1.28 |
| Cactus | R1 | 47.20 | 7.50 | 2840.66 | 451.42 |
|  | R2 | 7.50 | 1.37 | 7.50 | 1.37 |
|  | R3 | 6.93 | 1.24 | 6.93 | 1.24 |
|  | R4 | 6.83 | 1.22 | 6.83 | 1.22 |

## Description of computing platform used to determine decoding times reported in section 5.1

In order to obtain the decoding times shown in section 6.1, a shared cluster from the University of Bristol was used. This cluster contains SandyBridge CPUs with 16 cores, 2.6 GHz clock speed and 4GB memory each. We ran each decoding job in a single core with 4 GB of allocated memory. This cluster also contains GPU nodes with NVIDIA K20, which were used to obtain the ViSTRA + GPU decoding ratios presented. It is important to note that these execution times may vary depending on the current workload of each compute node.

## Memory usage of decoder

|  |  | HM 16.16 | JEM 7.0 | ViSTRA |
| --- | --- | --- | --- | --- |
|  | Rate point | Max RAM [GB] | Max RAM [GB] | Max RAM [GB] |
| FoodMarket4 | R1 | 0.955 | 1.227 | 2.138 |
|  | R2 | 0.959 | 1.229 | 2.047 |
|  | R3 | 0.970 | 1.226 | 1.256 |
|  | R4 | 0.970 | 1.226 | 1.292 |
| CatRobot1 | R1 | 0.969 | 1.224 | 2.134 |
|  | R2 | 0.969 | 1.224 | 1.273 |
|  | R3 | 0.970 | 1.224 | 1.270 |
|  | R4 | 0.970 | 1.226 | 1.255 |
| DaylightRoad2 | R1 | 0.969 | 1.225 | 2.021 |
|  | R2 | 0.969 | 1.234 | 1.157 |
|  | R3 | 0.969 | 1.226 | 1.154 |
|  | R4 | 0.970 | 1.241 | 1.250 |
| ParkRunning3 | R1 | 0.951 | 1.224 | 1.855 |
|  | R2 | 0.969 | 1.224 | 1.969 |
|  | R3 | 0.970 | 1.224 | 1.919 |
|  | R4 | 0.970 | 1.224 | 1.244 |
| Campfire | R1 | 0.969 | 1.228 | 1.996 |
|  | R2 | 0.969 | 1.225 | 1.906 |
|  | R3 | 0.969 | 1.223 | 2.262 |
|  | R4 | 0.970 | 1.224 | 1.255 |

## Complexity characteristics of additional decoder tools

This proposal presents a single tool (ViSTRA) so the decoding time ratios are due to this tool only.

## Degree of capability for decoder parallel processing

The CNN-based up-sampling in JEM-ViSTRA decoder is suitable for parallel processing as a frame is segmented into independent blocks during the up-sampling operation.

# Encoder complexity analysis

## Encoding time and measurement methodology

### Constraint Set 1

#### SDR-A

|  |  | ViSTRA vs HM 16.16 | ViSTRA vs JEM 7.0 |
| --- | --- | --- | --- |
|  | Rate point | Enc. Time ratio | Enc. Time ratio |
| FoodMarket4 | R1 | 3.77 | 0.58 |
|  | R2 | 4.10 | 0.60 |
|  | R3 | 7.06 | 1.02 |
|  | R4 | 8.69 | 1.02 |
| CatRobot1 | R1 | 3.53 | 0.58 |
|  | R2 | 7.19 | 1.14 |
|  | R3 | 6.55 | 1.03 |
|  | R4 | 7.58 | 1.07 |
| DaylightRoad2 | R1 | 4.36 | 0.61 |
|  | R2 | 7.52 | 1.01 |
|  | R3 | 9.50 | 1.14 |
|  | R4 | 8.32 | 1.04 |
| ParkRunning3 | R1 | 4.20 | 0.57 |
|  | R2 | 4.72 | 0.60 |
|  | R3 | 5.20 | 0.62 |
|  | R4 | 8.93 | 1.03 |
| Campfire | R1 | 7.29 | 0.59 |
|  | R2 | 8.54 | 0.60 |
|  | R3 | 18.94 | 1.31 |
|  | R4 | 14.78 | 1.02 |

#### SDR-B

|  |  | ViSTRA vs HM 16.16 | ViSTRA vs JEM 7.0 |
| --- | --- | --- | --- |
|  | Rate point | Enc. Time ratio | Enc. Time ratio |
| BQTerrace | R1 | 18.41 | 1.05 |
|  | R2 | 16.73 | 1.04 |
|  | R3 | 19.25 | 1.13 |
|  | R4 | 17.58 | 1.04 |
| RitualDance | R1 | 7.35 | 0.59 |
|  | R2 | 12.39 | 1.05 |
|  | R3 | 11.04 | 1.02 |
|  | R4 | 6.16 | 1.00 |
| MarketPlace | R1 | 4.37 | 0.61 |
|  | R2 | 4.17 | 0.59 |
|  | R3 | 8.36 | 1.09 |
|  | R4 | 8.50 | 1.03 |
| BasketballDrive | R1 | 11.16 | 1.27 |
|  | R2 | 9.12 | 1.03 |
|  | R3 | 11.28 | 1.03 |
|  | R4 | 13.51 | 1.01 |
| Cactus | R1 | 9.21 | 1.31 |
|  | R2 | 8.52 | 1.06 |
|  | R3 | 8.61 | 1.00 |
|  | R4 | 9.46 | 1.07 |

### Constraint Set 2 – Low Delay

#### SDR-B

|  |  | ViSTRA vs HM 16.16 | ViSTRA vs JEM 7.0 |
| --- | --- | --- | --- |
|  | Rate point | Enc. Time ratio | Enc. Time ratio |
| BQTerrace | R1 | 4.53 | 1.04 |
|  | R2 | 5.00 | 1.02 |
|  | R3 | 6.14 | 1.02 |
|  | R4 | 6.94 | 1.03 |
| RitualDance | R1 | 7.84 | 0.63 |
|  | R2 | 13.47 | 1.02 |
|  | R3 | 14.55 | 1.04 |
|  | R4 | 13.24 | 1.06 |
| MarketPlace | R1 | 4.24 | 0.59 |
|  | R2 | 4.37 | 0.57 |
|  | R3 | 9.89 | 1.11 |
|  | R4 | 9.41 | 1.03 |
| BasketballDrive | R1 | 14.49 | 1.41 |
|  | R2 | 11.55 | 1.04 |
|  | R3 | 12.33 | 1.02 |
|  | R4 | 12.45 | 1.04 |
| Cactus | R1 | 11.20 | 1.38 |
|  | R2 | 9.94 | 1.07 |
|  | R3 | 10.09 | 1.02 |
|  | R4 | 12.20 | 1.05 |

## Description of computing platform used to determine encoding times reported in section 6.1

In order to obtain the decoding times shown in section 5.1, a shared cluster from the University of Bristol was used. This cluster contains Lenovo nx360 m5 compute nodes which have two 14 core 2.4 GHz Intel E5-2680 v4 (Broadwell) CPUs, and 128 GB of RAM. We run each decoding job in a single core with 8GB of allocated memory. It is important to note that these execution times may vary depending on the current workload of each compute node.

## Memory usage of encoder

|  |  | HM 16.16 | JEM 7.0 | ViSTRA |
| --- | --- | --- | --- | --- |
|  | Rate point | Max RAM [GB] | Max RAM [GB] | Max RAM [GB] |
| FoodMarket4 | R1 | 1.865 | 6.009 | 1.78 |
|  | R2 | 1.830 | 6.011 | 1.78 |
|  | R3 | 1.865 | 6.010 | 6.01 |
|  | R4 | 1.847 | 6.009 | 6.01 |
| CatRobot1 | R1 | 1.829 | 6.009 | 1.79 |
|  | R2 | 1.882 | 6.010 | 6.01 |
|  | R3 | 1.863 | 6.008 | 6.01 |
|  | R4 | 1.863 | 6.005 | 6.01 |
| DaylightRoad2 | R1 | 1.863 | 6.008 | 1.77 |
|  | R2 | 1.846 | 6.010 | 6.01 |
|  | R3 | 1.854 | 6.005 | 6.01 |
|  | R4 | 1.889 | 6.055 | 6.01 |
| ParkRunning3 | R1 | 1.821 | 6.005 | 1.79 |
|  | R2 | 1.821 | 6.004 | 1.80 |
|  | R3 | 1.823 | 6.008 | 1.79 |
|  | R4 | 1.821 | 6.010 | 6.01 |
| Campfire | R1 | 1.807 | 6.008 | 1.78 |
|  | R2 | 1.806 | 6.003 | 1.78 |
|  | R3 | 1.804 | 6.002 | 6.01 |
|  | R4 | 1.807 | 6.006 | 6.01 |

## Complexity characteristics of additional encoder tools

This proposal presents a single tool (ViSTRA) so the encoding time ratios are due to this tool only.

## Degree of capability for encoder parallel processing

The capability of JEM-ViSTRA encoder for parallel processing remains the same as the JEM anchor.

# Algorithmic characteristics

## Random access characteristics

The Random Access characteristics of the proposed method are identical to the JEM anchor.

## Delay characteristics

The delay characteristics of the proposed method are identical to the JEM anchor.

## Compression performance discussion

Based on the observation of all the test results, it is noted that the proposed JEM-ViSTRA codec has the following characteristics:

- JEM-ViSTRA offers more significant improvement on overall coding efficiency for UHD sequences (SDR-A) than for HD ones (SDR-B).

- JEM-ViSTRA offers more significant improvement on overall coding efficiency for sequences with relatively complex motion (e.g. ParkRunning3 and Campfire sequences) than for those with relatively simple motion (e.g. CatRobot1and DaylightRoad2).

- JEM-ViSTRA offers more significant improvement on overall coding efficiency at lower bitrates (when QP values are relatively high) than for higher bitrate ranges (when QP values are relatively low).

# Software implementation description

The software for this proposal is based on the JEM software version 7.0.

The decoder was implemented in C++. The employed CNN model in the decoder is based on CAFFE v1.0 deep learning framework [12] and two options are provided: a GPU (requires a NVIDIA GPU) and a CPU decoder. Both versions require additional libraries (.dll) dependencies which are provided with the decoder. In addition, the CNN model parameters are stored in .caffemodel files.

The encoder was implemented in C++ and MATLAB. The latter is for the development of the Quantisation-resolution Optimisation module, and resolution down-sampling. These will be combined with the C++ encoder in the future versions.

# Closing remarks

This contribution has described a new resolution adaptation approach (ViSTRA) to video encoding. This approach trades the relationship between resolutions (spatial resolution and bit depth) and quantisation parameters, and selects optimal resolutions for encoding the input video frames. At the decoder, a deep CNN-based super resolution method is employed for resolution up-sampling. The proposed method was evaluated on the SDR test sets in the CfP, and is found to achieve average bit-rate saving -4.54% for CS1 and -0.52% for CS2. The savings were found to be greater on the SDR-A (UHD) test set than on SDR-B (HD).

The ViSTRA codec is the topic of ongoing development which will further improve compression performance. This includes:

- Fully integration of ViSTRA into the encoding loop.

- Temporal and spatio-temporal resolution adaptation.

- Bitstream syntax modification.

- Further improvements to deal with dynamic textures.

# Patent rights declaration(s)

University of Bristol may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | A. Mackin, M. Afonso, F. Zhang and D. Bull, "SRQM: A Video Quality Metric for Spatial," in Picture Coding Symposium (PCS), 2018. |
| --- | --- |
| [2] | D. Bull, Communicating Pictures: a Course in Image and Video Coding, Elsevier, 2014. |
| [3] | A. V. Katsenou, M. Afonso, D. Agrafiotis and D. R. Bull, "Predicting Video Rate-Distortion Curves using," in Picture Coding Symposium (PCS), 2016. |
| [4] | "Normalized 2-D cross-correlation," Matlab documentation, [Online]. Available: https://uk.mathworks.com/help/images/ref/normxcorr2.html. [Accessed March 2018]. |
| [5] | "Magnitude-squared coherence," Matlab documentation, [Online]. Available: https://uk.mathworks.com/help/signal/ref/mscohere.html. [Accessed March 2018]. |
| [6] | M. Afonso, F. Zhang, A. Katsenou, D. Agrafiotis and D. Bull, "Low Complexity Video Coding Based on Spatial Resolution Adaptation," in IEEE International Conference on Image Processing, 2017. |
| [7] | M. Afonso, A. Katsenou, F. Zhang, D. Agrafiotis and D. Bull, "Video Texture Analysis based on HEVC Encoding Statistics," in PCS, 2016. |
| [8] | S. G. Ken Turkowski, "Filters for Common Resampling Tasks," in Graphics Gems I, Academic Press, 1990, p. 147–165. |
| [9] | J. Kim, J. K. Lee and K. M. Lee, "Accurate Image Super-Resolution Using Very Deep Convolutional Networks," in IEEE Conference on Computer Vision and Pattern Recognition, 2016. |
| [10] | "Harmonic Inc free 4K demo footage," [Online]. Available: https://www.harmonicinc.com/4k-demo-footage-download/. [Accessed March 2018]. |
| [11] | "Tampere 4K database," [Online]. Available: http://ultravideo.cs.tut.fi/#testsequences. |
| [12] | M. Papadopoulos, F. Zhang, D. Agrafiotis and D. Bull, "A VIDEO TEXTURE DATABASE FOR PERCEPTUAL COMPRESSION AND QUALITY ASSESSMENT," in Proc. IEEE Int Conf. on Image Processing, 2015. |
| [13] | "LIVE Netflix Video Quality of Experience Database," [Online]. Available: http://live.ece.utexas.edu/research/LIVE_NFLXStudy/nflx_index.html. [Accessed March 2018]. |

# Annex: Questionnaire

## Codec level Q&A

| Codec Level Q&A |  | JEM | JEM-ViSTRA |
| --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 128×128 chroma 64×64 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/128×128 chroma 2×2/64×64 | luma 4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 2×2 | luma 4×4 chroma 2×2 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | template matching in FRUC, LIC | template matching in FRUC, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | 64-bit arithmetic for LIC, BIO and LMC | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | Yes, in I-slices | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |
| 12 | If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input) | n/a | The spatial resolution and bit depth may be reduced for encoding by a single ratio (2 for spatial resolution and 1bit for bit depth), e.g. bit depth from 10bit to 9 bit, and/or spatial resolution from 4K to HD. |

## Tool level Q&A

### Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 1. Partitioning structure |  | JEM | JEM-ViSTRA |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | No | No |
| 1.b | Maximum tree depth | QT maximum depth 5 BT maximum depth 3 | QT maximum depth 5 BT maximum depth 3 |

### Category 2. Entropy coding

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 2. Entropy coding |  | JEM | JEM-ViSTRA |
| 2.a | Total number of context models | 329 | 329 |
| 2.b | Memory storage size for each context model | 30-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[512×64×9bits]; RenormTable[128×3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×329×8 + 329×15×5 bits | 3×329×8 + 329×15×5 bits |
| 2.e | Other memory associated with context model adaptation | n/a | n/a |

### Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM-ViSTRA |
| 3.a | Temporal motion data storage granularity | 4×4 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 4 | 4 |

### Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM-ViSTRA |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC FRUC not constrained | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained |

### Category 5. Intra prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 5. Intra prediction |  | JEM | JEM-ViSTRA |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 4, chroma 2 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) |

### Category 6. Inter prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 6. Inter prediction |  | JEM | JEM-ViSTRA |
| 6.a | Motion vector accuracy | luma 1/16 pel chroma 1/32 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

### Category 7. Quantization

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 7. Quantization |  | JEM | JEM-ViSTRA |
| 7.a | Storage size associated with quantization | n/a | n/a |

### Category 8. Transforms

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 8. Transforms |  | JEM | JEM-ViSTRA |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) |
| 8.c | List all the non-separable transforms used | 4×4/8×8 HyGT | 4×4/8×8 HyGT |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | factorized form not equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations |

### Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 9. In-loop filtering |  | JEM | JEM-ViSTRA |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) |

### Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | JEM | JEM-ViSTRA |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | ViSTRA. Since this is the only tool proposed, the coding runtime and gain are the same as presented in Section 4. |

[end of document]