| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0010 |
| --- | --- |

| Title: | JVET AHG report: Encoding algorithm optimizations (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alberto Duenas Alexis Michael Tourapis C. Helmrich Sergey Ikonin Andrey Norkin Rickard Sjöberg | Email: | alberto.duenas@arm.com atourapis@apple.com elena.alshina@gmail.com christian.helmrich@hhi.fraunhofer.de Sergey.Ikonin@huawei.com anorkin@netflix.com rickard.sjoberg@ericsson.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

The document summarizes the activities of the AHG on Encoding algorithm optimizations between the 11th meeting in Ljubljana, SI (10-18, July 2018) and the 12th meeting in Macao, CN (3-12, October 2018).

# Introduction

At the 11th JVET meeting, the AHG on Encoding algorithm optimizations was established with the following mandates:

- Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization.

- Study the impact of adaptive quantization on individual tools in the test model.

- Study the quantization adaptation tool in the test model.

- Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on.

- Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding.

- Study methods of rate control and their impact on performance, subjective and objective quality

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Overview of input documents related to the AHG

The following input documents were identified to be related to the AHG:

## JVET-L0210: CE 7: Adaptive quantization via perceptually optimized QP adaptation (Test 7.2.6) by Fraunhofer HHI

In JVET-H0047, the authors proposed a CTU-wise subjectively optimized QP adaptation (QPA) along with a correspondingly weighted PSNR (WPSNR) distortion measure.  This QPA approach was further improved in JVET-K0206 and accepted for integration into (and optional activation in) the VTM/BMS software.  Note that this is a non-normative encoder optimization – the delta-QP values are signaled in a HEVC-like fashion.

This document reports on Bjøntegaard delta (BD) PSNR and MS-SSIM results gathered from comparative measurements between VTM 2.0.1 with activated perceptual QPA and VTM 2.0.1 with fixed-QP encoding

## JVET-L0241: AHG10: Adaptive lambda ratio estimation for rate control in VVC by Wuhan University and Tencent

This contribution presents some improvements based on the current rate control scheme proposed in JVET-K0390. With the proposed adaptive lambda ratio estimation algorithm, when using the anchor bit rate of BMS2.1 with VTM configuration as the target, there are X%/X%/X% for Y/U/V coding efficiency improvements in random access configuration when compared with the rate control algorithm in K0390.

## JVET-L0365: MS-SSIM as an additional metric

This contribution proposes to include the MS-SSIM metric as additional metric in VTM and make MS-SSIM Y mandatory in the CTC for SDR video. A patch for MS-SSIM integrated into VTM 2.0.1 and an updated excel template for the CTC for SDR video are provided.

# Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.