| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0021 |
| --- | --- |

| Title: | Summary report for CE1: Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Core experiment report |  |  |
| Author(s) or Contact(s): | Jackie Ma Fraunhofer HHI Fabrice Le Léannec Technicolor Min Woo Park Samsung Electronics | Email: | jackie.ma@hhi.fraunhofer.de fabrice.leleannec@technicolor.com m.w.park@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document evaluates CE1: Partitioning JVET-K1021. In total there are 11 tests for VTM. All tests have been cross-checked by at least one cross-checker. The cross-checkers reported in some cases mismatches in timings, otherwise there were no mismatches reported.

# Introduction

During the 11th Meeting in Ljubljana in July 2018 it was decided to have several technologies to be studied in a further CE on Partitioning. Some technologies were part of [1] or related to it. The tests described in [2] can be categorized in three categories:

- Picture boundary handling

- Split constraints

- Separate trees

The software for these tests that are conducted in SubCE can be found here

https://vcgit.hhi.fraunhofer.de/JVET-K-CE1/VVCSoftware_VTM.git

# SubCEs

## SubCE1

### SubCE1-1.1.1

(JVET-L0080, MediaTek)

In SubCE1-1.1.1 an alternative picture boundary handling method is proposed that does not use adaptive BT depths at the boundary.

#### Test description

In SubCE1-1.1.1 it is proposed to forgo the QT-split restrictions at the picture boundary and always infer a QT split if three corner points exceed the picture boundary. Further, the additional BT depth counter at the boundary is removed to ensure the BT depth does not violate the specified maximum BT depth.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 1.1.1 | S-T. Hsiang (MediaTek) | Cross-check by T. Poirer: Cross-check by H. Gao: The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. No mismatches were reported. | T. Poirier (Technicolor) H. Gao (Huawei) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 99% | 0,00% | 0,03% | 0,03% | 99% | 98% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 99% | 100% | 0,00% | 0,00% | -0,06% | 100% | 100% |  |  |  |  |  |
| Class B | 0,00% | 0,02% | 0,05% | 99% | 98% | 0,04% | 0,11% | -0,01% | 98% | 99% | 0,05% | 0,12% | 0,06% | 99% | 101% |
| Class C | 0,00% | 0,02% | 0,05% | 99% | 97% | 0,02% | 0,15% | 0,06% | 97% | 96% | 0,04% | -0,08% | -0,16% | 98% | 96% |
| Class E | 0,00% | -0,01% | 0,00% | 99% | 97% |  |  |  |  |  | -0,03% | -0,50% | -0,84% | 98% | 95% |
| Overall | 0,00% | 0,01% | 0,02% | 99% | 98% | 0,02% | 0,08% | 0,01% | 99% | 98% | 0,03% | -0,10% | -0,24% | 98% | 98% |
| Class D | 0,02% | 0,14% | -0,02% | 97% | 100% | 0,04% | 0,29% | 0,21% | 94% | 97% | 0,05% | 0,25% | -0,18% | 97% | 96% |
| Class F (optional) | 0,01% | 0,06% | 0,07% | 99% | 98% | 0,02% | 0,10% | 0,04% | 98% | 96% | 0,04% | 0,10% | -0,14% | 98% | 95% |

### SubCE1-1.2.1

(JVET-L0268, Bytedance)

In SubCE1-1.2.1 and SubCE1-1.2.2 a new CU type is introduced which is called Zero-Unit (ZU). A ZU is a CU with width or height not being a power of 2. Hence, a ZU can only appear at the picture boundary. In current VTM a CU with non-power of 2 width or height is further split.

#### Test description/configuration

In SubCE1-1.2.1 the cbf_flag for ZUs must be signaled to be 0 by an encoder restriction.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 1.2.1 | K. Zhang (Bytedance) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. No mismatches were reported. | H. Gao (Huawei) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,07% | 0,06% | 100% | 104% | -0,05% | -0,03% | -0,09% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0,00% | 0,06% | 0,01% | 100% | 102% | -0,01% | 0,01% | -0,15% | 100% | 99% |  |  |  |  |  |
| Class B | 0,03% | 0,08% | 0,07% | 99% | 100% | -0,05% | -0,06% | 0,11% | 101% | 100% | -0,01% | -0,17% | -0,41% | 101% | 102% |
| Class C | 0,12% | 0,23% | 0,33% | 95% | 97% | 0,10% | 0,16% | 0,31% | 98% | 97% | 0,09% | 0,31% | 0,11% | 97% | 95% |
| Class E | 0,14% | 0,02% | 0,07% | 99% | 101% |  |  |  |  |  | -0,50% | -0,19% | -0,51% | 99% | 91% |
| Overall | 0,06% | 0,10% | 0,12% | 98% | 101% | 0,00% | 0,02% | 0,07% | 100% | 99% | -0,10% | -0,02% | -0,27% | 99% | 97% |
| Class D | 0,32% | 0,66% | 0,44% | 93% | 103% | 0,07% | 0,33% | 0,30% | 97% | 102% | 0,15% | 1,04% | 0,10% | 98% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### SubCE1-1.2.2

(JVET-L0268, Bytedance)

This is a further test on the proposed ZUs (cf. SubCE1-1.2.1).

#### Test description/configuration

In SubCE1-1.2.2 ZUs must be further split by an encoder restriction.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 1.2.2 | K. Zhang (Bytedance) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. No mismatches were reported. | H. Gao (Huawei) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,08% | 0,00% | 102% | 105% | 0,11% | 0,14% | 0,09% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0,02% | 0,06% | 0,03% | 100% | 100% | 0,07% | 0,14% | 0,00% | 101% | 99% |  |  |  |  |  |
| Class B | 0,06% | 0,05% | 0,06% | 100% | 100% | 0,13% | 0,15% | 0,22% | 100% | 99% | 0,16% | 0,08% | 0,10% | 100% | 100% |
| Class C | 0,12% | 0,23% | 0,33% | 96% | 98% | 0,14% | 0,23% | 0,34% | 98% | 100% | 0,15% | 0,02% | 0,01% | 97% | 94% |
| Class E | 0,14% | 0,02% | 0,07% | 100% | 98% |  |  |  |  |  | 0,33% | 0,27% | 0,13% | 99% | 94% |
| Overall | 0,07% | 0,09% | 0,11% | 100% | 100% | 0,12% | 0,17% | 0,18% | 100% | 100% | 0,20% | 0,10% | 0,07% | 99% | 96% |
| Class D | 0,33% | 0,63% | 0,46% | 93% | 99% | 0,41% | 0,64% | 0,67% | 96% | 102% | 0,46% | 1,45% | 1,08% | 97% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### SubCE1-1.3.1

(JVET-L0310, Tencent)

In SubCE1-1.3.1 the concept of a partial CU is introduced. A partial CU is a CU which contains areas inside and outside the picture. However, only the area inside the picture is further coded. Moreover, only partial CUs are considered when the CU is coded as non-split and the part inside the picture boundaries has width or height non-power of 2.

#### Test description/configuration

The concept of a partial CU is only used for in non-intra slices. Further, the residual is not coded for a partial CU and the skip flag is inferred as true for the part inside the picture.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 1.3.1 | M. Xu (Tencent) | The cross checker (CC) confirms that the implementation matches the description and the simulation results provided by the proponent are correct. CC reports mismatches in encoding times (7% more than runtimes proponent) and decoding times (10%-12% more than runtimes by proponent). | H. Gao (Huawei) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 94% | 94% | -0,04% | 0,00% | -0,02% | 94% | 93% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 94% | 95% | -0,01% | -0,06% | -0,10% | 94% | 93% |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 93% | 93% | -0,01% | 0,01% | -0,07% | 93% | 87% | -0,01% | 0,00% | 0,07% | 95% | 97% |
| Class C | 0,00% | 0,00% | 0,00% | 92% | 90% | 0,00% | -0,01% | -0,03% | 92% | 81% | 0,03% | -0,14% | -0,17% | 91% | 82% |
| Class E | 0,00% | 0,00% | 0,00% | 93% | 90% |  |  |  |  |  | -0,34% | -0,39% | -0,74% | 92% | 84% |
| Overall | 0,00% | 0,00% | 0,00% | 93% | 92% | -0,02% | -0,01% | -0,06% | 93% | 88% | -0,08% | -0,14% | -0,21% | 93% | 88% |
| Class D | 0,00% | 0,00% | 0,00% | 92% | 83% | -0,06% | -0,08% | -0,09% | 92% | 75% | 0,03% | 0,49% | 0,04% | 92% | 76% |
| Class F (optional) | 0,00% | 0,00% | 0,00% | 92% | 91% | -0,01% | -0,03% | -0,02% | 93% | 83% | -0,18% | 0,50% | 0,26% | 91% | 84% |

## SubCE2

SubCE2 consists of four tests with the aim to reduce the VPDA size in VVC. Virtual pipeline data units (VPDUs) are non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In HEVC the VPDU size is set to maximum transform block size which is 32x32-L/16x16-C. This is compared to 128x128-L/64x64-C in VVC which leads to the request of larger VPDA sizes.

All four tests in this SubCE are designed/configured so that the following two conditions are not violated:

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

Further, the processing order of CUs shall not leave a VPDU and re-visit it later.

### SubCE1-2.1.1

(JVET-L0081, MediaTek)

#### Test description/configuration

The following non-normative splitting procedure was tested. If one of the following conditions is true, then a CU has to be further split:

- If the top-left position of the current CU is the top-left position of a VPDU and

  - the width of the current CU is larger than 64 but not a multiple of 64 or

  - the height of the current CU is larger than 64 but not a multiple of 64 or

  - the VPDU to the left and the VPDU to the top of the current CU are not yet completely encoded

- If the current CU is not completely contained by a VPDU.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 2.1.1 | C.-W. Hsu (MediaTek) | The cross-checker (CC) confirms the simulations results. CC confirms the implementation of the constraint for skipping a CU if processing order would revisit VPDU. CC also confirms the implementation matches the CE description. It was noted that the implementation imposes conditions as encoder search restriction on CU mode testing rather than a normative restriction. Also, a fast search was disabled to avoid triggering the undesired CU sizes. | C. Rosewarne (Canon) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,41% | 0,60% | 0,48% | 90% | 105% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,52% | 0,48% | 0,38% | 95% | 107% |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,21% | 0,30% | 0,22% | 95% | 103% | 0,15% | 0,21% | 0,02% | 98% | 103% |
| Class C | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,03% | 0,14% | 0,03% | 98% | 101% | 0,06% | 0,08% | 0,03% | 99% | 100% |
| Class E | 0,00% | 0,00% | 0,00% | 100% | 100% |  |  |  |  |  | 0,85% | 0,45% | 0,74% | 96% | 100% |
| Overall | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,26% | 0,35% | 0,25% | 95% | 104% | 0,30% | 0,23% | 0,20% | 98% | 101% |
| Class D | 0,00% | 0,00% | 0,00% | 100% | 101% | 0,08% | 0,15% | 0,09% | 98% | 99% | 0,10% | 0,49% | -0,55% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### SubCE1-2.1.2

(JVET-L0081, MediaTek)

#### Test description/configuration

The following normative prohibitions were tested (all together):

- A 128x128 CTU cannot be split

- 64x128 and 128x64 CUs cannot be partitioned by a TT split

- 128x64 CU cannot be partitioned using a horizontal BT split

- 64x128 CU cannot be partitioned using a vertical BT split

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 2.1.2 | C.-W. Hsu (MediaTek) |  | M. Park (Samsung) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,22% | 0,32% | 0,22% | 90% | 100% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 101% | 0,38% | 0,34% | 0,26% | 94% | 101% |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 101% | 0,08% | 0,16% | 0,11% | 95% | 100% | 0,06% | 0,07% | -0,16% | 98% | 101% |
| Class C | 0,00% | 0,00% | 0,00% | 99% | 101% | 0,02% | 0,07% | 0,04% | 98% | 101% | 0,02% | 0,01% | -0,17% | 99% | 101% |
| Class E | 0,00% | 0,00% | 0,00% | 100% | 100% |  |  |  |  |  | 0,10% | 0,22% | -0,12% | 95% | 97% |
| Overall | 0,00% | 0,00% | 0,00% | 100% | 101% | 0,15% | 0,20% | 0,14% | 94% | 101% | 0,06% | 0,09% | -0,15% | 98% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 100% | 102% | 0,00% | -0,01% | 0,03% | 98% | 100% | 0,06% | 0,28% | 0,01% | 99% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### SubCE1-2.1.3

(JVET-L0081, MediaTek)

#### Test description/configuration

The maximum BT and TT sizes are set to 64.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 2.1.3 | C.-W. Hsu (MediaTek) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. No mismatches were reported. | Y. Zhao (Huawei) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,69% | 0,88% | 0,63% | 78% | 101% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,67% | 0,64% | 0,45% | 86% | 102% |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,24% | 0,28% | 0,16% | 88% | 101% | 0,17% | 0,08% | 0,14% | 92% | 102% |
| Class C | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,01% | 0,11% | 0,01% | 95% | 100% | 0,05% | 0,07% | -0,21% | 98% | 101% |
| Class E | 0,00% | 0,00% | 0,00% | 100% | 100% |  |  |  |  |  | 0,51% | 0,60% | 0,55% | 81% | 100% |
| Overall | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,35% | 0,43% | 0,27% | 87% | 101% | 0,22% | 0,21% | 0,13% | 91% | 101% |
| Class D | 0,00% | 0,00% | 0,00% | 100% | 102% | 0,06% | 0,10% | 0,01% | 94% | 99% | 0,05% | 0,15% | 0,13% | 98% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### SubCE1-2.1.4

(JVET-L0081, MediaTek)

#### Test description/configuration

A 128x128 CTU is forced to be split into four 64x64 CUs.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 2.1.4 | C.-W. Hsu (MediaTek) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. No mismatches were reported. | J. Ma (Fraunhofer HHI) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,00% | 0,00% | 100% | 100% | 2,66% | 2,32% | 2,21% | 73% | 104% |  |  |  |  |  |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 100% | 2,87% | 2,27% | 2,04% | 82% | 102% |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 100% | 1,25% | 0,96% | 0,88% | 84% | 101% | 0,85% | 0,49% | 0,30% | 89% | 99% |
| Class C | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,10% | 0,14% | 0,04% | 94% | 100% | 0,10% | -0,13% | 0,11% | 97% | 99% |
| Class E | 0,00% | 0,00% | 0,00% | 100% | 100% |  |  |  |  |  | 2,26% | 2,35% | 1,78% | 73% | 100% |
| Overall | 0,00% | 0,00% | 0,00% | 100% | 100% | 1,55% | 1,28% | 1,15% | 84% | 102% | 0,95% | 0,75% | 0,60% | 87% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 100% | 100% | 0,06% | 0,13% | -0,04% | 93% | 98% | 0,05% | 0,46% | -0,10% | 97% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## SubCE3

This SubCE studies the use of separate trees for intra in inter slices. Results are reported for CTC and for synthetic sequences that were also provided by the proponents. The synthetic sequences were generated by copying sample values from another sequence in a checker-board pattern. The used sequences vary per GOP. It expected that these synthetic sequences lead to higher failure rate of inter prediction. The use of separate trees is dependent on  a threshold signaled in the SPS. If the number of luma samples exceed the threshold, then separate trees are available. Likewise there is a threshold in the SPS for if the number of samples is below the threshold, separate trees are not available. In the remaining case there is a flag in the bit-stream that controls if separate trees are available or not.

### SubCE1-3.1.1

(JVET-L0424, Sharp)

#### Test description/configuration

Separate tree support at CU level for intra-coded CUs in inter slices is tested. No modifications for intra slices.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 3.1.1 | K. Misra (Sharp) | Cross-check by T. Poirier: Cross-check by A. Wieckowski: The cross-checker (CC) confirmed the proponents’ results. However, different timings were obtained. Deviations up to roughly 20% per class can be observed. CC noted that the own timings were also not reliable. Further, software comments of cosmetic nature have been expressed by the CC. | T. Poirier (Technicolor) A. Wieckowski (Fraunhofer HHI) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,04% | -0,01% | 109% | 112% | -0,49% | -4,00% | -2,10% | 127% | 101% |  |  |  |  |  |
| Class A2 | -0,01% | -0,03% | -0,12% | 121% | 122% | -0,06% | -2,14% | -0,90% | 108% | 91% |  |  |  |  |  |
| Class B | 0,01% | 0,00% | 0,08% | 121% | 116% | -0,13% | -2,26% | -2,64% | 123% | 102% | 0,01% | -0,64% | -1,12% | 114% | 98% |
| Class C | -0,03% | -0,23% | -0,05% | 112% | 113% | 0,07% | -1,26% | -1,45% | 129% | 106% | 0,06% | -0,36% | -0,32% | 115% | 103% |
| Class E | -0,01% | -0,12% | -0,29% | 88% | 92% |  |  |  |  |  | -0,22% | -0,65% | -0,22% | 117% | 108% |
| Overall | -0,01% | -0,07% | -0,06% | 111% | 111% | -0,13% | -2,32% | -1,87% | 122% | 101% | -0,03% | -0,55% | -0,63% | 115% | 102% |
| Class D | 0,02% | 0,02% | -0,26% | 107% | 109% | 0,15% | -0,71% | -0,15% | 117% | 100% | 0,05% | -0,32% | -0,24% | 109% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Results for synthetic sequences

|  | Random Access Main 10 (Synthetic) |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0,81% | -2,92% | -1,55% | 143% | 107% |  |  |  |  |  |
| Class A2 | -0,49% | -1,91% | -1,00% | 147% | 102% |  |  |  |  |  |
| Class B | -0,75% | -4,24% | -5,16% | 134% | 100% | -0.47% | -4.18% | -5.06% | 126% | 82% |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,69% | -3,24% | -3,04% | 140% | 102% | -0.47% | -4.18% | -5.06% | 126% | 82% |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

### SubCE1-3.1.2

(JVET-L0424, Sharp)

#### Test description/configuration

CU level separate trees for intra-coded CUs in inter slices combined with support for both separate and shared trees in intra slices is tested, i.e. both slice types have separate tree signaling at CU level.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 3.1.2 | K. Misra (Sharp) | The cross-checker (CC) confirmed the proponents’ results. However, different timings were obtained. Deviations up to roughly 20% per class can be observed. CC noted that the own timings were also not reliable. Further, software comments of cosmetic nature have been expressed by the CC. | A. Wieckowski (Fraunhofer HHI) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,11% | 0,00% | 83% | 86% | -0,49% | -3,91% | -2,28% | 130% | 103% |  |  |  |  |  |
| Class A2 | 0,00% | -0,06% | -0,14% | 107% | 106% | -0,03% | -2,11% | -0,92% | 129% | 105% |  |  |  |  |  |
| Class B | 0,03% | 0,05% | 0,10% | 87% | 86% | -0,11% | -2,16% | -2,62% | 125% | 102% | 0,01% | -0,77% | -1,31% | 106% | 88% |
| Class C | 0,02% | -0,22% | -0,07% | 90% | 91% | 0,09% | -1,23% | -1,41% | 134% | 109% | 0,07% | -0,38% | -0,63% | 115% | 103% |
| Class E | 0,01% | -0,14% | -0,24% | 81% | 83% |  |  |  |  |  | -0,12% | -0,85% | -0,54% | 111% | 105% |
| Overall | 0,02% | -0,05% | -0,05% | 89% | 90% | -0,11% | -2,25% | -1,89% | 129% | 104% | 0,00% | -0,66% | -0,89% | 110% | 97% |
| Class D | 0,14% | 0,22% | -0,14% | 102% | 103% | 0,17% | -0,52% | -0,19% | 126% | 107% | 0,05% | 0,27% | -0,20% | 105% | 96% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Results for synthetic sequences

|  | Random Access Main 10 (Synthetic) |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0,81% | -2,89% | -1,54% | 140% | 103% |  |  |  |  |  |
| Class A2 | -0,49% | -1,89% | -1,00% | 157% | 104% |  |  |  |  |  |
| Class B | -0,75% | -4,27% | -5,17% | 139% | 96% | -0.47% | -4.09% | -5.07% | 116% | 80% |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,69% | -3,24% | -3,04% | 144% | 100% | -0.47% | -4.09% | -5.07% | 116% | 80% |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

### SubCE1-3.2.1

(JVET-L0424, Sharp)

#### Test description/configuration

Separate tree support at CTU level for intra-coded CTUs in inter slices. No modifications for intra slices.

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 3.2.1 | K. Misra (Sharp) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. Tiny mismatches for RA have been reported most likely due to parallel setting mismatches. | C. Rosewarne (Canon) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,04% | -0,01% | 79% | 91% | 0,00% | 0,09% | 0,00% | 92% | 92% |  |  |  |  |  |
| Class A2 | -0,01% | -0,03% | -0,12% | 97% | 103% | 0,02% | -0,12% | 0,03% | 91% | 91% |  |  |  |  |  |
| Class B | 0,01% | 0,00% | 0,08% | 89% | 95% | 0,01% | 0,03% | 0,07% | 88% | 92% | 0,00% | 0,02% | 0,14% | 79% | 87% |
| Class C | -0,03% | -0,23% | -0,05% | 86% | 91% | 0,02% | 0,03% | -0,03% | 107% | 106% | 0,00% | 0,03% | -0,14% | 85% | 90% |
| Class E | -0,01% | -0,12% | -0,29% | 75% | 82% |  |  |  |  |  | -0,17% | -0,32% | -0,37% | 84% | 89% |
| Overall | -0,01% | -0,07% | -0,06% | 85% | 92% | 0,01% | 0,01% | 0,02% | 94% | 95% | -0,04% | -0,06% | -0,08% | 82% | 89% |
| Class D | 0,02% | 0,02% | -0,26% | 83% | 88% | 0,02% | -0,26% | 0,04% | 97% | 98% | 0,02% | 0,00% | -0,73% | 86% | 88% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Results for synthetic sequences

|  | Random Access Main 10 (Synthetic) |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.01% | 88% | 95% |  |  |  |  |  |
| Class A2 | 0.00% | 0.02% | 0.02% | 92% | 94% |  |  |  |  |  |
| Class B | -0.02% | -0.01% | 0.01% | 87% | 90% | 0.01% | 0.01% | 0.03% | 77% | 80% |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 88% | 92% | 0.01% | 0.01% | 0.03% | 77% | 80% |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

### SubCE1-3.2.2

(JVET-L0424, Sharp)

#### Test description/configuration

For all slices a flag is sent at CTU level. If the flag is true, then the CTU is coded as intra CTU using separate trees. If the flag is false, then CU level separate trees are used at CU-level for intra-coded CUs in inter slices and for intra slices both, shared and separate trees are supported (cf. SubCE1-3.1.2).

#### Cross-checking activity

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 3.2.2 | K. Misra (Sharp) | The cross checker confirms that the implementation matches the description and the simulation results provided by the proponent are correct. Tiny mismatches for RA have been reported most likely due to parallel setting mismatches. | C. Rosewarne (Canon) |

#### Results

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,11% | 0,00% | 84% | 93% | -0,48% | -3,89% | -2,25% | 104% | 90% |  |  |  |  |  |
| Class A2 | 0,00% | -0,06% | -0,14% | 95% | 104% | -0,02% | -2,09% | -0,91% | 101% | 87% |  |  |  |  |  |
| Class B | 0,04% | 0,05% | 0,10% | 83% | 89% | -0,10% | -2,15% | -2,60% | 112% | 93% | 0,03% | -0,53% | -1,34% | 89% | 87% |
| Class C | 0,02% | -0,22% | -0,07% | 89% | 92% | 0,10% | -1,23% | -1,40% | 105% | 90% | 0,08% | -0,35% | -0,57% | 99% | 90% |
| Class E | 0,01% | -0,14% | -0,24% | 74% | 82% |  |  |  |  |  | 0,01% | -0,39% | -0,10% | 88% | 89% |
| Overall | 0,02% | -0,05% | -0,05% | 85% | 91% | -0,11% | -2,24% | -1,87% | 106% | 90% | 0,04% | -0,43% | -0,78% | 92% | 89% |
| Class D | 0,14% | 0,22% | -0,14% | 85% | 89% | 0,17% | -0,52% | -0,18% | 103% | 89% | 0,05% | 0,28% | -0,19% | 93% | 86% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Results for synthetic sequences

|  | Random Access Main 10 (Synthetic) |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -2.89% | -1.53% | 121% | 100% |  |  |  |  |  |
| Class A2 | -0.48% | -1.89% | -1.00% | 129% | 98% |  |  |  |  |  |
| Class B | -0.74% | -4.27% | -5.16% | 122% | 93% | -0.47% | -4.10% | -5.06% | 114% | 82% |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.69% | -3.24% | -3.04% | 124% | 96% | -0.47% | -4.10% | -5.06% | 114% | 82% |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” JVET-J1021, Apr. 2018, San Diego.

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” JVET-K1021, July 2018, Ljubljana.

# A	Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
|  | Yu-Chen Sun | Alibaba Inc. | yuchen.s@alibaba-inc.com |  |  |
|  | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
|  | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
|  | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
|  | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
|  | Kai Zhang | Bytedance | zhangkai.video@bytedance.com | X |  |
|  | Li Zhang | Bytedance | lizhang.idm@bytedance.com |  |  |
|  | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |  |  |
|  | Chris Rosewarne | Canon | chris.rosewarne@cisra.canon.com.au |  |  |
|  | Andrew Dorrell | Canon | andrew.dorrell@cisra.canon.com.au |  |  |
|  | Hyung Kim | Cisco | hyungk@cisco.com |  |  |
|  | Julien Le Tanou | Ericsson | julien.le.tanou@ericsson.com |  |  |
|  | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |  |
|  | Yao-Jen Chang | Foxconn | yaojen.chang.foxconn@gmail.com |  |  |
|  | Jackie Ma | Fraunhofer HHI | jackie.ma@hhi.fraunhofer.de |  | X |
|  | Adam Wieckowski | Fraunhofer HHI | adam.wieckowski@hhi.fraunhofer.de |  | X |
|  | Jiangqing Zhu | Fujitsu | zhujianqing@cn.fujitsu.com |  |  |
|  | Jie Yao | Fujitsu | yaojie@cn.fujitsu.com |  |  |
|  | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
|  | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
|  | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
|  | Heeji Han | HNU | sarhakor@naver.com |  |  |
|  | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
|  | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
|  | Aram Baek | HNU | aram98123@naver.com |  |  |
|  | Alexey Filippov | Huawei | alexey.filippov@huawei.com |  |  |
|  | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  |  |
|  | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  |  |
|  | Yin Zhao | Huawei | yin.zhao@huawei.com |  |  |
|  | Han Gao | Huawei | han.gao@huawei.com |  | X |
|  | Semih Eselink | Huawei | semih.esenlik@huawei.com |  |  |
|  | Jianle Chen | Huawei | jianle.chen@huawei.com |  |  |
|  | Rahul Vanam | InterDigital | rahul.vanam@interdigital.com |  |  |
|  | Po-Han Lin | ITRI | pohan@itri.com |  |  |
|  | Ching-Chieh Lin | ITRI | JackLin@itri.com |  |  |
|  | Sriram Sethuraman | Ittiam Systems Pvt. Ltd. | sriram.sethuraman@protonmail.com |  |  |
|  | Kai Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
|  | Yi-Wen Chen | Kwai Inc. | yiwenchen@kwai.com |  |  |
|  | Xianglin Wang | Kwai Inc. | xianglinwang@kwai.com |  |  |
|  | Ling Li | LGE | l.li@lge.com |  |  |
|  | Junghak Nam | LGE | junghak.nam@lge.com |  |  |
|  | Shih-Ta Hsiang | MediaTek | shih-ta.hsiang@mediatek.com | X |  |
|  | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X |  |
|  | Olena Chubach | MediaTek | olena.chubach@mediatek.com |  |  |
|  | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
|  | Mohammed Sarwer | MediaTek | Mohammed.sarwer@mediatek.com |  |  |
|  | Chong Soon Lim | Panasonic | chongsoon.lim@sg.panasonic.com |  |  |
|  | Han Boon Teo | Panasonic | hanboon.teo@sg.panasonic.com |  |  |
|  | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
|  | Ted Hsieh | Qualcomm | thsieh@qti.qualcomm.com |  |  |
|  | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  |  |
|  | Han Huang | Qualcomm | hanhuang@qti.qualcomm.com |  |  |
|  | Yinji Piao | Samsung | yj1003.piao@samsung.com |  |  |
|  | Min Woo Park | Samsung | m.w.park@samsung.com |  |  |
|  | Minsoo Park | Samsung | mss.park@samsung.com |  | X |
|  | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
|  | Dongjae Won | Sejong University | bigdj2002@sju.ac.kr |  |  |
|  | Kiran Misra | Sharp | misrak@sharplabs.com | X |  |
|  | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
|  | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
|  | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
|  | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |
|  | Kenji Kondo | Sony | kenji.kondo@sony.com |  |  |
|  | Fabrice Le Léannec | Technicolor | fabrice.leleannec@technicolor.com |  |  |
|  | Tangi Poirier | Technicolor | tangi.poirier@technicolor.com |  |  |
|  | Fabrice Urban | Technicolor | fabrice.urban@technicolor.com |  |  |
|  | Franck Galpin | Technicolor | franck.galpin@technicolor.com |  |  |
|  | Edouard François | Technicolor | edouard.francois@technicolor.com |  |  |
|  | Xiang Li | Tencent | xlxiangli@tencent.com |  |  |
|  | Meng Xu | Tencent | mengxxu@tencent.com | X |  |
|  | Tao Lin | Tongji University | lintao@tongji.edu.cn |  |  |
|  | Eric Chai | Ubilinx | eric.chai@ublnx.com |  |  |
|  | Weimin Zeng | Ubilinx | weimin.zeng@ublnx.com |  |  |
|  | John Son | WILUS Inc. | john.son@wilusgroup.com |  |  |
|  | Koohyar Minhoo |  | koohyar.minoo@gmail.com |  |  |
|  | Jae Hoon Kim |  | i.jaehoon.kim@icloud.com |  |  |
|  | Dan Grois |  | grois@ieee.org |  |  |
|  | Limin Wang |  | cuiwang@gmail.com |  |  |