| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0028-v2 |
| --- | --- |

| Title: | CE8: Summary Report on Current Picture Referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Karsten Müller Li Wang | Tel: Email: | xiaozhongxu@tencent.com karsten.mueller@hhi.fraunhofer.de wangli7@hikvision.com |
| Source: | CE8 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 8 on current picture referencing. Four tests have been agreed to carry out in CE8 in between JVET-K and JVET-L meetings, to study and evaluate technologies related to current picture referencing. In this report, coding performance and complexity of these tests are reported and analyzed. In particular, test results against 1) VTM-2 anchor 2) VTM-2 + CPR anchor are provided to show the coding efficiency and complexity trade-off of each tool. Crosschecking results for the performed tests are integrated in this contribution.

# Introduction

As is agreed in Ljubljana meeting, the following tests in table 1 are performed in CE8.

Table 1: Summary of tests performed in CE8

| Test | Tester | Doc. | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| 8.1 | G. Venugopal (HHI) | JVET-L0077 | Intra region-based template matching | V. Drugeon ( Panasonic) |
| 8.2 | X. Xu (Tencent) | JVET-L0290 | Current picture referencing with separate trees | X. Zheng (DJI) |
| 8.3 | X. Xu (Tencent) | JVET-L0293 | Current picture referencing with constraints - Allow CPR coded block to be compensated only from the current CTU - Allow CPR coded block to be compensated only from the current CTU and the CTU to its left | G. Venugopal (HHI) Y.-W. Chen (Kwai) |
|  | X. Xu (Tencent) | JVET-L0295 | Current picture referencing with constraints - Exclude the current CTU and the CTU to its left from CPR compensation area - Exclude the current CTU and the CTU to its left from CPR compensation area. In addition, disable all loop-filters - Exclude the current CTU and the two CTUs to its left from CPR compensation area - Exclude the current CTU and the two CTUs to its left from CPR compensation area. In addition, disable all loop-filters | K. Zhang (ByteDance) W. Zhu (Sharp) |
| 8.4 | Withdrawn |  |  |  |

The detailed description of each tools is provided in the next section.

# Description of CE tools

## JVET-L0077: Intra region-based template matching

In intra region-based template matching, the prediction signal for the luma component is given by already reconstructed blocks inside the current picture. The location of the blocks used is specified by integer-precision displacement vectors. However, the displacement vectors are not explicitly coded.  Instead, a region index is transmitted, and the displacement vectors are determined by a defined decoder-side displacement search inside the indicated region. The basic concept of intra region-based template matching is illustrated in Fig. 1 below.

Fig. 1: Illustration of intra template matching search window with 9 regions (A1, …, A5 are 8 pixel).

Note that compared to previous versions, smaller, rectangular and more regions are used in order to reduce memory requirements. In the current setting, all regions are now restricted to 8-pixel width. Thus, the search area size could be reduced. In addition, the maximum number of SSD calculations is now restricted to 9.

A template refers to the samples above and left of the current luma block. The template is 1-sample wide, with worst-case maximum number of SSDs equal to 9.

The template matching intra mode finds the best match for the template of a current block Tc compared to any possible template in the reconstructed picture be Tr by minimizing the sum of squared differences (SSD). Thus, the best template match Tb is the template Tr that gives the minimum SSD error, as described in eq.1. The block associated with Tb is the predictor P.

(eq.1)

The proposed intra RTM mode searches for the best template match within a window in the immediate neighborhood of the block to be predicted. This search window has nine regions as shown in Figure 1. However, at the decoder side only one region is considered. The index information about the region under consideration is transmitted in the bitstream.

If two blocks yield the same SSD, the one that is processed first is considered the better match. Let , and  denote the three best matches, with .  The associated predictor blocks of , and  are denoted by , and , respectively.

The number of predictors used for final prediction is made dependent on the SSD error of the first predictor as following:

Let  .

, if and

= , if (i.e. discards )

= , otherwise (i.e. discards , )

The current Test 8.1 configuration uses the following settings:

- The search window is divided into rectangular regions, where A1 = A2 = A3 = A4 = A5 = 8 luma samples. The worst-case size of the block required for template matching search is 40 x 104 .

- Intra RTM is restricted to all block sizes less than or equal to 32 (for luma as well as chroma).

- The size of the template is 1-sample wide with worst-case maximum number of SSDs as 9.

- The worst-case number of regions added for full RD test is 2.

- Intra RTM is applied to chroma samples through its intra derived mode.

- From the MTS transform list only DST7 (for both horizontal and vertical) is tested and applied for RTM predicted blocks.

- Intra RTM finds three predictors during the template matching search process. However, the number of predictors used for final prediction depends on the SSD error of the first predictor.

For chroma samples, intra RTM is applied through the intra derived mode. There are two cases here: DualITree = 1 and DualITree = 0.

When DualITree = 1, the chroma block (component Cb,) derives the region index from the corresponding luma block and does template matching search for finding its predictors. Cr uses the predictors of Cb.

When DualITree = 0, the chroma block derives the predictors from the corresponding luma block [6]. They are then scaled for final prediction based on the chroma channel format. Thus, there is no template matching search in this case.

This test has further investigated the interaction and coding performance with CPR on. Also, the different parameters of the Intra RTM tool like template width, region size, region shape, etc. where optimized in order to reduce the hardware complexity, These include in particular:

- Width of all regions reduced to 8 pixels

- Shape of all regions restricted to rectangular

- Number of regions has been increased, as regions are smaller

- Number of SSD calculations has been reduced to max. 9

- Number of regions added for full RD check is reduced to max. 2

- The biggest block allowed for intra RTM is 32 x 32

## JVET-L0290: Current picture referencing with separate coding trees

Currently, separate trees are enabled only in I slices. However, when CPR is enabled, the previous I slices are converted to P slices. This test is to enable the use of two tools together in P slice where the current picture is the only reference picture.

When separate coding trees for luma and chroma are used, a chroma CU may be coded in CPR mode, with the chroma block vector of each chroma block derived from the block vector for corresponding collocated luma samples. More specifically, a chroma CU’s corresponding luma samples are checked in terms of their samples coverage by CPR mode. The following methods are proposed to handle different cases when chroma separate coding tree is enabled. For a chroma CU:

- In the case that all the corresponding luma sub-blocks are coded in CPR mode., the current chroma CU can choose to be coded in CPR mode. For each chroma sub-block, its block vector is derived from its corresponding luma block’s block vector, or it can choose from other available modes (intra, PCM, etc.) or split further.

- Otherwise (no all corresponding luma samples of the chroma CU are coded in CPR mode), the current chroma CU will be coded in one of the available modes (intra mode, PCM, etc.), or split further, but not CPR mode.

## JVET- L0293: Current picture referencing with local search range constraints

In JVET-K0076, all the reconstructed area inside current picture are available for referencing (except for some area of the top right CTUs for the consideration of WPP constraints).

In this test, the following sub-tests are performed to constraint the available reference area for CPR compensation:

- The already reconstructed samples in the current CTU is the only allowed reference area

  - 1a: chroma interpolation is used when necessary

  - 1b: chroma interpolation is disabled, chroma block vector is clipped to integer

- The already reconstructed samples in the current CTU and its left CTU are the only allowed reference area

  - 2a: chroma interpolation is used when necessary

  - 2b: chroma interpolation is disabled, chroma block vector is clipped to integer

## JVET- L0295: Current picture referencing with non-local search range constraints

In JVET-K0076, all the reconstructed area inside current picture are available for referencing (except for some area of the top right CTUs for the consideration of WPP constraints).

In this test, the following sub-tests are performed to constraint the available reference area for CPR compensation:

- All available reference area in JVET-K0076 but the current CTU and the CTU to its left are excluded.

- All available reference area in JVET-K0076 but the current CTU and the CTU to its left are excluded, plus all in-loop filters (deblocking, SAO and ALF) are disabled.

- All available reference area in JVET-K0076 but the current CTU and the two CTUs to its left are excluded.

- All available reference area in JVET-K0076 but the current CTU and the two CTUs to its left are excluded, plus all in-loop filters (deblocking, SAO and ALF) are disabled.

# CE results summary

To reduce software implementation efforts on both VTM and BMS platforms, it is agreed that BMS version BMS-2.1rc1 shall be used for these experiments.

- The proposed methods in this CE shall be tested with VTM configurations (without changing the JEM_TOOLS Macro).

- The proposed methods in this CE should be compared with CPR turned on (VTM config + CPR=1) and CPR turned off (VTM config).

The mandatory common test conditions (CTC) as specified in the JVET-K1010 document from this meeting [5] shall be followed to perform the simulations in this CE.

The mandatory test sequences as described in common test conditions [5] shall be used for these tests.

In addition to that, to evaluate the benefit of proposed tools on screen content materials, the sequences in TGM YUV 4:2:0 1080p category from HEVC SCC CTC JCT-VC-U1015 document [6] and class F in common test conditions [5] shall be tested. The details of these sequences are listed in the table below.

Table 3: screen content test sequences used in CE8

| Category | Resolution | Sequence name | fps | Bit depth | Frames to be encoded | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 | 8 8 8 8 | 0-299* 0-599 0-599 0-599 | M M M M | M M M M | O O O O |
| Class F | 832x480 1024x768 1280x720 1280x720 | BasketballDrillText ChinaSpeed SlideEditing SlideShow | 50 30 30 20 | 8 8 8 8 | 0-499 0-499 0-299 0-499 | M M M M | M M M M | O O O O |
| *Note that only the first 300 frames of this sequence are used. M refers to mandatory test in the configuration; O refers to optional test in the configuration |  |  |  |  |  |  |  |  |

The followings are a summary table of the tests in this CE.

Table 4: CE8 test results against VTM anchor

Table 5: CE8 test results against VTM+CPR anchor

# Input contributions

Table 5: input contributions for tests that are evaluated and studied in this CE

| Test | Title | Contribution # |
| --- | --- | --- |
| 8.1 | CE8: Intra Region-based Template Matching (Test 8.1) | JVET-L0077 |
| 8.2 | CE8: CPR mode with dual-tree support (Test CE8.2) | JVET-L0290 |
| 8.3 | CE8: CPR mode with local search ranges (Test CE8.3.1 and CE8.3.2) | JVET-L0293 |
|  | CE8: CPR mode with non local search ranges (Test CE8.3.3, CE8.3.4, CE8.3.5 and CE8.3.6 ) | JVET-L0295 |
|  | Cross-check report of CE8.3.5 and CE8.3.6 | JVET-L0508 |

# Crosscheck reports

Table 6: crosscheck reports that are associated with the tests in this CE.

| Test # | Cross-checker | Software review/ studied? (Y/N) | Any significant inconsistencies between the description and the provided software? | Results available (Full/ Partial) | Resutls matched? (Y/N) | Runtime behavior matched? (Y/N) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE8.1 | V. Drugeon ( Panasonic) | Yes | Some aspects are new compared to JVET-J0039, but are described in JVET-L0077: - 9 regions are now enabled for template matching searching. - Only DST7 or DCT2 can be used when region-based template matching is selected as intra prediction mode. - When performing template matching between the template and candidates within the regions, only some pixels of the template are considered based on a sampling parameter (i.e. template downsampling is performed). In addition, I would like to point out the following: - NSST is disabled when region-based template matching is selected as intra prediction mode, but NSST is not used in VTM. In the first version of the software, when the chroma mode was equal to DC and RTM was used for luma, then RTM was automatically applied to chroma. It came from setting the intra mode to DC when RTM is used for luma, so that the chroma derived mode would also be set to DC. But there was no way to make the difference between DC and DM for chroma. After discussion with the proponents, it was recognised as a bug and the software was modified accordingly. | Partial, for the software version with the chroma bug. Partial for some sequences in all intra configuration for the final software version. | The results exactly match the proponent's results for AI conditions and for RA conditions classes C and D for the version of the software without the bug fix. In addition, the results exactly match for 4 sequences of class A for all intra configuration for the final software version. The second anchor (VTM+CPR) could not be tested due to a lack of computer resources. | Y | The source code for this technology is very heavy (17 files of VTM are modified and IntraPrediction.cpp contains almost 800 lines of additional code). But the provided source code is generally well written and surprisingly easy to follow given the complexity of the technology. |
| CE8.2 | X. Zheng (DJI) | Yes | No | Full | Yes | Yes (for AI and RA) | 1. very minor difference observed in partial sequences which might be related to different OS's behavior. 2. runtime for LB from crosscheck isn't reliable |
| CE8.3.1a | G. Venugopal (HHI) | Yes | No | Full | Yes | Partially Yes | Difference in run-time for the test against VTM |
| CE8.3.1b | G. Venugopal (HHI) | Yes | No | Full | Yes | Partially Yes |  |
| CE8.3.2a | Y.-W. Chen (Kwai) | Yes | No | Partial (Class SCC not checked yet) | Yes | Yes |  |
| CE8.3.2b | Y.-W. Chen (Kwai) | Yes | No | Partial (Class SCC not checked yet) | Yes | Yes |  |
| CE8.3.3 | K. Zhang (ByteDance) | Yes | No | Partial (Class F and SCC not checked yet) | Yes | Yes | AI results exactly match. Very minor difference (about 0.01%) on RA/LB tests. Anchor results also have a very slight gap. So difference may come from rounding error |
| CE8.3.4 | K. Zhang (ByteDance) | Yes | No | Partial (Class F and SCC not checked yet) | Yes | Yes | AI results exactly match. Very minor difference (about 0.01%) on RA/LB tests. Anchor results also have a very slight gap. So difference may come from rounding error |
| CE8.3.5 | W. Zhu (Sharp) | Yes | No | Partial | Yes | Partially Yes | 1. The run-time in the cross-check report is not very reliable due to our homogeneous cluster 2. The post data processing in RA of are different from what the proponent have, which resulted in a very minor difference(less than 0.02%) for some sequences. |
| CE8.3.6 | W. Zhu (Sharp) | Yes | No | Partial | Yes | Partially Yes |  |

# Related contributions

Table 6: Related contributions and crosschecks for this CE

| Title | Contribution # |
| --- | --- |
| Non-CE8: Rotate intra block copy | JVET-L0041 |
| Non-CE8: Block vector predictor for IBC | JVET-L0159 |
| Cross Check report of JVET-L0159: Non-CE8: Block vector predictor for IBC | JVET-L0472 |
| CE8-related: CPR with local search range optimization | JVET-L0297 |
| Cross check of JVET-L0297 (CE8-related: CPR mode with local search range optimization) | JVET-L0518 |
| CE8-related: CPR with merge mode improvements | JVET-L0299 |
| CE8-related: Restrictions for the search area of the IBC blocks in CPR | JVET-L0404 |