| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0029_v1 |
| --- | --- |

| Title: | CE9: Summary Report on Decoder Side MV Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Semih Esenlik Yi-Wen Chen Fangdong Chen | Tel: Email: | semih.esenlik@huawei.com yiwenchen@kwai.com chenfandong@hikvision.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The tools in the scope of this CE include bi-directional optical flow, template matching and bilateral matching based techniques for motion vector derivation and refinement at the decoder side.

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9.1: Partial Usage of Refined MVs (7 tests)

- CE9.2: DMVR Design (15 tests)

This report summarises the status of each experiment. Crosscheck results are integrated in the document.

# Introduction

The description of Core Experiment 9 (CE9) on Decoder Side Motion Vector Derivation [1] has been finalized on 8th of August 2018. A total 23 tests among 2 sub-tests are defined in CE9 description. After the finalization of the CE description document, 1 test was withdrawn by proponent, therefore results of 22 tests are reported in CE summary report.  The goal of this Core Experiment (CE) is to further investigate the techniques of decoder side motion vector derivation in VTM. Tested solutions for decoder side motion vector derivation techniques are evaluated according to BD-rate gain, runtime and additionally according to tool questionnaire that is developed for worst case complexity assessment.

# Experimental Conditions and Evaluation

## Software

BMS2.1rc1 software has been used (Commit: 5b93362a62b8e8a54f0a12069cd729c94defe7e8), due to the fact that the release of BMS2.1 software was delayed. The decision to use the BMS2.1rc1 software was announced in the CE9 email reflector on 13th of September 2018, and a reminder email has been sent on 18th of September. It is noted that between BMS2.1 and BMS2.1rc1 the following difference exists:

- Merge Request 111: fix maximum ternary split size as reported in issue #87

## Anchors

- Anchor for Mandatory simulations: BMS2.1rc1 with VTM encoder configuration

- Test condition for mandatory simulations: Anchor + DMVR on

- The simulation results with BMS encoder configuration has been made optional due to the short amount of time between the integration start date (13th of September 2018) and document upload deadline (24th of September 2018)

## Test Sequences, Bit Rates and Coding Conditions

CTC as described in [2].

## Reporting of the Results

All test results and crosscheck results are to going to be uploaded to git servers. All crosscheck results will be included in the CE summary report.

## Evaluation of CE Results

The following requirements and the complexity characteristics are reported as part of the CE summary report.

### Requirements

- Reference block pre-fetches shall be guaranteed at least for a region of 128x128

- Refined motion vectors of left 128x128 CTU shall not be used for reference data pre-fetch by the DMVR of the current CTU

- Reconstructed samples of neighboring blocks (except for the top CTU boundary neighboring blocks) shall not be used for template matching of the DMVR

- Wavefront parallelism shall be possible. Refined motion vectors or reconstructed samples from the wavefront diagonal or from the right of the wavefront diagonal shall not be used (in particular the top-right CTU shall not be used).

### Worst case latency characteristics

- Description of  processing steps (parallel/sequential)

- Description of data dependency within a 128x128 region of DMVR

### Memory storage

- Intermediate buffers

- Description of the usage of refined motion vectors after consumed by MC

### Complexity in terms of operations

- Number of multiplies and additions

- Number of comparisons

- Number of on-chip memory accesses

### Memory BW increase

- Description of reference block sizes

### Coding efficiency measurements

- Measure impacts on bitrate/PSNR using provided data (4 QP points, BD-rate).

### Runtime measurement

- The encoding and decoding times must be provided using the SSE42 SIMD setting (both anchor and test).

## Complexity Assessment of DMVR in BMS2.1

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (DCTIF) 1.b Calculate block averages 2. perform 9 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (DCTIF) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (DCTIF) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5. copy final prediction |
| --- | --- |
| Data dependency within 128x128 region | none |
| Intermediate Buffers | buffer 1: 2 × (w + 4) × (h + 4) buffer 2: 2 × (w + 1) × h buffer 3: 2 × w × (h + 1) |
| Usage of refined MVs | Motion compensation and temporal MV prediction |
| Number of operations for interpolation* | Adds: 10906 Mult: 12464 (including final motion compensation) |
| Number of operations for MRSAD* | Adds: 4032 |
| Additional operations* | DMVR application condition: 5 “if” checks MRSAD cost comparisons: 18 comparisons Final MV averaging: 64 additions |
| Smallest block applying DMVR | 4x4 |
| On Chip memory accesses | 1. Read/Write for 1a and 1b 2. Read for 2 3. Read for 3a 4. Read/Write for 3b and 3c 5. Read for 4a and 4b Total: 5 Reads and 2 Writes (excluding final MC) |
| External memory BW increase* | 60% |

*For 8x8 reference block.

# CE9.1: Partial Usage of Refined MVs

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.1.1 | The refined MV is used for de-blocking and temporal MV prediction, and not for spatial prediction. | S. Esenlik (Huawei) | D. Luo (InterDigital) |
| CE9.1.2 | Unrefined MVs and refined MVs are used for spatial MV prediction with the same CTU and across CTU boundaries. Refined MVs are used for temporal MV prediction and unrefined MV are used for de-blocking. | D. Luo (InterDigital) | M. Xu (Tencent) |
| CE9.1.3 | Unrefined MVs and refined MVs are used for spatial MV prediction with the same CTU row and across CTU top boundaries. Refined MVs are used for temporal MV prediction and unrefined MV are used for de-blocking. | D. Luo (InterDigital) | C.-C. Chen (Qualcomm) |
| CE9.1.4 | Top CTB-row refined MV availability rules. - Lag=2 – use top-left and top CTB refined MVs for AMVP or as refinement center | S. Sethuraman (Ittiam) | X. Chen (HiSilicon) |
| CE9.1.5 | The refined MV is used for de-blocking and temporal MV prediction, and not for spatial prediction. Or using refined mv with some limitation for spatial or de-blocking prediction | X. Chen (HiSilicon) | C.-C. Chen (Qualcomm) |
| CE9.1.6 | Refined MV is used for MC and original MV is used for MV prediction and deblocking. | C.-C. Chen (Qualcomm) | D. Luo (InterDigital) |
| CE9.1.7 | The CE requirements will be fulfilled. In addition, the refined MV in previous CU is not used as predictor. - The MV in previous CU is marked as unavailable - The MV in previous CU is not marked as unavailable but only unrefined MV can be used and not compressed | M. Xu (Tencent) | S. Esenlik (Huawei) |

## Test Results

|  |  |  |  | VTM configuration of BMS |  |  |  |  | Crosscheck |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE Subtest | # | Tester | Crosschecker | Y | U | V | EncT | DecT | EncT | DecT |
| Subtest 0 | DMVR in BMS2.1 | S. Esenlik | C.-C. Chen | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| Subtest 1 | CE9.1.1 | S. Esenlik | D. Luo | -1.66% | -1.72% | -1.85% | 104% | 119% |  |  |
|  | CE9.1.2 | D. Luo | M. Xu | -1.96% | -1.94% | -2.08% | 104% | 119% |  |  |
|  | CE9.1.3 | D. Luo | C.-C. Chen | -1.83% | -1.86% | -2.00% | 104% | 117% |  |  |
|  | CE9.1.4 | S. Sethuraman | X. Chen | -1.86% | -1.89% | -1.99% | 102% | 113% |  |  |
|  | CE9.1.5 | X. Chen | C.-C. Chen |  |  |  |  |  |  |  |
|  | CE9.1.6 | C.-C. Chen | D. Luo |  |  |  |  |  |  |  |
|  | CE9.1.7 | M. Xu | S. Esenlik | -1.80% | -1.84% | -1.94% | 104% | 120% |  |  |

## Detailed Description of the Tests

### CE9.1.1 as in JVET_L0215 (Huawei)

The test CE9.1.1 is designed to test the effect of using refined motion vectors for deblocking process. In the current BMS2.1, the refined motion vectors after DMVR are used only in the temporal motion vector prediction. According to test CE9.1.1, the refined MVs are used also for deblocking process.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.1.1 | -1.66% | -1.72% | -1.85% | 104% | 119% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | yes, refined MV not used in spatial MV prediction |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Motion compensation, deblocking and temporal MV prediction |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.1.2 as in JVET_L0253 (InterDigital)

The proposed method uses the unrefined MVs of current DMVR CU to predict MVs of it neighboring CUs that are in the same CTU as the current CU. But, for the neighboring CUs that are not in the same CTU as current CU, the refined MVs of the current CU are used as the predictor to achieve a better MV prediction accuracy. Additionally, the refined MVs and the unrefined MVs are used for the TMVP/ATMVP and deblocking filter, respectively. Figure 1 illustrates idea of this solution. As shown in Figure 1, with this method, the decoding of multiple inter CUs inside one CTU can be performed in parallel while the decoding of the CUs of the current CTU still depends on that of neighboring CTUs. In this solution, if a CU is located on the bottom or right boundary of the CTU, then both refined and unrefined MVs need to be stored for spatial MV prediction. For all other CUs, only the unrefined MVs need to be stored for spatial MV prediction.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.1.2 | -1.96% | -1.94% | -2.08% | 104% | 119% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | Refined MVs are used in spatial MV prediction across CTU boundaries |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | motion compensation, spatial MV prediction across CTU boundaries and temporal MV prediction |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.1.3 as in JVET_L0254 (InterDigital)

The proposed method uses the unrefined MVs of one DMVR CU to predict the MVs of its neighboring CU that is in the same CTU row. For the neighboring CUs located in different CTU rows, the refined MVs are used for spatial MV prediction. By this way, multiple CUs within the same CTU row can be decoded in parallel while the decoding of the CUs on the top boundary of one CTU is still dependent on that of its top neighboring CTUs. Figure 1 illustrates idea of this solution. As illustrated in Figure 1, it only needs to store both refined and unrefined MVs for the CUs located on the bottom boundary of one CTU.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.1.3 | -1.83% | -1.86% | -2.00% | 104% | 117% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | Refined MVs are used in spatial MV prediction across CTU row boundaries |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | motion compensation, spatial MV prediction across CTU row boundaries and temporal MV prediction |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.1.4 as in JVET_LXXXX (Ittiam)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.1.4 | -1.86% | -1.89% | -1.99% | 102% | 113% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | Yes. Refined MVs of neighbor CUs in 1. top-left and top CTUs are considered as available 2. left CTU are considered as unavailable 3. current CTU are considered as unavailable |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Motion compensation, from neighbor Cus in top left and top CTUs for merge list and AMVP, deblocking and temporal MV prediction |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | BMS2.1 operations + 6 checks for Top spatial predictors CTU boundary check |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.1.5 as in JVET_LXXXX (HiSilicon)

### CE9.1.6 as in JVET_L0243 (Qualcomm)

Only non-refined motion field is available for later use, including spatial MV prediction, temporal MV prediction and BS calculation in deblocking filter.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.1.6 | -1.48% | -1.64% | -1.75% | 104% | 119% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | yes, refined MV not used in spatial MV prediction |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Only motion compensation of current block, not for DBF and TMVP/ATMVP |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.1.7 as in JVET_L0311 (Tencent)

a constraint region is signaled. The refined MV in the constraint region cannot be used as spatial MVP for the neighboring blocks. The refined MV outside of the constraint region may be used as spatial MVP. In this contribution, the constraint region includes the current CTU, the left CTU, and the top-right CTU. In addition, the refined MV from immediate previous CU cannot be used as MVP in current block. It is either marked as unavailable or the unrefined MV (MV_init) is used as MVP for the current block.

Test 1: MV in previous CU not marked as unavailable.

Test 2: MV in previous CU is marked as unavailable.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| Test1 | -1.80% | -1.84% | -1.94% | 104% | 120% |  |  |
| Test2 | -0.87% | -0.79% | -0.86% | 108% | 121% |  |  |

#### Complexity Assessment

| CE requirements are satisfied? | yes, refined MV from current, left, and top-right CTUs and previous CU are not used in spatial MVP |
| --- | --- |
| Description of processing steps | Same as BMS2.1 |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Motion compensation, spatial MV prediction (only top and top-left CTUs) and temporal MV prediction |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

# CE9.2: DMVR Design

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.2.1 | SAD/MRSAD computation on integer samples without interpolation | S. Esenlik (Huawei) | F. Chen (Hikvision) |
| CE9.2.2 | Padding applied to eliminate memory extension | S. Esenlik (Huawei) | K. Abe (Panasonic) |
| CE9.2.3 | CE9.1.1 and CE9.1.2 combined. | S. Esenlik (Huawei) | C.-C. Chen (Qualcomm) |
| CE9.2.4 | A new search pattern for refinement - DCTIF - Bilinear interpolation | S. Sethuraman (Ittiam) | F. Chen (Hikvision) |
| CE9.2.5 | Use of parametric error surface for estimation of sub-pixel distance offsets using bilinear interpolation. - Bilateral matching with symmetric mirroring - Averaged bilateral template based independent sub-pixel error surface for L0 and L1 | S. Sethuraman (Ittiam) | D. Luo (InterDigital) |
| CE9.2.7 | Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions | D. Luo (InterDigital) | S. Esenlik (Huawei) |
| CE9.2.8 | Bilateral template matching is applied on the current DMVR configuration in BMS. | X. Chen (HiSilicon) | S. Sethuraman (Ittiam) |
| CE9.2.9 | 1. Restrict DMVR to different block sizes. 2. Calculate SAD for every two rows. | H. Liu (Bytedance) | S. Esenlik (Huawei) |
| CE9.2.10 | Generate the template with the neighboring MVs under different test conditions | F. Chen (Hikvision) | S. Sethuraman (Ittiam) |
| CE9.2.11 | Combine proposed low latency template matching with low latency DMVR | F. Chen (Hikvision) | Y. Li (USTC) |
| CE9.2.12 | withdrawn |  |  |
| CE9.2.13 | - Test early-termination method based on MV difference between merge candidates. - Test shorter interpolation filters (2-, 4-tap) for motion compensation | C.-C. Chen (Qualcomm) | S. Esenlik (Huawei) |
| CE9.2.14 | - Use 4-tap filter for final motion compensation - Use 6-tap filter for final motion compensation | M. Xu (Tencent) | Y. Li (USTC) |

## Specifications of Joint Tests

| CE9.2.15 | - Combination of 9.2.2, 9.2.7 and 9.2.13. - Bilinear interpolation applied for search. - Padding is applied to eliminate memory extension. - Test early-termination method based on MV difference between merge candidates. - Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions | S. Esenlik (Huawei) | H. Liu (Bytedance) |
| --- | --- | --- | --- |
| CE9.2.16 | - Combination of 9.2.2, 9.2.7, 9.2.9 and 9.2.13. - Bilinear interpolation applied for search. - Padding is applied to eliminate memory extension. - Test early-termination method based on MV difference between merge candidates. - Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions - Restrict DMVR to different block sizes. - Calculate SAD for every two rows. | S. Esenlik (Huawei) | M. Xu (Tencent) |
| CE9.2.6 | Study combinations of CE9.1.4 and CE9.2.5 with other proposed complexity reduction methods (on top of CE9.2.16) | S. Sethuraman (Ittiam) | C.-C. Chen (Qualcomm) |

## Test Results

|  |  |  |  | VTM configuration of BMS |  |  |  |  | Crosscheck |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE Subtest | # | Tester | Crosschecker | Y | U | V | EncT | DecT | EncT | DecT |
| Subtest 0 | DMVR in BMS2.1 | S. Esenlik | C.-C. Chen | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| Subtest 2 | CE9.2.1 | S. Esenlik | F. Chen | -1.39% | -1.53% | -1.61% | 104% | 113% |  |  |
|  | CE9.2.2 | S. Esenlik | K. Abe | -1.64% | -1.73% | -1.84% | 105% | 118% | 104% | 120% |
|  | CE9.2.3 | S. Esenlik | C.-C. Chen | -1.39% | -1.54% | -1.65% | 104% | 113% |  |  |
|  | CE9.2.4 | S. Sethuraman | F. Chen | -1.63% | -1.70% | -1.83% | 103% | 115% |  |  |
|  | CE9.2.5 | S. Sethuraman | D. Luo | -1.68% | -1.77% | -1.90% | 102% | 109% |  |  |
|  | CE9.2.7 | D. Luo | S. Esenlik | -1.64% | -1.72% | -1.81% | 103% | 114% |  |  |
|  | CE9.2.8 | X. Chen | S. Sethuraman |  |  |  |  |  |  |  |
|  | CE9.2.9 | H. Liu | S. Esenlik |  |  |  |  |  |  |  |
|  | CE9.2.10 | F. Chen | S. Sethuraman |  |  |  |  |  |  |  |
|  | CE9.2.11 | F. Chen | Y. Li |  |  |  |  |  |  |  |
|  | CE9.2.12 | Withdrawn |  |  |  |  |  |  |  |  |
|  | CE9.2.13 | C.-C. Chen | S. Esenlik |  |  |  |  |  |  |  |
|  | CE9.2.14 | M. Xu | Y. Li | -1.61% | -1.71% | -1.82% | 103% | 119% |  |  |
| Joint Tests | CE9.2.15 | S. Esenlik | H. Liu | -1.65% | -1.74% | -1.88% | 103% | 111% |  |  |
|  | CE9.2.16 | S. Esenlik | M. Xu | -1.59% | -1.70% | -1.78% | 102% | 110% |  |  |
|  | CE9.2.6 | S. Sethuraman | C.-C. Chen | -1.89% | -1.89% | -2.00% | 101% | 108% |  |  |

## Detailed Description of the Tests

### CE9.2.1 as in JVET_L0215 (Huawei)

In CE test 9.2.1, the 8-tap interpolation filter is eliminated and instead integer reference samples are used for MRSAD comparison process of DMVR. The integer reference samples are obtained by rounding the initial motion vector to the nearest integer sample coordinate before DMVR. This means that no interpolation filtering is necessary until the final refined motion vectors are obtained. Only at this step (after the final refined MV is obtained), 8-tap interpolation filtering is applied to obtain the final motion compensation results. It is noted that the final motion vector can still have fractional sample resolution, since the MV rounding operation is reverted before final motion compensated prediction.

The process is elaborated in the following figure:

Figure X: Application of rounding before MRSAD search and inversion of the rounding operation in order to obtain refined MV.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.1 | -1.39% | -1.53% | -1.61% | 104% | 113% |  |  |

#### Complexity Assessment

| Description of processing steps | 1. Calculate block averages 2.a perform 9 point MRSAD computation (integer refinement) 2.b perform 5 point MRSAD computation (integer refinement) 2.c perform 4 point MRSAD computation (half-pel refinement) 3. Perform Final motion compensation |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | none |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 2576, Mult: 2944 (same as regular bi-pred) |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | 1. Read for 1 2. Read for 2a, 2b and 2c (total 2 reads) |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.2 as in JVET_L0215 (Huawei)

In this test padding is applied in order to extend the reference sample block that is pointed by the initial motion vector. If the size of the coding block are given by “w” and “h”, then it is assumes that a block of size w+7 and h+7 is retrieved from the reference picture buffer. The retrieved buffer is then extended by 2 samples in each direction by repetitive sample padding using the nearest sample. Afterwards the extended reference sample block is used to generate the final prediction once the refined motion vector is obtained (which can deviate from the initial motion vector 2 samples in each direction).

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.1 | -1.64% | -1.73% | -1.84% | 105% | 118% |  |  |

#### Complexity Assessment

| Description of processing steps | Same as BMS2.1 |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | 0% (same as bi-pred) |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.3 as in JVET_L0215 (Huawei)

The reference sample padding (CE9.2.2) and integer DMVR (CE9.2.1) are applied together.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.1 | -1.39% | -1.54% | -1.65% | 104% | 113% |  |  |

#### Complexity Assessment

| Description of processing steps | 1. Calculate block averages 2.a perform 9 point MRSAD computation (integer refinement) 2.b perform 5 point MRSAD computation (integer refinement) 2.c perform 4 point MRSAD computation (half-pel refinement) 3. Perform Final motion compensation |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | none |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 2576, Mult: 2944 (same as regular bi-pred) |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | 1. Read for 1 2. Read for 2a, 2b and 2c (total 2 reads) |
| External memory BW increase* | 0% (same as bi-pred) |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.4 as in JVET_LXXXX (Ittiam)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.4 | -1.63% | -1.70% | -1.83% | 103% | 115% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (DCTIF) 1.b Calculate block averages 2. perform 5 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (DCTIF) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (DCTIF) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5. copy final prediction |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Adds: 3264 |
| Additional operations* | (1) 5 if checks (2) 14comparisons (3) 64 additions |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.5 as in JVET_LXXXX (Ittiam)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.5 | -1.39% | -1.54% | -1.65% | 104% | 113% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (Bilinear) 1.b Calculate block averages 2. perform 5 point MRSAD computation 3.a perform 8 point MRSAD computation 3.b.perform error surface to compute sub-pixel movement 4. Perform final MC (DCTIF) |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | buffer: 2x(w+4) x(h+4) |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 3176, Mult: 4144 |
| Number of operations for MRSAD* | Adds: 2784 |
| Additional operations* | DMVR condition: 5 if checks MRSAD comparisons: 18 comparisons Final MC averaging: 64 additions Error Surface: 40 operations |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | 1. Read/Write for 1a and 1b 2. Read for 2 3. Read for 3a |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.7 as in JVET_LXXXX (InterDigital)

The sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the initial MVs of the current CU is calculated. If the SAD is no larger than a predefined threshold, i.e., 2BDepth-9 per sample, the DMVR is skipped; otherwise, the DMVR is still applied to refine the two motion vectors of the current block.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.1 | -1.64% | -1.72% | -1.81% | 103% | 114% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (DCTIF) 1.b Calculate block averages 1.c Calculate SAD cost for center point 2. perform 9 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (DCTIF) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (DCTIF) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5. copy final prediction |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | 128 additions |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.8 as in JVET_LXXXX (HiSilicon)

### CE9.2.9 as in JVET_L0267 (ByteDance)

This contribution tests two modifications on Decoder Side Motion Vector Refinement (DMVR) in the benchmark set (BMS): 1) disable DMVR for small blocks like 4x4, 4x8, 8x4 or 8x8; 2) calculate partial Mean Removed Sum of Absolute Differences (MRSAD) instead of full MRSAD. Every second row of block samples are used to calculate MRSAD cost.

Test1: Partial MRSAD, calculate MRSAD using every second row of block samples.

Test2: Disable for 4x4.

Test3: Disable 4x4, 4x8 and 8x4

Test4: Disable 4x4, 4x8, 8x4 and 8x8

#### Experimental results

The results are presented w.r.t the anchor “BMS-2.1rc1 + VTM configuration + DMVR on”.

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 0.02% | 0.04% | 0.04% | 99% | 97% |  |  |
| Test 2 | -0.01% | -0.03% | -0.02% | 100% | 100% |  |  |
| Test 3 | -0.01% | -0.03% | -0.02% | 100% | 100% |  |  |
| Test 4 | 0.06% | 0.08% | 0.10% | 98% | 97% |  |  |
| Test1+ test2 | 0.02% | 0.03% | 0.00% | 98% | 97% |  |  |
| Test1+ test3 | 0.03% | 0.05% | 0.07% | 98% | 97% |  |  |
| Test1+ test4 | 0.08% | 0.09% | 0.12% | 98% | 95% |  |  |

#### Complexity Assessment

| Description of processing steps | Same as BMS2.1 |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Same as BMS2.1 |
| Number of operations for MRSAD* | 2304 additions (test1) |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | 8x4 (test2), 8x8(test3) , 8x16 (test 4) |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | Same as BMS2.1 |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.10 as in JVET_LXXXX (Hikvision)

### CE9.2.11 as in JVET_LXXXX (Hikvision)

### CE9.2.13 as in JVET_L0244 (Qualcomm)

Test 1:

If the MV difference between the selected merge candidate and any of the previous ones in the same merge list is less than a pre-defined threshold (that is, ¼-, ½- and 1-pixel-wide intervals for CUs with less than 64 pixels, less than 256 pixels and at least 256 pixels, respectively), DMVR is disabled.

Test 2.1: Test 1 + bilinear filters for search + following interpolation filters for final MC:

- Luma 8-tap filters for d=0 and 2-tap filters for d>0.

- Chroma 4-tap filters for c=0 and 2-tap filters for c>0.

Test 2.2: Test 1 + bilinear filters for search + following interpolation filters for final MC:

- Luma 8-tap filters for d=0 and 4-tap filters for d>0.

- Chroma 4-tap filters for c=0 and 2-tap filters for c>0.

Test 2.3: Test 1 + bilinear filters for search + following interpolation filters for final MC:

- Luma 8-tap filters for d=0, 6-tap filters for d=1 and 4-tap filters for d>1.

- Chroma 4-tap filters for c=0 and 2-tap filters for c>0.

Test 2.4: Test 1 + bilinear filters for search + following interpolation filters for final MC:

- Luma 8-tap and 4-tap filters for horizontal interpolation when dx=0 and dx>0, respectively, and 8-tap and 4-tap filters for vertical interpolation when dy=0 and dy>0, respectively.

- Chroma 4-tap and 2-tap filters for horizontal interpolation when cx=0 and cx>0, respectively, and 4-tap and 2-tap filters for vertical interpolation when cy=0 and cy>0, respectively.

“d” and “c” depend on the difference between refined and non-refined MV.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| Test 1 | -1.66% | -1.73% | -1.88% | 102% | 118% |  |  |
| Test2.1 | -1.29% | -1.40% | -1.41% | 102% | 113% |  |  |
| Test2.2 | -1.66% | -1.36% | -1.28% | 101% | 112% |  |  |
| Test2.3 | -1.63% | -1.38% | -1.34% | 102% | 114% |  |  |
| Test2.4 | -1.68% | -1.56% | -1.57% | 102% | 113% |  |  |

#### Complexity Assessment

| Description of processing steps | XXXX |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | XXXX |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | XXXX |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | XXXX |
| External memory BW increase* | XXXX |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.14 as in JVET_L312 (Tencent)

Test 1: Bilinear filter is applied for search. 8-tap filter is applied for final MC if refined MV is equal to initial MV. Otherwise 4-tap interpolation filter is applied.

Test 2: 4-tap filter is applied for search. 8-tap filter is applied for final MC if refined MV is equal to initial MV. Otherwise 4-tap interpolation filter is applied.

Test 3: 6-tap filter is applied for search. 8-tap filter is applied for final MC if refined MV is equal to initial MV. Otherwise 6-tap interpolation filter is applied.

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| Test 1 | -1.61% | -1.71% | -1.82% | 103% | 119% |  |  |
| Test2 | -1.64% | -1.72% | -1.86% | 105% | 121% |  |  |
| Test3 | -1.63% | -1.74% | -1.84% | 105% | 124% |  |  |

#### Complexity Assessment

The following table is generated for test 1.

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (bilinear) 1.b Calculate block averages 2. perform 9 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (bilinear) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (bilinear) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5.a If mv_final == mv_init: apply 8-tap filter 5.b If mv_final != mv_init: apply 4-tap filter |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | Same as BMS2.1 |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 3786, Mult: 5364 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | Same as BMS2.1 |
| Smallest block applying DMVR | Same as BMS2.1 |
| On Chip memory accesses | Same as BMS2.1 |
| External memory BW increase* | 0% |

*For 8x8 reference block.

#### Crosschecker Comments

## Detailed Description of the Joint Tests

### CE9.2.15 as in JVET_L0163 (Huawei, Qualcomm, Interdigital, Bytedance)

Following changes are proposed:

- Apply reference padding as in to eliminate memory BW increase (CE9.2.2).

- Bilinear interpolation is applied for refinement search (tested in CE9.2.5, CE9.2.13, CE9.2.14).

- DMVR is disabled for 4x4 blocks (CE9.2.9).

- DMVR is skipped if selected merge candidate is similar to previous candidates in the list (CE9.2.13)

- DMVR is skipped if initial SAD cost is smaller than 2bitdepth – 9 (CE9.2.7)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.15 | -1.65% | -1.74% | -1.88% | 103% | 111% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (bilinear) 1.b Calculate SAD cost for center point 1.c Calculate block averages 2. perform 9 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (bilinear) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (bilinear) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5. Perform final MC (DCTIF) |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | buffer 1: 2x(w+1)x(h) buffer 2: 2x(w)x(h+1) |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 3784, Mult: 5360 |
| Number of operations for MRSAD* | Same as BMS2.1 |
| Additional operations* | DMVR condition: 5 if checks MRSAD comparisons: 18 comparisons Final MC averaging: 64 additions SAD early exit: 128 addition 1 comparison MVD early exit: 20 addition, 30 comparisons |
| Smallest block applying DMVR | 8x4 |
| On Chip memory accesses | same as BMS2.1 |
| External memory BW increase* | 0% (same as bi-pred) |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.16 as in JVET_ L0163 (Huawei, Qualcomm, Interdigital, Bytedance)

Following changes are proposed:

- Apply reference padding as in to eliminate memory BW increase (CE9.2.2).

- Bilinear interpolation is applied for refinement search (tested in CE9.2.5, CE9.2.13, CE9.2.14).

- DMVR is disabled for blocks 8x4 and 4x4 (CE9.2.9).

- MRSAD is calculated using every second line (CE9.2.9).

- DMVR is skipped if selected merge candidate is similar to previous candidates in the list (CE9.2.13)

- DMVR is skipped if initial SAD cost is smaller than 2bitdepth – 9 (CE9.2.7)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.16 | -1.59% | -1.70% | -1.78% | 102% | 110% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (bilinear) 1.b Calculate SAD cost for center point 1.c Calculate block averages 2. perform 9 point MRSAD computation 3.a perform 5 point MRSAD computation 3.b. Generate L0 and L1 half pel predictions for [w+1, h] blocks (bilinear) 3.c. Generate L0 and L1 predictions for [w, h+1] blocks (bilinear) 4.a. Perform 2 MRSAD on first half-pel plane 4.b. Perform 2 MRSAD on second half-pel plane 5. Perform final MC (DCTIF) |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | buffer 1: 2x(w+1)x(h) buffer 2: 2x(w)x(h+1) |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 3784, Mult: 5360 |
| Number of operations for MRSAD* | 2304 |
| Additional operations* | DMVR condition: 5 if checks MRSAD comparisons: 18 comparisons Final MC averaging: 64 additions SAD early exit: 128 addition 1 comparison MVD early exit: 20 addition, 30 comparisons |
| Smallest block applying DMVR | 8x8 |
| On Chip memory accesses | same as BMS2.1 |
| External memory BW increase* | 0% (same as bi-pred) |

*For 8x8 reference block.

#### Crosschecker Comments

### CE9.2.6 as in JVET_LXXXX (Ittiam)

Following changes are proposed:

- Apply reference padding as in to eliminate memory BW increase (CE9.2.2).

- Bilinear interpolation is applied for refinement search (tested in CE9.2.5, CE9.2.13, CE9.2.14).

- DMVR is disabled for blocks 8x4 and 4x4 (CE9.2.9).

- MRSAD is calculated using every second line (CE9.2.9).

- DMVR is skipped if selected merge candidate is similar to previous candidates in the list (CE9.2.13)

- DMVR is skipped if initial SAD cost is smaller than 2bitdepth – 9 (CE9.2.7)

- Half-pel DMVR step is removed, instead a parametric equation is used to refine the MV(CE9.2.5)

- Refined MV from top and top-left CTU is allowed to be used in spatial prediction (CE9.1.4)

- Refined MV is used in deblocking to eliminate line memory arising from previous modification (CE9.1.4, CE9.1.1)

#### Experimental results

|  | Y | U | V | EncT | DecT | CC EncT | CC DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR in BMS2.1 | -1.63% | -1.73% | -1.85% | 104% | 121% |  |  |
| CE9.2.6 | -1.89% | -1.89% | -2.00% | 101% | 108% |  |  |

#### Complexity Assessment

| Description of processing steps | 1.a Generate L0 and L1 predictions for [w+4, h+4] blocks (bilinear) 1.b Calculate SAD cost for center point 1.c Calculate block averages 2. Perform 5 point MRSAD computation 3.a Perform 8 point MRSAD computation 4. Perform error surface to compute sub-pixel movement 5. Perform final MC (DCTIF) |
| --- | --- |
| Data dependency within 128x128 region | Same as BMS2.1 |
| Intermediate Buffers | buffer: 2x(w+4) x(h+4) |
| Usage of refined MVs | Same as BMS2.1 |
| Number of operations for interpolation* | Adds: 3176, Mult: 4144 |
| Number of operations for MRSAD* | 1536 |
| Additional operations* | DMVR condition: 5 if checks MRSAD comparisons: 18 comparisons Final MC averaging: 64 additions SAD early exit: 128 addition 1 comparison MVD early exit: 20 addition, 30 comparisons Error Surface: 40 operations |
| Smallest block applying DMVR | 8x8 |
| On Chip memory accesses | 1. Read/Write for 1a and 1b 2. Read for 2 3. Read for 3a |
| External memory BW increase* | 0% (same as bi-pred) |

*For 8x8 reference block.

#### Crosschecker Comments

# Recommendations

- Review ALL CE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing CE results.

# Acknowledgement

- Thanks to all proponents for providing CE s/w in time with proper announcement and explanation about modification in the CE9 reflector.

- Thanks to all volunteers for providing the comprehensive cross-checks.

# References

- “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-K1029, S. Esenlik, Y.-W. Chen, F. Chen

- “JVET common test conditions and software reference configurations”, JVET-K1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.