| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0041-v1 |
| --- | --- |

| Title: | Rotate Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhengdong Zhang, Vivienne Sze 77 Massachusetts Ave Cambridge MA 02139 USA | Tel: Email: | +1-617-324-7352 sze@mit.edu |
| Source: | Massachusetts Institute of Technology |  |  |

_____________________________

# Abstract

This contribution proposes a method called rotate intra block copy, which extends the intra block copy technique by making the block matching process invariant to rotation. HEVC intra prediction plus rotate intra block copy gives an average of 20% reduction in residual energy (i.e., prediction error) compared to HEVC intra prediction plus intra block copy.  As the motion vector correlation in rotate intra block copy is different from the intra block copy, a new method of motion vector coding is presented. The impact of angular resolution on residual energy reduction is also evaluated. The reduction in residual energy translates to BD-rate change of -3.4% over HEVC intra prediction plus intra block copy for both screen content and camera-captured gray scale images.

# Introduction

Screen content, such as maps, text and graphics, have different characteristics from camera-captured images with patterns that tend to be clean and repetitive in texture. A technique called intra block copy [1, 2, 3] exploits this property for coding gain by finding a matching block that can be used as a predictor.

Images can also contain many blocks that can share common patterns but with different orientations. For example, in the screen content such as maps, as shown in left of Figure 1, there can be rotated repeated text.  Blocks on the same object boundary tend to also contain rotated repeated patterns, as shown in the right of Figure 1. Unfortunately, due to this orientation difference, such blocks cannot be used as predictors by intra block copy for coding gains.

Fig. 1: Rotated patch matching. Left: matched texts on screen content with different orientation.  Right: matched blocks along the curved boundary of an object

Accordingly, this contribution proposes extending the intra block copy so that the block matching process is invariant to rotation using a technique called rotate intra block copy [4]. As a result, the residual energy (i.e. prediction error) is substantially reduced, which can result in coding gains.

This contribution presents the following contributions for rotate intra block copy:

- a method to rotate the predictor to reduce the residual energy

- a method of coding the rotated motion vector

- an analysis of the impact of angular resolution on residual energy

- an evaluation of the coding gains relative to HEVC intra prediction plus intra block copy in a full codec

This work appeared in Z. Zhang, V. Sze, “Rotate Intra Block Copy for Still Image Coding,” IEEE International Conference on Image Processing (ICIP), September 2015 [4].

# Proposed Method

The encoder processes blocks of pixels of an image in a raster scan order. To encode a target block P(x0) with top left corner located at x0, the encoder can use any previously coded pixels before P(x0), denoted by Ω(x0), within the same frame as a predictor as shown in Figure 2. To predict P(x0), intra block copy method seeks a block anchored at x within Ω(x0), that minimizes the sum of squared differences (SSD) between P(x) and P(x0). Rotate intra block copy adds additional flexibility by allowing the block to rotate by an arbitrary angle Θ. Let PΘ(x) represent the block rotated by Θ which starts from x.  Note that PΘ(x) is always represented by an axis aligned matrix despite its rotation.

Fig. 2: Intra block copy vs. rotate intra block copy.

To determine the intensity of each pixel in PΘ(x), we apply the standard image transformation technique illustrated in Figure 3. Specifically, it first establishes the rotating mapping between the coordinate system of the rotated patch PΘ(x) and the coordinate system of the source image. This mapping projects each pixel in PΘ(x) back to the source image. Since the mapped points may be fractional, bilinear interpolation is performed to compute their intensity values.

Fig. 3: Rotating an image with bilinear interpolation.

In the proposed approach, the allowed rotation angle Θ is restricted to a discrete list of candidates, 0:(π/N):π, leaving N as a parameter to tune. Section 1.2 describes how to select N.

The optimal solution is found through an exhaustive search by enumerating each candidate Θ, finding x given the fixed Θ, repeating this for all candidates Θ, and selecting the best (x, Θ) pair overall. Given the candidate Θ, the search for x can be done in two ways (Figure 4):

- rotate the target block by -Θ and search for a rotated block on the original axis-aligned image

- rotate the original image by Θ and search for an axis-aligned block

The first approach is trying to minimize the SSD between P'(x) and P’-Θ(x) performs worse due to the interpolation introduced by the rotation of the target block. This mismatch can affect the rate-distortion optimization in a codec. In practice, we observe that the SSD between the target patch and the optimal patch found during the encoder search increases by 1% if we adopt the first scheme. Therefore, the second method is used to avoid this unnecessary loss of accuracy.

Fig. 4: Left: Rotate the target block and search on the original image. Note that the rotated patch is padded to a larger rectangular patch and the padded area should be excluded for comparison during the search.  Right: Rotate the image and search for an axis-aligned block.

Figure 5 visualizes the residual generated by the HEVC intra prediction, HEVC plus intra block copy (HEVC+BC), and HEVC plus rotate intra block copy (HEVC+ROTATE) on PartyScene from the JCT-VC common test sequences. The residual of the rotate intra block copy is visually sparser compared with the other two baseline methods. This represents a 40% and 27% reduction in residual energy (i.e. SSD) as compared with the original HEVC intra prediction and HEVC plus intra block copy, respectively.

Fig. 5: Absolute residual from different intra prediction methods for PartyScene (cropped).

## Motion Vector Coding

Suppose x* is the location of the optimal patch we found to predict the target patch located at x0. To signal x* to the decoder, we compute and signal the motion vector similar to the intra block copy. Since the search for x* is performed on the grid of the rotated coordinate system (uΘ, vΘ), x* is an integer in (uΘ, vΘ) but fractional in the source coordinate system (u, v). To avoid encoding fractional numbers, we choose to encode x* in (uΘ, vΘ) instead of (u, v). Note that x0 is in the original unrotated coordinate system, for it to help the prediction of x* we need to rotate it to (uΘ, vΘ) by multiplying it with the rotation matrix RΘ with rotate angle Θ defined as

RΘ =cosθ-sinθsinθcosθ

Since RΘx0 may be fractional, we need to perform rounding on it.

Let us denote the output by round(RΘx0).

Then we define the motion vector in the rotated coordinate system (uΘ, vΘ) as mv = x* - round(RΘx0).

The encoder computes mv and signals it together with Θ.

At the decoder side, mv, x0 and Θ are all known so the decoder can retrieve x* = mv + round(RΘx0).

For intra block copy, the motion vectors of neighboring blocks are correlated, and this correlation is exploited to reduce signaling cost. However, with rotate intra block copy, the rotation angle of the neighboring blocks can be different, which causes the motion vectors to no longer be correlated as shown in Figure 6 and explained later. This can be addressed by transforming the motion vectors of neighboring block to the same coordinate system to increase correlation.

Fig. 6: Rotate motion vector prediction

To be specific, suppose there are two adjacent target blocks with rotation angle Θ1, Θ2 and motion vector mv(1), and mv(2), as shown in Figure 6(a). The motion vector for the first block mv(1)  is encoded as mvθ1(1)  in the rotated coordinate system uθ1,vθ1 (Fig. 6(b)). Similarly mv(2) is encoded as mvθ2(2) in uθ2,vθ2 Although mv(1) is close to mv(2), there is a big gap between mvθ1(1) and mvθ2(2) due to different rotation angle Θ1 and Θ2. To compensate, we have to rotate mvθ1(1)from uθ1,vθ1 to uθ2,vθ2 as shown in Figure 6(c). This is achieved by multiplying mvθ1(1) with a rotation matrix Rθ2-θ1.

The motion vector predictor is generated based on the rotated motion vector of the neighboring block (top neighbor if first block in row, otherwise left neighbor). If the previously encoded block is not rotate intra block copy, the predictor is set to zero.  Motion vector prediction can be adaptively enabled using a flag.  If enabled the motion vector difference is encoded, while if disabled the motion vector is explicitly encoded. Both the motion vector difference and the motion vector are encoded with a third order Exp-Golomb codeword.

We collected the statistics of the motion vector codes when encoding class C sequences of JCT-VC test sequences. The results suggest that motion vector prediction is enabled on 24.5% of the blocks, validating the need for a flag to dynamically enable motion vector prediction for each coding unit (CU). In addition, for the blocks where motion vector prediction are enabled, the rotating motion vector compensation technique reduces the average bit rates for the motion vector difference by 25%, which proves its effectiveness.

## Angular resolution of rotation

The angular resolution for the search has a significant impact on both the speed and coding performance of the encoder. Higher angular resolution increases the chance of locating a more accurate match for the target block; however, it also slows down the searching process and requires more bits to represent the rotation angle.

The tradeoff between these factors was evaluated with several different angular resolutions across several test images as shown in Fig. 7. Step size of π/32 is selected for favorable balance between coding gain and encoding speed.

Rather than sending the actual rotation angle, a list is constructed containing all supported rotation angles, and the index of the rotation angle is transmitted.  For a step size of π/32, the rotation index is coded with a 6-bit fixed length codeword. No correlation was observed between rotation angles of neighboring blocks.

Fig. 7: Tradeoff between angular resolution and residual energy

# Experimental Results

A codec was implemented in MATLAB to evaluate how the reduction in residual energy impacts coding efficiency. The codec pipeline consists of intra prediction, transform-quantization, and entropy coding.

The intra prediction module consists of HEVC intra prediction (with DC, planar and 33 angular prediction directions, but without intra smoothing), intra block copy and rotates intra block copy. The HEVC discrete cosine transform (DCT) and quantization is applied to the prediction error to compute the residual. The residual is then encoded using a zigzag scan and run-length-encoding (rle). The run and length values are mapped to codewords using an optimal Huffman table based on rle sequences of all blocks. The codec also supports an HEVC style quad-tree block structure with coding units (CU) from 16x16 down to 4x4 block sizes. Each prediction unit (PU) can either be HEVC intra prediction, intra block copy or rotate intra block copy (if enabled).

To signal the encoded image requires the following bits for various sets of information including

- Picture: height, width, QP, Coding Tree Unit (CTU) size, max CU depth, encoding mode (HEVC only, HEVC+BC, HEVC+ROTATE), rotation range and resolution

- Block Structure and Prediction: split flag, enable intra block copy or rotate intra block copy, HEVC intra prediction mode, motion vector info (motion vector predictor enable flag, motion vector/motion vector difference), rotation angle index

- Residual: run, length

The rate distortion optimization is performed using the standard objective function D+ λR, where D is the quantization error of a block, and R is the estimated bits of the block including residual bits, motion vector info bits, and intra prediction mode/rotation angle index bits. The residual bits are estimated by multiplying the number of run-length values with the average codeword length in the rle sequence, which is learnt from the test results.

The same exhaustive intra motion vector search strategy is applied for both intra block copy and rotate intra block copy with the same motion vector coding to make the comparison fair (for intra block copy, no neighbor rotation is needed). For rotate intra block copy, the search for the motion vector is limited to the area within +/- 128 pixels around the target patch along both axes (Figure 8 (left))).  A similar restriction is applied on intra block copy, but the vertical motion vector component can only be negative (Figure 8 (right)).

Fig. 8: Restriction on the search range. (Left) rotate intra block copy; (Right) intra block copy

Table 1 shows the coding efficiency of HEVC intra prediction plus intra block copy (HEVC+BC) and HEVC intra prediction plus rotate intra block copy (HEVC+ROTATE) evaluated using the first frame of the test sequences in the JCT-VC common test conditions [5]. The BD-rate [6] is measured for QP={22, 27, 32, 37} and only the luma component is considered. Note that the Huffman dictionary is not included in the bits for HEVC+BC or HEVC+ROTATE; this is because on average the bits needed to encode the Huffman dictionary of HEVC + ROTATE (2153 bits) is smaller than the other (2310 bits). We exclude this artificial gain in BD-rate because the actual HM pipeline is not using Huffman table for entropy coding.

For HEVC+ROTATE, the encoder performs both HEVC and rotate intra block copy predictions and picks the best from the rate-distortion optimization; the same goes for HEVC+BC. It can be seen that HEVC+ROTATE achieves on average residual energy reduction of 20% reduction compared with HEVC+BC. With the proposed codec, this reduction translates into a 3.4% coding gain in BD-rate averaged over all the test sequences. Note that the bits for the block structure and prediction actually increase by 26% to signal the rotate angle, but those bits only account for 27% of the total bits and their increase is surpassed by the reduction of the residual bits and as a whole the proposed technique achieves the aforementioned coding again.

|  | Sequence | Residual Reduction | BD-rate |
| --- | --- | --- | --- |
| Class C | RaceHorse | 23.66% | -4.54 |
|  | PartyScene | 27.64% | -4.45 |
|  | BQMall | 17.92% | -2.63 |
|  | BasketballDrill | 22.12% | -3.40 |
| Class D | BQSquare | 30.82% | -4.99 |
|  | BasketballPass | 15.44% | -1.84 |
|  | BlowingBubble | 7.59% | -2.81 |
|  | RaceHorse | 28.97% | -4.42 |
| Class E | FourPeople | 18.09% | -2.54 |
|  | Johnny | 12.79% | -2.35 |
|  | KristenAndSara | 15.67% | -2.43 |
| Class F (screen content) | BasketBallDrillText | 21.15% | -3.64 |
|  | SlideShow | 29.01% | -7.43 |
|  | SlideEditing | 19.12% | -0.74 |
| Class C Average |  | 22.83% | -3.76 |
| Class D Average |  | 20.70% | -3.52 |
| Class E Average |  | 15.52% | -2.44 |
| Class F Average |  | 23.09% | -3.94 |
| Overall Average |  | 20.71% | -3.44 |

Table 1: Residual energy reduction and BD-rate of HEVC+ROTATE compared with HEVC+BC. The BD-rate comparison was performed on the first frame of each test sequence, under the JCT-VC common test conditions, with the BD-rate measured at QP={22, 27, 32, 37} and only the luma component is considered.  The encoder/decoder were built in MATLAB and used Huffman code rather than CABAC.

# Conclusions

This contribution proposes an intra prediction method that delivers significant residual energy reduction compared with existing intra coding tools such as HEVC intra prediction and the intra block copy technique. It also presented an efficient way to encode the rotated motion vector and determined an angular search resolution that gives a favorable search vs. coding efficiency tradeoff. Experiments were conducted on JCT-VC common test sequences to measure how the reduction in residual energy can be translated into coding gains.

References

[1] M.  Budagavi and  D.-K.  Kwon,“JCTVC-M0350: AHG8:   Video  coding  using  Intra  motion  compensa-tion,” Joint Collaborative Team on Video Coding (JCT-VC), April 2013.

[2]  S. L. Yu and C. Chrysafis, “JVT-C151r1: New intra pre-diction using intra-macroblock motion compensation,” Joint Video Team (JVT), May 2002.

[3]  C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi,“JCTVC-N0256:   Non-RCE3:Intra  motion  compensation with 2-D MVs,” Joint Collaborative Team on Video Coding (JCT-VC), July 2013.

[4]   Z. Zhang, V. Sze, “Rotate Intra Block Copy for Still Image Coding,” IEEE International Conference on Image Processing (ICIP), September 2015.

[5]  F. Bossen,  “JCTVC-L1100:  Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC), Jan 2013

[6] G. Bjøntegaard,  “VCEG-M33:  Calculation of Average PSNR Differences between RD curves,” Video Coding Experts Group (VCEG), April 2001.

# Patent rights declaration(s)

Massachusetts Institute of Technology does not have any current or pending patent rights relating to the technology described in this contribution.

First frame of BasketballPass, BQSquare, BasketballDrillText

0:(π/32): π contains 33 values, requiring 6 bits to encode instead of 5