| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0050-v1 |
| --- | --- |

| Title: | CE1-related: Split Constraint Considering Picture Boundary Condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Minsoo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | m.w.park@samsung.com mss.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes a split constraint, which is basically similar to CE1 Test 2.1.2, to prohibit 32x128 and 128x32 partitions. In addition to this, it is proposed methods for handling of picture boundary CTUs to have the same split constraint with CTUs inside a picture. For instance, if a 64x128 CU is laid on right picture boundary (this means that its parent CU is partitioned with binary split in vertical direction), then the CU should be partitioned by using binary split in vertical direction. In this case, 32x128 CU can occur. To resolve this issue, this proposal presents three methods as follows: For a right or bottom picture boundary CU, 1) if the CU size is 64x128 or 128x64, quad split is used, 2) if the CU is non-square and longer side of the CU is greater than 64, then binary split with different direction is used, and 3) if the CU size is 128x128, then forced quad split is applied. The proposed constraint reportedly shows 0.08% BD-rate with 98% encoding time on RA, 0.00% BD-rate with 98% encoding time on LDB. The proposed methods for handling picture boundary reportedly show 0.14%, 0.10%, and 0.11% BD-rate with 98% encoding time on RA for methods 1), 2), and 3).

Introduction

In VVC [1], in order to support 64x64 pipeline friendly architecture, a ternary split at the top level is currently prohibited. At the last meeting, it was also agreed to prohibit 32x128 and 128x32 partitions for the same reason. To solve this issue, several ideas were proposed and tested in CE1 [2]. In terms of coding efficiency impact, only allowing 1:2 and 2:1 ratio between width and height at the top level (i.e. only allowing 128x64 and 64x128 at the top level) seems to be desirable.

Figure 1. Example of picture boundary CUs

However, since the picture boundary handling fix was integrated into VVC, this constraint needs to be properly applied to picture boundary CUs as well. For example, if a 64x128 CU is laid on right picture boundary as shown in Figure 1 (this means that its parent CU is partitioned with binary split in vertical direction), the CU should be partitioned by binary split in vertical direction. In this case, 32x128 CU can occur. To resolve this issue, this proposal proposes several methods.

Proposed Methods

Proposed split constraint without picture boundary handling

In the example case shown in Figure 1, after the 64x128 CU is partitioned by implicit vertical binary split, the left size 32x128 CU which became to be located inside a picture can be further partitioned by using binary split in vertical direction if Test 2.1.2 condition is used. To prevent these cases, it is proposed a modified condition as following.

| Test 2.1.2 in CE1 | - if width = = 64 and height > 64 Prohibit vertical binary split - if height = = 64 and width > 64 Prohibit horizontal binary split |
| --- | --- |
| Proposed Method | - if width != height and height > 64 Prohibit vertical binary split - if width != height and width > 64 Prohibit horizontal binary split |

The proposed constraint has the same behavior with Test 2.1.2 when a CU is located inside a picture, but has different results for picture boundary CUs. Figure 2 shows an example of comparison between Test 2.1.2 and the proposed method. For a 32x128 CU inside picture, which is in a picture boundary CTU, while Test 2.1.2 allows the vertical binary split, the proposed method does not allow the vertical binary split as shown in Figure.

Figure 2. Example of comparison between Test 2.1.2 in CE1 and the proposed method for a CU inside picture in a picture boundary CTU

Picture boundary handling methods

To handle the issue raised in this contribution, three possible methods are proposed on top of the proposed split constraint.

Method 1: Use implicit quad split

If a CU is on right or bottom picture boundary and the constraint condition is met, an implicit quad split can be used to solve problematic cases as shown in Figure 3. But, this idea may violate the rule that quad split is not allowed after binary split.

Figure 3. Example of using implicit quad split

Method 2: Use implicit binary split with different direction

If a CU is on right or bottom picture boundary and the constraint condition is met, an implicit binary split with different direction can be used as shown in Figure 4. This idea can solve problematic cases without any violation of the current partitioning rules.

Figure 4. Example of using implicit binary split with different direction

This modification can be represented in a simple way as following working draft changes. Changes are highlighted in green.

- Allowed binary split process Input to this process is a binary split mode btSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth, a MaxMttDepth offset depthOffset and a partition index partIdx. Output of this process is the variable allowBtSplit. Table 6-2 – Specification of parallelTtSplit, perpCbPos, perpCbSize, perpMaxPicSize and cbSize based on btSplit. btSplit = = SPLIT_BT_VER; btSplit = = SPLIT_BT_HOR. parallelTtSplit; SPLIT_TT_VER; SPLIT_TT_HOR. cbSize; cbWidth; cbHeight. perpCbSize; cbHeight; cbWidth. The variables parallelTtSplit and cbSize are derived as specified in Table 6-2. The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - cbSize is less than or equal to MinBtSizeY - cbWidth is greater than MaxBtSizeY - cbHeight is greater than MaxBtSizeY - mttDepth is greater than or equal to MaxMttDepth + depthOffset - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - y0 + cbHeight is greater than pic_height_in_luma_samples - cbWidth is equal to cbHeight or cbHeight is smaller than or equal to 64 - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_HOR - x0 + cbWidth is greater than pic_width_in_luma_samples - y0 + cbHeight is smaller than or equal to pic_height_in_luma_samples - cbWidth is equal to cbHeight or cbWidth is smaller than or equal to 64 - Otherwise if the following condition is true, allowBtSplit is set equal to FALSE: - cbWidth is not equal to cbHeight and perpCbSize is greater than 64 - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - mttDepth is greater than 0 - partIdx is equal to 1 - MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit – Otherwise, allowBtSplit is set equal to TRUE.

Method 3: Use implicit quad split at the top level

To prevent problematic cases in advance, an implicit quad split at the top level can be applied. This modification can be represented as following working draft changes. Changes are highlighted in green.

- Allowed binary split process Input to this process is a binary split mode btSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth, a MaxMttDepth offset depthOffset and a partition index partIdx. Output of this process is the variable allowBtSplit. Table 6-2 – Specification of parallelTtSplit, perpCbPos, perpCbSize, perpMaxPicSize and cbSize based on btSplit. btSplit = = SPLIT_BT_VER; btSplit = = SPLIT_BT_HOR. parallelTtSplit; SPLIT_TT_VER; SPLIT_TT_HOR. cbSize; cbWidth; cbHeight. perpCbSize; cbHeight; cbWidth. The variables parallelTtSplit and cbSize are derived as specified in Table 6-2. The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - cbSize is less than or equal to MinBtSizeY - cbWidth is greater than MaxBtSizeY - cbHeight is greater than MaxBtSizeY - mttDepth is greater than or equal to MaxMttDepth + depthOffset - cbWidth is not equal to cbHeight and perpCbSize is greater than 64 - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - y0 + cbHeight is greater than pic_height_in_luma_samples - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_HOR - x0 + cbWidth is greater than pic_width_in_luma_samples - y0 + cbHeight is smaller than or equal to pic_height_in_luma_samples - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - mttDepth is greater than 0 - partIdx is equal to 1 - MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit – Otherwise, allowBtSplit is set equal to TRUE. #### 7.3.4.3 Coding quadtree syntax coding_quadtree( x0, y0, log2CbSize, cqtDepth, treeType ) {; Descriptor. if( ( ( ( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples ) ? 1 : log2CbSize = = 7 ? −1 : 0 ) + ( ( y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples ) ? 1 : log2CbSize = = 7 ? −1 : 0 ) + ( ( ( 1 << log2CbSize ) <= MaxBtSizeY ) ? 1 : 0 ) ) >= 2 && log2CbSize > MinQtLog2SizeY ). qt_split_cu_flag[ x0 ][ y0 ]; ae(v). }. if( qt_split_cu_flag[ x0 ][ y0 ] ) {. x1 = x0 + ( 1 << ( log2CbSize − 1 ) ). y1 = y0 + ( 1 << ( log2CbSize − 1 ) ). coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1, treeType ). if( x1 < pic_width_in_luma_samples ). coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1, treeType ). if( y1 < pic_height_in_luma_samples ). coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1, treeType ). if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ). coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1, treeType ). } else. multi_type_tree( x0, y0, 1 << log2CbSize, 1 << log2CbSize, 0, 0, 0, treeType ). }.

Experimental Results

The simulation was carried out under the JVET common test condition [3]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions. Table 1 shows summary of all results

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Split constraint | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Method 1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Method 2 | 0.00% | 0.00% | 0.00% | 100% | 97% |
|  | Method 3 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| RA | Split constraint | 0.08% | 0.11% | 0.04% | 98% | 100% |
|  | Method 1 | 0.14% | 0.17% | 0.11% | 97% | 100% |
|  | Method 2 | 0.10% | 0.14% | 0.09% | 98% | 99% |
|  | Method 3 | 0.11% | 0.13% | 0.07% | 97% | 100% |
| LDB | Split constraint | 0.00% | -0.06% | -0.19% | 98% | 100% |
|  | Method 1 | 0.15% | 0.15% | -0.01% | 98% | 101% |
|  | Method 2 | 0.04% | -0.05% | -0.07% | 98% | 100% |
|  | Method 3 | 0.09% | 0.02% | -0.14% | 97% | 101% |

Results for proposed split constraint

Table 2. Results for Proposed split constraint

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.27% | 0.16% | 96% | 100% |
| Class A2 | 0.17% | 0.14% | 0.02% | 97% | 100% |
| Class B | 0.05% | 0.03% | -0.01% | 99% | 100% |
| Class C | 0.01% | 0.07% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.11% | 0.04% | 98% | 100% |
| Class D | 0.00% | 0.02% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.03% | 0.02% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.08% | -0.32% | 98% | 100% |
| Class C | 0.04% | -0.01% | -0.19% | 100% | 98% |
| Class E | -0.14% | -0.10% | 0.01% | 97% | 103% |
| Overall | 0.00% | -0.06% | -0.19% | 98% | 100% |
| Class D | 0.01% | 0.34% | -0.14% | 100% | 100% |
| Class F (optional) | 0.04% | 0.46% | -0.01% | 103% | 100% |

Results for Method 1 (using implicit quad split)

Table 3. Results for Method 1 (Using implicit quad split)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 102% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.37% | 0.25% | 95% | 99% |
| Class A2 | 0.23% | 0.17% | 0.11% | 97% | 100% |
| Class B | 0.13% | 0.12% | 0.08% | 98% | 101% |
| Class C | 0.04% | 0.09% | 0.05% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.14% | 0.17% | 0.11% | 97% | 100% |
| Class D | 0.08% | 0.12% | 0.08% | 100% | 100% |
| Class F (optional) | 0.08% | 0.10% | 0.08% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.14% | 0.16% | 98% | 100% |
| Class C | 0.00% | -0.04% | -0.34% | 100% | 99% |
| Class E | 0.43% | 0.41% | 0.14% | 96% | 103% |
| Overall | 0.15% | 0.15% | -0.01% | 98% | 101% |
| Class D | 0.08% | 0.33% | -0.27% | 99% | 99% |
| Class F (optional) | 0.02% | 0.32% | 0.43% | 99% | 101% |

Results for Method 2 (Using implicit binary split with different direction)

Table 4. Results for Method 2 (Using implicit binary split with different direction)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.32% | 0.26% | 95% | 98% |
| Class A2 | 0.19% | 0.13% | 0.13% | 97% | 99% |
| Class B | 0.09% | 0.10% | 0.01% | 98% | 100% |
| Class C | 0.00% | 0.09% | 0.05% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.14% | 0.09% | 98% | 99% |
| Class D | 0.01% | 0.03% | 0.01% | 101% | 101% |
| Class F (optional) | 0.02% | 0.03% | 0.01% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.12% | -0.03% | 98% | 99% |
| Class C | 0.01% | 0.00% | -0.21% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.05% | 97% | 102% |
| Overall | 0.04% | -0.05% | -0.07% | 98% | 100% |
| Class D | 0.07% | 0.26% | -0.22% | 100% | 99% |
| Class F (optional) | 0.02% | 0.25% | 0.03% | 99% | 101% |

Results for Method 3 (Using implicit quad split at the top level)

Table 5. Results for Method 3 (Using implicit quad split at the top level)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.31% | 0.19% | 94% | 100% |
| Class A2 | 0.20% | 0.16% | 0.10% | 97% | 100% |
| Class B | 0.09% | 0.06% | 0.04% | 97% | 100% |
| Class C | 0.01% | 0.08% | -0.01% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.13% | 0.07% | 97% | 100% |
| Class D | 0.04% | 0.06% | -0.04% | 96% | 100% |
| Class F (optional) | 0.04% | 0.06% | 0.06% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.08% | -0.14% | 98% | 101% |
| Class C | 0.06% | 0.01% | -0.17% | 99% | 99% |
| Class E | 0.15% | 0.22% | -0.09% | 94% | 104% |
| Overall | 0.09% | 0.02% | -0.14% | 97% | 101% |
| Class D | 0.06% | 0.31% | -0.20% | 99% | 100% |
| Class F (optional) | 0.03% | 0.73% | 0.49% | 98% | 101% |

Conclusion

In this contribution, a split constraint to support 64x64 pipeline architecture design and methods for handling picture boundary CUs in problematic cases were proposed. With the proposed methods, VVC can fully support 64x64 pipeline architecture design with the minimum coding efficiency impact. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

References

[1] “Versatile Video Coding (Draft 2),” JVET-K1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] “Description of Core Experiment 1 (CE1): Partitioning,” JVET-K1021, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[3] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).