| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0051-v1 |
| --- | --- |

| Title: | CE1-related: Partitioning Clean-ups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minsoo Park Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | mss.park@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes two clean-ups about the partitioning structure in VVC. One is to remove the adaptive signalling of the maximum binary tree size from slice header, and the other is to propose to set the maximum binary and ternary tree sizes to the possible maximum sizes by default. The first clean-up reportedly shows –0.08% BD-rate with 107% encoding time on RA. When an encoder optimization technique is applied on top of the first clean-up, the result reportedly shows 0.02% BD-rate with 99% encoding time. The second clean-up reportedly shows –0.57% BD-rate with 140% encoding time on AI, –0.12% BD-rate with 105% encoding time on RA. When an encoder optimization technique is applied on top of the second clean-up, the results reportedly show –0.05% for Y, –0.87% for U and –0.86% for V BD-rates with 102% encoding time on AI, and –0.01% for Y, –0.39% for U and –0.35% for V BD-rates with 101% encoding time on RA. Results of two clean-ups with the encoder optimization techniques reportedly show –0.05% for Y, –0.87% for U and –0.86% for V BD-rates with 102% encoding time on AI, and 0.02% for Y, –0.37% for U and –0.33% for V BD-rates with 99% encoding time on RA.

Introduction

In VVC [1], there are three split methods: quad, binary, and ternary trees. Because of this, combinations of the split methods are numerous. In order to reduce the number of these combinations, VVC defines the maximum size for each split as shown in Table 1. Moreover, in case of binary tree in intra slice, the maximum sizes between luma and chroma components are different, and in case of ternary tree, the sizes between intra and inter slices are different.

Table 1. Maximum size for each split in VVC

|  | Intra slice |  | Inter slice |
| --- | --- | --- | --- |
|  | Luma | Chroma | Luma & Chroma |
| Quad tree | 128 | 128 | 128 |
| Binary tree | 32 | 64 | 128 |
| Ternary tree | 32 | 32 | 64 |

In case of binary tree split, although the possible maximum size is 128 in inter slices, the maximum binary tree size can be adjusted by slice level signalling to reduce complexity. In the current VTM software, the maximum size for a inter slice is decided based on an average CU size of its previously coded slices. The average CU size is calculated by following equation.

average CU size=total area of CUtotal number of CU

The maximum binary tree size is decided based on the average CU size as shown in Table 2. For example, if the average CU size is larger than or equal to 60, the maximum binary tree size is set to 128. The decided maximum size is signalled in slice header.

Table 2. Maximum BT size setting depending on the average CU size (T)

|  | T >= 60 | 60 > T >= 30 | 30 > T |
| --- | --- | --- | --- |
| Maximum BT size | 128 | 64 | 32 |

Proposed Methods

In this contribution, two clean-ups for partitioning are proposed to have less restrictions in the partitioning structure.

Removal of adaptive Max BT size signalling

Firstly, it is proposed to remove the adaptive signalling of the maximum binary tree size and to make this scheme as an encoder only option. This modification can be represented by following working draft changes.

- Slice header syntax slice_header( ) {; Descriptor. slice_pic_parameter_set_id; ue(v). slice_address; u(v). slice_type; ue(v). if ( slice_type != I ). log2_diff_ctu_max_bt_size; ue(v). dep_quant_enabled_flag; u(1). if( !dep_quant_enabled_flag ). sign_data_hiding_enabled_flag; u(1). byte_alignment( ). }. - Slice header semantics … log2_diff_ctu_max_bt_size specifies the difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split. The value of log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When log2_diff_ctu_max_bt_size is not present, the value of log2_diff_ctu_max_bt_size is inferred to be equal to 2. …

Default Maximum BT and TT sizes

In order to align the maximum binary and ternary trees sizes between luma and chroma components, and between inter and intra slices, it is proposed to set the maximum binary and ternary trees sizes to the possible maximum sizes. Table 3 shows the proposed default the maximum size parameters.

Table 3. Proposed default maximum size for each split

|  | Intra slice |  | Inter slice |
| --- | --- | --- | --- |
|  | Luma | Chroma | Luma & Chroma |
| Quad tree | 128 | 128 | 128 |
| Binary tree | 32 64 | 64 | 128 |
| Ternary tree | 32 64 | 32 64 | 64 |

On top of this clean-up, an encoder option can be applied to decrease encoder run time. For luma in intra slices, the allowed maximum BT and TT sizes in encoder can be set to 32 as shown in Table 4.

Table 4. Default maximum sizes and the sizes used in encoder

|  | Default maximum size |  |  | Allowed maximum size in encoder |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra slice |  | Inter slice | Intra slice |  | Inter slice |
|  | Luma | Chroma | Luma & Chroma | Luma | Chroma | Luma & Chroma |
| Quad tree | 128 | 128 | 128 | 128 | 128 | 128 |
| Binary tree | 64 | 64 | 128 | 32 | 64 | 128 |
| Ternary tree | 64 | 64 | 64 | 32 | 64 | 64 |

This modification can be represented by following working draft changes.

The variables MinQtLog2SizeY, MaxBtLog2SizeY, MinBtLog2SizeY, MaxTtLog2SizeY, MinTtLog2SizeY, MaxBtSizeY, MinBtSizeY, MaxTtSizeY, MinTtSizeY and MaxMttDepth are derived as follows: MinQtLog2SizeY = ( slice_type = = I ) ? MinQtLog2SizeIntraY : MinQtLog2SizeInterY (7-25) MaxBtLog2SizeY = ( slice_type = = I ) ? 6 : 7 CtbLog2SizeY − log2_diff_ctu_max_bt_size (7-26) MinBtLog2SizeY = MinCbLog2SizeY (7-27) MaxTtLog2SizeY = 6( slice_type = = I ) ? 5 : 6 (7-28) MinTtLog2SizeY = MinCbLog2SizeY (7-29) MinQtSizeY = 1 << MinQtLog2SizeY (7-30) MaxBtSizeY = 1 << MaxBtLog2SizeY (7-31) MinBtSizeY = 1 << MinBtLog2SizeY (7-32) MaxTtSizeY = 1 << MaxTtLog2SizeY (7-33) MinTtSizeY = 1 << MinTtLog2SizeY (7-34) MaxMttDepth = ( slice_type = = I ) ? max_mtt_hierarchy_depth_intra_slices : max_mtt_hierarchy_depth_inter_slices (7-35)

Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions. Table 5 shows summary of all results.

Table 5. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Test1: clean-up 1 | 0.00% | 0.00% | 0.00% | 99% | 95% |
|  | Test2: clean-up 1 w/ enc. opt. | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  | Test3: clean-up 2 | -0.57% | -1.30% | -1.37% | 140% | 96% |
|  | Test4: clean-up 2 w/ enc. opt | -0.05% | -0.87% | -0.86% | 102% | 98% |
|  | Test5: clean-up 1&2 w/ enc. opt. | -0.05% | -0.87% | -0.86% | 102% | 97% |
| RA | Test1: clean-up 1 | -0.08% | -0.11% | -0.12% | 107% | 98% |
|  | Test2: clean-up 1 w/ enc. opt. | 0.02% | 0.06% | 0.01% | 99% | 95% |
|  | Test3: clean-up 2 | -0.12% | -0.54% | -0.43% | 105% | 99% |
|  | Test4: clean-up 2 w/ enc. opt | -0.01% | -0.39% | -0.35% | 101% | 99% |
|  | Test5: clean-up 1&2 w/ enc. opt. | 0.02% | -0.37% | -0.33% | 99% | 95% |
| LDB | Test1: clean-up 1 | -0.09% | -0.28% | -0.12% | 108% | 93% |
|  | Test2: clean-up 1 w/ enc. opt. | 0.04% | 0.05% | -0.04% | 99% | 95% |
|  | Test3: clean-up 2 | -0.04% | -0.36% | -0.26% | 98% | 93% |
|  | Test4: clean-up 2 w/ enc. opt | 0.00% | -0.02% | -0.05% | 98% | 94% |
|  | Test5: clean-up 1&2 w/ enc. opt. | 0.03% | 0.01% | -0.23% | 98% | 95% |

Test 1: Clean-up 1 (Removal of adaptive Max BT signaling)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 94% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 91% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 92% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | -0.03% | 108% | 97% |
| Class A2 | -0.09% | -0.13% | -0.15% | 106% | 96% |
| Class B | -0.12% | -0.14% | -0.18% | 106% | 100% |
| Class C | -0.07% | -0.06% | -0.10% | 109% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.11% | -0.12% | 107% | 98% |
| Class D | -0.08% | -0.07% | -0.09% | 116% | 106% |
| Class F (optional) | -0.05% | -0.07% | -0.09% | 104% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.39% | -0.35% | 106% | 93% |
| Class C | -0.10% | -0.23% | -0.11% | 117% | 92% |
| Class E | -0.07% | -0.16% | 0.26% | 100% | 95% |
| Overall | -0.09% | -0.28% | -0.12% | 108% | 93% |
| Class D | -0.16% | 0.26% | -0.09% | 159% | 99% |
| Class F (optional) | -0.13% | 0.05% | 0.16% | 107% | 97% |

Test 2: Clean-up 1 (Removal of adaptive Max BT signaling) with encoder optimization

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 92% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 92% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.05% | 99% | 95% |
| Class A2 | 0.02% | 0.02% | -0.06% | 100% | 95% |
| Class B | 0.02% | 0.06% | 0.05% | 100% | 98% |
| Class C | 0.03% | 0.08% | -0.02% | 96% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.06% | 0.01% | 99% | 95% |
| Class D | 0.05% | 0.08% | 0.05% | 97% | 99% |
| Class F (optional) | 0.02% | 0.00% | 0.01% | 97% | 91% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.02% | -0.08% | 98% | 94% |
| Class C | 0.08% | -0.03% | 0.03% | 101% | 95% |
| Class E | -0.03% | 0.28% | -0.09% | 98% | 97% |
| Overall | 0.04% | 0.05% | -0.04% | 99% | 95% |
| Class D | 0.03% | 0.06% | -0.05% | 100% | 109% |
| Class F (optional) | -0.10% | 0.50% | 0.20% | 97% | 99% |

Test 3: Clean-up 2 (Default Maximum BT and TT sizes)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | -2.03% | -2.36% | 173% | 96% |
| Class A2 | -0.59% | -1.52% | -1.15% | 145% | 91% |
| Class B | -0.51% | -1.38% | -1.59% | 140% | 93% |
| Class C | -0.15% | -0.42% | -0.39% | 118% | 100% |
| Class E | -0.61% | -1.40% | -1.52% | 138% | 100% |
| Overall | -0.57% | -1.30% | -1.37% | 140% | 96% |
| Class D | -0.10% | -0.18% | -0.49% | 121% | 94% |
| Class F (optional) | -0.23% | -0.63% | -0.42% | 122% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.44% | -0.41% | 109% | 97% |
| Class A2 | -0.20% | -0.81% | -0.53% | 104% | 96% |
| Class B | -0.11% | -0.81% | -0.76% | 108% | 102% |
| Class C | 0.01% | -0.07% | 0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.54% | -0.43% | 105% | 99% |
| Class D | 0.02% | -0.24% | -0.18% | 100% | 105% |
| Class F (optional) | -0.09% | -0.11% | -0.23% | 102% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.17% | -0.48% | 98% | 95% |
| Class C | 0.07% | -0.03% | -0.19% | 98% | 91% |
| Class E | -0.25% | -1.13% | 0.01% | 98% | 94% |
| Overall | -0.04% | -0.36% | -0.26% | 98% | 93% |
| Class D | -0.02% | 0.05% | -0.41% | 98% | 99% |
| Class F (optional) | -0.14% | 0.30% | 0.34% | 98% | 96% |

Test 4: Clean-up 2 (Default Maximum BT and TT sizes) with encoder optimization

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.99% | -1.05% | 101% | 100% |
| Class A2 | -0.08% | -1.01% | -0.75% | 103% | 94% |
| Class B | -0.04% | -0.96% | -1.10% | 102% | 95% |
| Class C | -0.07% | -0.38% | -0.31% | 102% | 102% |
| Class E | -0.05% | -1.10% | -1.09% | 101% | 102% |
| Overall | -0.05% | -0.87% | -0.86% | 102% | 98% |
| Class D | -0.02% | -0.18% | -0.41% | 100% | 95% |
| Class F (optional) | -0.05% | -0.46% | -0.30% | 103% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.27% | -0.19% | 101% | 97% |
| Class A2 | -0.02% | -0.57% | -0.41% | 97% | 97% |
| Class B | 0.00% | -0.61% | -0.55% | 107% | 102% |
| Class C | 0.00% | -0.08% | -0.17% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.39% | -0.35% | 101% | 99% |
| Class D | -0.01% | -0.18% | -0.28% | 98% | 106% |
| Class F (optional) | 0.01% | -0.27% | -0.24% | 98% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.30% | 98% | 95% |
| Class C | 0.04% | -0.09% | -0.07% | 98% | 91% |
| Class E | -0.05% | 0.01% | -0.61% | 97% | 95% |
| Overall | 0.00% | -0.02% | -0.05% | 98% | 94% |
| Class D | 0.02% | 0.14% | -0.17% | 98% | 100% |
| Class F (optional) | -0.14% | 0.26% | -0.13% | 98% | 97% |

Test 5: Clean-up 1 & 2 with encoder optimizations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.99% | -1.05% | 101% | 97% |
| Class A2 | -0.08% | -1.01% | -0.75% | 103% | 92% |
| Class B | -0.04% | -0.96% | -1.10% | 102% | 95% |
| Class C | -0.07% | -0.38% | -0.31% | 102% | 100% |
| Class E | -0.05% | -1.10% | -1.09% | 101% | 100% |
| Overall | -0.05% | -0.87% | -0.86% | 102% | 97% |
| Class D | -0.02% | -0.18% | -0.41% | 101% | 92% |
| Class F (optional) | -0.05% | -0.46% | -0.30% | 102% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.28% | -0.19% | 99% | 96% |
| Class A2 | -0.01% | -0.48% | -0.42% | 101% | 95% |
| Class B | 0.04% | -0.57% | -0.51% | 99% | 98% |
| Class C | 0.04% | -0.10% | -0.14% | 97% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.37% | -0.33% | 99% | 95% |
| Class D | 0.05% | -0.21% | -0.16% | 97% | 99% |
| Class F (optional) | 0.04% | -0.23% | -0.20% | 97% | 90% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.16% | -0.14% | 99% | 94% |
| Class C | 0.07% | -0.02% | 0.11% | 98% | 94% |
| Class E | -0.04% | -0.19% | -0.82% | 98% | 98% |
| Overall | 0.03% | 0.01% | -0.23% | 98% | 95% |
| Class D | 0.07% | -0.04% | -0.22% | 107% | 103% |
| Class F (optional) | 0.00% | 0.51% | 0.23% | 97% | 98% |

Conclusion

In this contribution, two partitioning clean-ups were proposed: one is about removal of the adaptive signalling of the maximum BT size, and the other is about setting default maximum size for each split method. With these clean-ups, VVC can have less restrictions and cleaner design. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

References

[1] “Versatile Video Coding (Draft 2),” JVET-K1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).