| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0052-v1 |
| --- | --- |

| Title: | CE3: Results on Multiple 4-tap filter (Test 3.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Min Woo Park Kiho Choi (Samsung) Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | narae1.choi@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

The proposed method uses multiple 4-tap linear interpolation filters which have different degrees of smoothness to generate a prediction block. Multi-type filters are alternatively applied depending on the position of the prediction pixels. In this contribution, each block uses one or two-type interpolation filters. Two types of interpolation filter are determined by the block size and the determined filters are switched in a block by the pixel position. Simulation results reportedly show average -0.39% luma BD-rate gain with 105% and 101% encoding and decoding time for all intra (AI), -0.16% luma BD-rate gain with 101% and 100% encoding and decoding time for random access (RA) based on VTM configuration.

Introduction

A combination of multi-type 4-tap interpolation filter is proposed in JVET-J0024 [1]. In this contribution, one or two types of interpolation filters are applied to produce a prediction block. Filters are switched depending on the pixel position, intra mode and block size. The degree of smoothness is increased with the distance from the reference pixel. Two pairs of filter set are denoted with (f0 (the 4-tap Cubic interpolation filter in JEM7 [2]) and f1), (f2 (the 4-tap Gaussian filter in JEM7) and f3). The additional filters f1 and f3 have smoother degrees than f0 and f2, respectively.

The filter switching condition is describe with pseudo algorithm.

- useSharpFilter = (x<8 || y<8);

- if (intra mode >=  DIA_IDX (mode 34))

sizeCondition = (width <= 16) && (height <= 32);

else

sizeCondition = (width <= 32) && (height <= 16);

- if (sizeCondition)

filter = (useSharpFilter) ? f0 : f1;

else

filter = (useSharpFilter) ? f2 : f3;

Multiple 4-tap filter is applied to luma and chroma block for angular prediction. The interpolation filter accuracy is equal to 1/32 fractional pixels.

Experimental Results

The proposed scheme is implemented into the BMS version-2.0.1 software. The first anchor is the BMS with ‘vtm’ configuration files and the second anchor is the BMS with ‘bms’ configuration files. Both the ‘All Intra’ and ‘Random Access’ test configurations were used. The simulation was carried out under the JVET common test condition [3]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions.

Table 2-1. Results for multiple four-tap filter (VTM, Sub-CE 3.3.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.19% | -0.09% | 106% | 102% |
| Class A2 | -0.13% | -0.23% | -0.28% | 104% | 102% |
| Class B | -0.37% | -0.57% | -0.57% | 104% | 100% |
| Class C | -0.67% | -1.07% | -1.02% | 103% | 101% |
| Class E | -0.62% | -0.72% | -0.84% | 111% | 101% |
| Overall | -0.39% | -0.59% | -0.59% | 105% | 101% |
| Class D | -0.49% | -0.62% | -0.83% | 103% | 105% |
| Class F (optional) | -0.37% | -0.69% | -0.88% | 103% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.11% | -0.05% | 101% | 100% |
| Class A2 | -0.08% | -0.37% | -0.40% | 101% | 100% |
| Class B | -0.17% | -0.48% | -0.22% | 101% | 98% |
| Class C | -0.25% | -0.49% | -0.44% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.39% | -0.28% | 101% | 100% |
| Class D | -0.21% | -0.70% | -0.36% | 103% | 102% |
| Class F (optional) | -0.03% | -0.33% | -0.21% | 99% | 96% |

Table 2-2. Results for multiple four-tap filter (BMS, Sub-CE 3.3.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.18% | -0.01% | 103% | 102% |
| Class A2 | -0.12% | -0.20% | -0.25% | 102% | 103% |
| Class B | -0.37% | -0.51% | -0.56% | 102% | 102% |
| Class C | -0.62% | -0.89% | -0.99% | 101% | 101% |
| Class E | -0.56% | -0.75% | -0.77% | 103% | 103% |
| Overall | -0.36% | -0.53% | -0.55% | 102% | 102% |
| Class D | -0.46% | -0.71% | -0.63% | 101% | 101% |
| Class F (optional) | -0.34% | -0.62% | -0.57% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.39% | -0.13% | 101% | 100% |
| Class A2 | -0.06% | -0.35% | -0.34% | 102% | 101% |
| Class B | -0.14% | -0.31% | -0.45% | 101% | 101% |
| Class C | -0.24% | -0.47% | -0.35% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.38% | -0.34% | 101% | 101% |
| Class D | -0.25% | -0.01% | -0.31% | 101% | 101% |
| Class F (optional) | -0.04% | -0.26% | -0.33% | 101% | 100% |

Conclusion

In this contribution, the interpolation filters with multiple degrees of smoothness are used to reconstruct a prediction block. Also, some restrictions are proposed to improve the coding performance. Multiple 4-tap interpolation filter method can be an alternative of the current interpolation filter of BMS without complexity increment.

References

[1] “Description of SDR, HDR and 360° video coding technology proposal considering mobile application scenario by Samsung, Huawei, GoPro, and HiSilicon,” JVET-J0024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[2]	“Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002.

[3] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).