| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0064 |
| --- | --- |

| Title: | Simplified NAL Unit Header and IRAP pictures for random access |  |  |
| --- | --- | --- | --- |
| Status: | Input document for JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gahyun Ryu Woongil Choi Minwoo Park Kiho Choi Youngo Park Kwangpyo Park | Tel: Email: | +82-10-3314-9288 ghryu@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This document describes a high level syntax, semantics for random access. A high level syntax on NAL unit header and NAL unit type are proposed.

Introduction

We propose a high level syntax and semantics for random access. A one bit flag indication of random access at NAL unit header. We also suggest IRAP type.

Proposal

We propose two parts for random access play. First, a 1 bit flag indicates signaling of random access status in NAL unit header. If this flag is set to 1, then player could immediately decide the right performance without parsing following syntax. So we propose 1 bit flag would be appropriate for indicates the random access situation. When set to 1, the bitstream contains “leading picture” that should not be referenced by non leading pictures which followed by IRAP when performs decoding. Discarding and not referenced by non leading pictures process provides error free random access and also provides parallel decoding process. Second, a simplified version of IRAP type in NAL unit type. IRAP refers to CRA pictures and BLA pictures. IRAP contains the leading pictures and that should not be referenced by the non leading pictures. The bitstream relying on IDR pictures for random access have significantly lower coding efficiency. So we propose to exclude IDR. Following is CTC test result for IDR.(TBD)

The proposed syntax and semantics are as follows.

- 7.3.1.2 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| broken_link_flag | u(1) |
| nal_unit_type | u(7)u(5) |
| Reserved | u(1) |
| } |  |

- 7.4.2.1 NAL unit header semantics

broken_link_flag indicates random access occurred or not. If this flag is set to1, current play is random access status then decoder first acquire POC(Picture Order Count) or related values and determines the location of leading pictures. The flag value 1 must guarantee that the bitstream following IRAP should not reference leading pictures. Otherwise, if set to 0 then it is normal play status.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

Following type is sematic definition of IRAP.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 1 | IRAP_NUT | Coded slice of an IRAP picture slice_layer_rbsp( ) | VCL |

IRAP refers to CRA pictures and BLA pictures.

When nal_unit_type is IRAP, the following pictures of IRAP should not reference the leading pictures. The decoding process should mark leading pictures as unreferenced pictures to prevent leading pictures from being referenced by IRAP following pictures. The decoder should skip leading pictures and not be referenced by non leading pictures when nal_unit_type is IRAP. The leading pictures could be determined by POC or related values.

Conclusion

We proposed “broken link” for random access indication. This one bit indication approach provides simplified decoding process method when random access performed. We also suggest a simplified IRAP feature for random access. This would lessen the complexity of decoding process for random access and provides parallel decoding processing for various applications these days.

Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)