| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0064 |
| --- | --- |

| Title: | Simplified NAL Unit Header and IRAP pictures for random access |  |  |
| --- | --- | --- | --- |
| Status: | Input document for JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gahyun Ryu Woongil Choi Minwoo Park Kiho Choi Youngo Park Kwangpyo Park | Tel: Email: | +82-10-3314-9288 ghryu@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This document describes a high level syntax, semantics for random access. A high level syntax on NAL unit header and NAL unit type are proposed.

Introduction

We propose two parts for high level syntax. First, we suggest simple definition of IRAP. In this proposal, we define IRAP as I picture which supports random access. IRAP bitstream are allowed to have leading pictures. The lading picture is a picture that precedes the associated IRAP picture in output order. If IRAP bitstream contains the leading pictures then the pictures followed by IRAP should not reference the leading pictures. This process provides safe and error free decoding when random access occurred. Second, we suggest 1 bit signalling of random access indication at NAL unit header. This bit is set to 1 when NAL unit type is IRAP and random access occurred. This one bit depicts the right decoding process for random access situation.

Proposal

IRAP type

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

Following type is sematic of IRAP based on [1].

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 1 | IRAP_NUT | Coded slice of an IRAP picture slice_layer_rbsp( ) | VCL |

IRAP refers to I pictures which support random access.

When nal_unit_type is IRAP and if bitstream contains leading pictures, the decoder first acquires POC or related value. The following pictures of IRAP should not reference the leading pictures. The decoding process should mark leading pictures as unreferenced pictures to prevent leading pictures from being referenced by pictures which follows IRAP. When nal_unit_type is IRAP then it should guarantee safe decoding process to the pictures which followed by IRAP. The leading pictures could be determined by POC or related values. Currently, a document [1] does not specify the method of reference picture set yet, so we are not going to describe the POC in detail here in this proposal.

A random access indication flag

The proposed syntax and semantics are as follows based on [1].

- 7.3.1.2 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| broken_link_flag | u(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| reserved | u(6) |
| } |  |

- 7.4.2.1 NAL unit header semantics

broken_link_flag indicates random access occurred or not. This flag is set to1, when random access occurred and nal_unit_type is IRAP. Otherwise, it is normal play status.

Conclusion

We suggest a simplified IRAP type. This picture type provides safe and error free decoding procedure for random access and parallel decoding processing for various applications these days. We also propose a simple random access indication flag in NAL unit header.

Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

References

[1] JVET-K1001-v6, Versatile Video Coding (Draft 2), Ljubljana, July 2018