| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0085-v1 |
| --- | --- |

| Title: | CE3.5.8: Line buffer reduction for LM chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, two tests to reduce the line buffer requirement for linear model (LM) mode are reported. In VTM2.0.1, two luma lines above the current coding unit (CU) are used for deriving LM parameters. In CE3.5.8.1, if the above side of the current CU is at a coding tree unit (CTU) boundary, only one line of luma reconstructed samples from the above CTU are used for deriving LM parameters. In CE3.5.8.2, if the above side of the current CU is at CTU boundary, no luma reconstructed samples from the above CTU are used for deriving LM parameters.

Compared with VTM2.01, results of All Intra (AI) configuration are as follows.
CE3.5.8.1: BD-rates = 0.01% (Y), 0.01% (U), 0.04% (V); EncT=100%; DecT=101%
CE3.5.8.2: BD-rates = 0.13% (Y), 0.65% (U), 0.67% (V); EncT=100%; DecT=101%

# Introduction

A cross-component linear model (CCLM) mode is proposed in COM16–C806 [1] for reducing the cross-components redundancy. CCLM includes two items; one is the conventional LM as in HM7.0, and the other is Cb-to-Cr residual prediction for non-LM intra chroma modes. In the JVET-K meeting [2], only the conventional LM is adopted. In this contribution, to further simplify the LM chroma mode design, restrictions at CTU boundary are proposed to reduce the line buffer requirement.

# Proposed methods

The following two methods are proposed to reduce the line buffer requirement for deriving LM parameters.

CE3.5.8.1: If the above side of the current CU is at a coding tree unit (CTU) boundary, only one line of luma reconstructed samples from the above CTU are used for deriving LM parameters.

CE3.5.8.2: If the above side of the current CU is at CTU boundary, no luma reconstructed samples from the above CTU are used for deriving LM parameters. Note that, if the above side of the current CU is not at CTU boundary, the number of the above luma lines used in the derivation of LM parameters is not changed (i.e., two above luma lines).

# Experimental results

The proposed method has been implemented on BMS2.0.1 [3], and results of CE3.5.8.1 and CE3.5.8.2 in AI and RA configurations are summarized in Table 1. The simulations were performed following the JVET common test conditions (CTC) [4]. More detailed results can be found in the accompanying Excel files.

Table 1. Results of CE3.5.8.1 (left side) and CE3.5.8.2 (right side)

|  | All Intra Main10 |  |  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.04% | 100% | 100% | 0.36% | 1.36% | 1.32% | 100% | 100% |
| Class A2 | 0.01% | 0.06% | -0.03% | 100% | 101% | 0.13% | 0.80% | 0.32% | 100% | 101% |
| Class B | 0.01% | 0.03% | 0.07% | 100% | 101% | 0.09% | 0.55% | 0.81% | 100% | 101% |
| Class C | 0.00% | 0.01% | 0.09% | 100% | 101% | 0.06% | 0.34% | 0.51% | 100% | 100% |
| Class E | 0.01% | -0.04% | 0.01% | 100% | 100% | 0.04% | 0.36% | 0.32% | 100% | 100% |
| Overall | 0.01% | 0.01% | 0.04% | 100% | 101% | 0.13% | 0.65% | 0.67% | 100% | 101% |
| Class D | 0.00% | 0.00% | -0.03% | 100% | 102% | 0.04% | 0.14% | 0.10% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.14% | 0.08% | 100% | 103% | 0.19% | 1.15% | 1.20% | 100% | 102% |
| Class A2 | 0.01% | -0.05% | 0.01% | 100% | 100% | 0.06% | 0.66% | 0.24% | 100% | 101% |
| Class B | 0.03% | 0.04% | 0.03% | 100% | 101% | 0.04% | 0.54% | 0.76% | 100% | 101% |
| Class C | 0.02% | 0.16% | -0.03% | 100% | 102% | 0.06% | 0.37% | 0.50% | 100% | 101% |
| Overall | 0.01% | 0.08% | 0.02% | 100% | 102% | 0.08% | 0.64% | 0.68% | 100% | 101% |
| Class D | 0.02% | -0.16% | -0.16% | 100% | 100% | -0.01% | 0.16% | 0.33% | 100% | 100% |

# Conclusion

This contribution presented two methods to reduce the line buffer requirement for deriving LM parameters. The test results of CE3.5.8.1 and CE3.5.8.2 are reported.

# References

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding,” ITU-T SG16 Doc. COM16–C806, Feb. 2015.

- G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 11th JVET Meeting (Ljubljana, 10–18 July 2018),” JVET-K1000, July 2018.

- BMS-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).